# SHARC+ Dual-Core DSP with Arm Cortex-A5

ADSP-SC582/SC583/SC584/SC587/SC589/ADSP-21583/21584/21587

### SYSTEM FEATURES

Dual enhanced SHARC+ high performance floating-point cores

Up to 500 MHz per SHARC+ core

Up to 5 Mb (640 kB) Level 1 (L1) SRAM memory per core with parity (optional ability to configure as cache)

32-bit, 40-bit, and 64-bit floating-point support 32-bit fixed point

Byte, short-word, word, long-word addressed Arm Cortex-A5 core

500 MHz/800 DMIPS with NEON/VFPv4-D16/Jazelle 32 kB L1 instruction cache/32 kB L1 data cache

256 kB Level 2 (L2) cache with parity Powerful DMA system

On-chip memory protection Integrated safety features

19 mm × 19 mm 349/529 BGA (0.8 pitch), RoHS compliant Low system power across automotive temperature range

### MEMORY

Large on-chip L2 SRAM with ECC protection, up to 256 kB On-chip L2 ROM (512 kB)

Two Level 3 (L3) interfaces optimized for low system power, providing a 16-bit interface to DDR3 (supporting 1.5 V capable DDR3L devices), DDR2, or LPDDR1 SDRAM devices

### ADDITIONAL FEATURES

Security and Protection

Cryptographic hardware accelerators Fast secure boot with IP protection Support for Arm TrustZone

Accelerators

High performance pipelined FFT/IFFT engine FIR, IIR, HAE, SINC offload engines

SYSTEM CONTROL

SECURITY  AND PROTECTION

PERIPHERALS

SIGNAL ROUTING UNIT (SRU)

CORE 0	CORE 1	CORE 2

40–28

6

PROGRAM FLOW

102–80

CLOCK, RESET, AND POWER

L3 MEMORY INTERFACES

SYSTEM L2 MEMORY

SRAM

SYSTEM ACCELERATION

DDR3 DDR2 LPDDR1

DDR3 DDR2 LPDDR1

(ECC)

2 Mb

(256 kB)

ROM

2 Mb

ROM

2 Mb

(256 kB) (256 kB)

10

DEBUG UNIT

Arm® CoreSightTM

WATCHPOINTS (SWU)

16	16

DATA	DATA

6

7

8

L2 CACHE 256 kB (PARITY)

L1 CACHE

32 kB L1 I-CACHE

32 kB L1 D-CACHE

ENCRYPTION/DECRYPTION

DSP FUNCTIONS (FFT/IFFT, FIR, IIR, HAE/SINC)

SYSTEM CROSSBAR AND DMA SUBSYSTEM

L1 SRAM (PARITY)

5 Mb (640 kB) SRAM/CACHE

L1 SRAM (PARITY)

5 Mb (640 kB) SRAM/CACHE

Arm® Cortex®-A5

SYSTEM PROTECTION (SPU)

SYSTEM MEMORY PROTECTION UNIT (SMPU)

FAULT MANAGEMENT

Arm® TrustZone® SECURITY

DUAL CRC

WATCHDOGS

OTP MEMORY

THERMAL MONITOR UNIT (TMU)

| 2×2 PRECISION CLOCK GENERATORS |  | 2x DAI 2x PIN BUFFER |
| --- | --- | --- |
| ASRC 2×4 PAIRS | FULL SPORT 2×4 |  |
| 2×1 S/PDIF Rx/Tx |  |  |

AEC-Q100 qualified for automotive applications

CLOCK GENERATION (CGU)

CLOCK DISTRIBUTION UNIT (CDU)

REAL TIME CLOCK (RTC)

RESET CONTROL (RCU)

POWER MANAGEMENT (DPM)

| 3× I2C |  |  |
| --- | --- | --- |
| 2× LINK PORTS |  | G P I O |
| 2× SPI + 1× QUAD SPI |  |  |
| 3× UARTs |  |  |
| 1× EPPI |  |  |
| 3× ePWM |  |  |
| 8× TIMERS + 1× COUNTER |  |  |
| ADC CONTROL MODULE (ACM) |  |  |
| ASYNC MEMORY (16-BIT) |  |  |
| 2× CAN2.0 |  |  |
| SD/SDIO/eMMC |  |  |
| MLB 3-PIN |  |  |
| 2× EMAC |  |  |
| SINC FILTER |  |  |
| 8x SHARC FLAGS |  |  |
| 2× USB 2.0 HS |  |  |
| MLB 6-PIN |  |  |
| PCIe2.0 (1 lane) |  |  |
| HADC (8 CHAN, 12-BIT) |  |  |

Figure 1. Processor Block Diagram

SHARC, SHARC+, and the SHARC logo are registered trademarks of Analog Devices, Inc.

Rev. B	Document Feedback

Information furnished by Analog Devices is believed to be accurate and reliable. However, no responsibility is assumed by Analog Devices for its use, nor for any

SYS EVENT CORE 0 (GIC)

SYS EVENT CORES 1-2 (SEC)

TRIGGER ROUTING (TRU)

infringements of patents or other rights of third parties that may result from its use. Specifications subject to change without notice. No license is granted by implication or otherwise under any patent or patent rights of Analog Devices. Trademarks and registered trademarks are the property of their respective owners.

One Technology Way, P.O. Box 9106, Norwood, MA 02062-9106 U.S.A. Tel: 781.329.4700 ©2018 Analog Devices, Inc. All rights reserved. Technical Support	www.analog.com

# TABLE OF CONTENTS

System Features	1

Memory	1

Additional Features	1

Table of Contents	2

Revision History	2

General Description	3

ARM Cortex-A5 Processor	5

SHARC Processor	6

SHARC+ Core Architecture	8

System Infrastructure	10

System Memory Map	11

Security Features	14

Security Features Disclaimer	15

Safety Features	15

Processor Peripherals	15

System Acceleration	20

System Design	21

System Debug	23

Development Tools	24

Additional Information	25

Related Signal Chains	25

ADSP-SC58x/ADSP-2158x Detailed Signal

Descriptions	26

349-Ball CSP_BGA Signal Descriptions	31

GPIO Multiplexing for the 349-Ball CSP_BGA Package	40

529-Ball CSP_BGA Signal Descriptions	43

GPIO Multiplexing for the 529-Ball CSP_BGA Package ... 55 ADSP-SC58x/ADSP-2158x Designer Quick Reference	58

Specifications	79

Operating Conditions	79

Electrical Characteristics	83

HADC	87

TMU	87

Absolute Maximum Ratings	88

ESD Caution	88

Timing Specifications	89

Output Drive Currents	153

Test Conditions	155

Environmental Conditions	157

ADSP-SC58x/ADSP-2158x 349-Ball BGA Ball

Assignments	158

Numerical by Ball Number	158

Alphabetical by Pin Name	160

Configuration of the 349-Ball CSP_BGA	162

ADSP-SC58x/ADSP-2158x 529-Ball BGA Ball

Assignments	163

Numerical by Ball Number	163

Alphabetical by Pin Name	166

Configuration of the 529-Ball CSP_BGA	169

Outline Dimensions	170

Surface-Mount Design	171

Automotive Products	172

Ordering Guide	173

## REVISION HISTORY

##### 12/2018—Rev. A to Rev. B

Changes to Additional Features	1

Changes to Table 3, General Description	3

Changes to One Time Programmable Memory (OTP)	10

Changes to Table 7 and Table 8, System Memory Map	11

Changes to Housekeeping Analog-to-Digital Converter (HADC)	19

Changes to Media Local Bus (Media LB)	19

Changes to ADSP-SC58x/ADSP-2158x Detailed Signal Descrip- tions	26

Changes to ADSP-SC58x/ADSP-2158x 349-Ball CSP_BGA Sig- nal Descriptions	31

Changes to ADSP-SC58x/ADSP-2158x 529-Ball CSP_BGA Sig- nal Descriptions	43

Changes to ADSP-SC58x/ADSP-2158x Designer Quick Refer- ence	58

Deleted Package Information from Specifications	79

Changes to Operating Conditions	79

Changes to Table 28, Operating Conditions	79

Changes to Table 29, Clock Related Operating Conditions 81 Changes to Total Internal Power Dissipation	85

Changes Universal Serial Bus (USB)	138

Changes 10/100 EMAC Timing (ETH0 and ETH1)	139

Changes to Program Trace Macrocell (PTM) Timing	151

Changes to Test Conditions	155

Changes to Automotive Products	172

# GENERAL DESCRIPTION

The ADSP-SC58x/ADSP-2158x processors are members of the SHARC® family of products. The ADSP-SC58x processor is based on the SHARC+ dual core and the Arm® Cortex®-A5 core. The ADSP-SC58x/ADSP-2158x SHARC processors are members of the SIMD SHARC family of digital signal proces- sors (DSPs) that feature Analog Devices, Inc., Super Harvard Architecture®. These 32-bit/40-bit/64-bit floating-point proces- sors are optimized for high performance audio/floating-point applications with large, on-chip, static random-access memory (SRAM), multiple internal buses that eliminate input/output (I/O) bottlenecks, and innovative digital audio interfaces (DAI). New additions to the SHARC+ core include cache enhance- ments and branch prediction, while maintaining instruction set compatibility to previous SHARC products.

By integrating a set of industry leading system peripherals and memory (see Table 1, Table 2, and Table 3), the Arm Cortex-A5 and SHARC processor is the platform of choice for applications that require programmability similar to reduced instruction set computing (RISC), multimedia support, and leading edge signal processing in one integrated package. These applications span a wide array of markets, including automotive, professional audio, and industrial-based applications that require high float- ing-point performance.

Table 2 provides comparison information for features that vary across the standard processors. (N/A in the table means not applicable.)

Table 3 provides comparison information for features that vary across the automotive processors. (N/A in the table means not applicable.)

##### Table 1. Common Product Features

| Product Features | ADSP-SC58x/ADSP-2158x |
| --- | --- |
| DAI (includes SRU) Full SPORTs S/PDIF receive/transmit ASRCs PCGs | 2 4 per DAI 1per DAI 4 pair per DAI 2 per DAI |
| I2C (TWI) Quad-data bit SPI Dual-data bit SPI CAN2.0 UARTs Link ports | 3 1 2 2 3 2 |
| Enhanced PPI | 1 |
| GP timer1 GP counter Enhanced PWMs2 Watchdog timers | 8 1 3 2 |
| ADC control module Static memory controller | Yes Yes |
| Hardware accelerators High performance FFT/IFFT FIR/IIR Harmonic analysis engine SINC filter Security cryptographic engine | Yes Yes Yes Yes Yes |
| Multichannel 12-bit ADC | 8-channel |

1 Eight timers are available in the 529-BGA package only. The 349-BGA package does not include Timer 6 and Timer 7.

2 On the 349-BGA package, the PWM2_AH/AL and PWM2_BH/BL signals are not available. The PWM2_CH/CL and PWM2_DH/DL signals, however, are available and can be used in conjunction with PWM2_TRIP0 and PWM2_SYNC signals.

##### Table 2. Comparison of ADSP-SC58x/ADSP-2158x Processor Features

| Processor Feature |  | ADSP- SC582 | ADSP- SC583 | ADSP- SC584 | ADSP- SC587 | ADSP- SC589 | ADSP- 21583 | ADSP- 21584 | ADSP- 21587 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arm Cortex-A5 (MHz, Max) Arm Core L1 Cache (I, D kB) Arm Core L2 Cache (kB) |  | 500 32, 32 256 | 500 32, 32 256 | 500 32, 32 256 | 500 32, 32 256 | 500 32, 32 256 | N/A N/A N/A | N/A N/A N/A | N/A N/A N/A |
| SHARC+ Core1 (MHz, Max) |  | 500 | 500 | 500 | 500 | 500 | 500 | 500 | 500 |
| SHARC+ Core2 (MHz, Max) |  | N/A | 500 | 500 | 500 | 500 | 500 | 500 | 500 |
| SHARC L1 SRAM (kB) |  | 640 | 384 | 640 | 640 | 640 | 384 | 640 | 640 |
| System Memory | L2 SRAM (Shared) (kB) | 256 | 256 | 256 | 256 | 256 | 256 | 256 | 256 |
|  | L2 ROM (Shared) (kB) | 512 | 512 | 512 | 512 | 512 | 512 | 512 | 512 |
|  | DDR3/DDR2/LPDDR1 Controller (16-bit) | 1 | 1 | 1 | 2 | 2 | 1 | 1 | 2 |
| USB 2.0 HS + PHY (Host/Device/OTG) |  | 1 | 1 | 1 | 1 | 1 | N/A | N/A | N/A |
| USB 2.0 HS + PHY (Host/Device) |  | N/A | N/A | N/A | 1 | 1 | N/A | N/A | N/A |
| 10/100 Std EMAC |  | N/A | N/A | N/A | 1 | 1 | N/A | N/A | N/A |
| 10/100/1000 /AVB EMAC + Timer IEEE 1588 |  | 1 | 1 | 1 | 1 | 1 | N/A | N/A | N/A |
| SDIO/eMMC |  | N/A | N/A | N/A | 1 | 1 | N/A | N/A | N/A |
| PCIe 2.0 (1 Lane) |  | N/A | N/A | N/A | N/A | 1 | N/A | N/A | N/A |
| RTC |  | N/A | N/A | N/A | 1 | 1 | N/A | N/A | 1 |
| GPIO Ports |  | Port A to E | Port A to E | Port A to E | Port A to G | Port A to G | Port A to E | Port A to E | Port A to G |
| GPIO + DAI Pins |  | 80 + 28 | 80 + 28 | 80 + 28 | 102 + 40 | 102 + 40 | 80 + 28 | 80 + 28 | 102 + 40 |
| 19 mm × 19 mm Package Options |  | 349-BGA | 349-BGA | 349-BGA | 529-BGA | 529-BGA | 349-BGA | 349-BGA | 529-BGA |

Table 3. Comparison of ADSP-SC58x/ADSP-2158x Processor Features for Automotive

| Processor Feature |  | ADSP-SC582W | ADSP-SC583W | ADSP-SC584W | ADSP-SC587W | ADSP-21583W | ADSP-21584W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Arm Cortex-A5 (MHz, Max) Arm Core L1 Cache (I, D kB) Arm Core L2 Cache (kB) |  | 450 32, 32 256 | 450 32, 32 256 | 500 32, 32 256 | 500 32, 32 256 | N/A N/A N/A | N/A N/A N/A |
| SHARC+ Core1 (MHz, Max) SHARC+ Core2 (MHz, Max) SHARC L1 SRAM (kB) |  | 450 N/A 640 | 450 450 384 | 500 500 640 | 500 500 640 | 450 450 384 | 500 500 640 |
| System Memory | L2 SRAM (Shared) (kB) L2 ROM (Shared) (kB) DDR3/DDR2/LPDDR1 Controller (16-bit) | 256 512 1 | 256 512 1 | 256 512 1 | 256 512 2 | 256 512 1 | 256 512 1 |
| USB 2.0 HS + PHY (Host/Device/OTG) |  | 1 | 1 | 1 | 1 | N/A | N/A |
| USB 2.0 HS + PHY (Host/Device) |  | N/A | N/A | N/A | 1 | N/A | N/A |
| 10/100 Std EMAC |  | N/A | N/A | N/A | 1 | N/A | N/A |
| 10/100/1000/AVB EMAC + Timer IEEE 1588 |  | 1 | 1 | 1 | 1 | N/A | N/A |
| SDIO/eMMC |  | N/A | N/A | N/A | 1 | N/A | N/A |
| PCIe 2.0 (1 Lane) |  | N/A | N/A | N/A | N/A | N/A | N/A |
| MLB 3-Pin/6-Pin |  | 1 | 1 | 1 | 1 | 1 | 1 |
| RTC |  | N/A | N/A | N/A | 1 | N/A | N/A |
| GPIO Ports |  | Port A to E | Port A to E | Port A to E | Port A to G | Port A to E | Port A to E |
| GPIO + DAI Pins |  | 80 + 28 | 80 + 28 | 80 + 28 | 102 + 40 | 80 + 28 | 80 + 28 |
| 19 mm × 19 mm Package Options |  | 349-BGA | 349-BGA | 349-BGA | 529-BGA | 349-BGA | 349-BGA |

## ARM CORTEX-A5 PROCESSOR

The Arm Cortex-A5 processor (see Figure 2) is a high perfor- mance processor with the following features:

- Instruction cache unit (32 kB) and data L1 cache unit (32 Kb)

- In order pipeline with dynamic branch prediction

- Arm, Thumb®, and ThumbEE instruction set support

- Arm TrustZone® security extensions

- Harvard L1 memory system with a memory management unit (MMU)

- Arm7TM debug architecture

- Trace support through an embedded trace macrocell (ETM) interface

- Extension—vector floating-point unit (IEEE 754) with trapless execution

- Extension—media processing engine (MPE) with NEONTM technology

- Extension—Jazelle® hardware acceleration

| PREFETCH UNIT AND BRANCH PREDICTOR (PFU) |  |
| --- | --- |
| INSTRUCTION MICRO-TLB |  |

EMBEDDED TRACE MACROCELL	CoreSightTM INTERFACE (ETM) INTERFACE

Arm® Cortex®-A5 PROCESSOR

DEBUG

CP15

DATA PROCESSING UNIT (DPU)

Arm® Cortex®-A5 BUS MASTER PORT

SYSTEM FABRIC

DATA MICRO-TLB

DATA MASTER PORTS SHARC PROCESSORS

L2 CACHE CONTROLLER

(CoreLinkTM PL-310) 256 kB

GENERIC INTERRUPT CONTROLLER

(PrimeCell® PL-390)

BUS INTERFACE UNIT (BIU)

INSTRUCTION CACHE UNIT (ICU)

32 kB

MAIN TRANSMISSION LOOKINSIDE BUFFER (TLB)

DATA CACHE UNIT (DCU)

32 kB

DATA STORE BUFFER (STB)

NEONTM MEDIA PROCESSING ENGINE

TO OTHER CORES

Figure 2.  Arm Cortex-A5 Processor Block Diagram

Generic Interrupt Controller (GIC), PL390 (ADSP-SC58x Only)

The generic interrupt controller (GIC) is a centralized resource for supporting and managing interrupts. The GIC splits into the distributor block (GICPORT0) and the CPU interface block (GICPORT1).

##### Generic Interrupt Controller Port0 (GICPORT0)

The GICPORT0 distributor block performs interrupt prioritiza- tion and distribution to the GICPORT1 blocks that connect to the processors in the system. It centralizes all interrupt sources, determines the priority of each interrupt, and forwards the interrupt with the highest priority to the interface, for priority masking and preemption handling.

##### Generic Interrupt Controller Port1 (GICPORT1)

The GICPORT1 CPU interface block performs priority masking and preemption handling for a connected processor in the sys- tem. GICPORT1 supports 8 software generated interrupts (SGIs) and 254 shared peripheral interrupts (SPIs).

#### L2 Cache Controller, PL310 (ADSP-SC58x Only)

The L2 cache controller, PL310 (see Figure 2), works efficiently with the Arm Cortex-A5 processors that implement system fab- ric. The cache controller directly interfaces on the data and instruction interface. The internal pipelining of the cache con- troller is optimized to enable the processors to operate at the same clock frequency. The cache controller supports the following:

- Two read/write 64-bit slave ports, one connected to the Arm Cortex-A5 instruction and data interfaces, and one connecting the Arm Cortex-A5 and SHARC+ cores for data coherency.

- Two read/write 64-bit master ports for interfacing with the system fabric.

## SHARC PROCESSOR

Figure 3 shows the SHARC processor integrates a SHARC+ SIMD core, L1 memory crossbar, I/D cache controller, L1 mem- ory blocks, and the master/slave ports. Figure 4 shows the SHARC+ SIMD core block diagram.

P-CACHE

B2 RAM

P-CACHE

B2 RAM

P-CACHE

B2 RAM

P-CACHE

B2 RAM

P-CACHE

B2 RAM

P-CACHE

B2 RAM

B3 (64)

B2 (64)

B1 (64)

B0 (64)

The SHARC processor supports a modified Harvard architec- ture in combination with a hierarchical memory structure. L1 memories typically operate at the full processor speed with little or no latency.

DM (64)

PM (64)

I-CACHE	D-CACHE

B3 RAM

B2 RAM

D-CACHE

B1

RAM

B0 RAM

SIMD Processor

CCLK DOMAIN

IO (32)

IO (32)

IO (32)

IO (32)

(32)

DM (64)

CMD (64)

PM (64)

PS (64/48)

CMI (64)

INTERRUPT SEC

INTERNAL MEMORY INTERFACE (IMIF) I/D CACHE CONTROL

MASTER PORT INSTRUCTION

MASTER PORT DATA

SHARC+® SIMD CORE

CORE MMR

SLAVE PORT 2

SYSTEM FABRIC SYSCLK DOMAIN

SLAVE PORT 1

Figure 3. SHARC Processor Block Diagram

SIMD Core

+

DMD/PMD 64

PM DATA 48

PM ADDRESS 24

PM ADDRESS 32

DM ADDRESS 32

PM DATA 64

TO IMIF

DM DATA 64

DATA SWAP

PEy DATA

REGISTER

Sx

16 × 40-BIT

ALU

MULTIPLIER

MSF 80-BIT

MSB 80-BIT

MRB 80-BIT

MRF 80-BIT

STYKy

STYKx

ASTATy

ASTATx

PEx DATA GISTER

Rx

× 40-BIT

RE 16

SHIFTER

ALU

SHIFTER

MULTIPLIER

DAG2 16 × 32

DAG1 16 × 32

SYSTEM

I/F

USTAT PX

11-STAGE PROGRAM SEQUENCER

CONFLICT CACHE

BTB BP

CEC

FLAGS

DEBUG TRACE

#### L1 Memory

Figure 4. SHARC+ SIMD Core Block Diagram

106.7k words of 48-bit instructions (or 40-bit data), or combi-

Figure 5 shows the ADSP-SC58x/ADSP-2158x memory map. Each SHARC+ core has a tightly coupled L1 SRAM of up to

5 Mb. Each SHARC+ core can access code and data in a single cycle from this memory space. The Arm Cortex-A5 core can also access this memory space with multicycle accesses.

In the SHARC+ core private address space, both cores have L1 memory.

SHARC+ core memory-mapped register (CMMR) address space is 0x 0000 0000 through 0x 0003 FFFF in normal word (32-bit). Each block can be configured for different combina- tions of code and data storage. Of the 5 Mb SRAM, up to 1024 Kb can be configured for data memory (DM), program

memory (PM), and instruction cache. Each memory block sup- ports single-cycle, independent accesses by the core processor and I/O processor. The memory architecture, in combination with its separate on-chip buses, allows two data transfers from the core and one from the DMA engine in a single cycle. The SRAM of the processor can be configured as a maximum of 160k words of 32-bit data, 320k words of 16-bit data,

nations of different word sizes up to 5 Mb. All of the memory can be accessed as 8-bit, 16-bit, 32-bit, 48-bit, or 64-bit words. Support of a 16-bit floating-point storage format doubles the amount of data that can be stored on chip.

Conversion between the 32-bit floating-point and 16-bit float- ing-point formats is performed in a single instruction. While each memory block can store combinations of code and data, accesses are most efficient when one block stores data using the DM bus for transfers, and the other block stores instructions and data using the PM bus for transfers.

Using the DM and PM buses, with each bus dedicated to a memory block, assures single-cycle execution with two data transfers. In this case, the instruction must be available in the cache. The system configuration is flexible, but a typical config- uration is 512 Kb DM, 128 Kb PM, and 128 Kb of instruction cache, with the remaining L1 memory configured as SRAM. Each addressable memory space outside the L1 memory can be accessed either directly or via cache.

The memory map in Table 4 gives the L1 memory address space and shows multiple L1 memory blocks offering a configurable mix of SRAM and cache.

#### L1 Master and Slave Ports

Each SHARC+ core has two master and two slave ports to and from the system fabric. One master port fetches instructions. The second master port drives data to the system world. Both slave ports allow conflict free core/direct memory access (DMA) streams to the individual memory blocks. For slave port addresses, refer to the L1 memory address map in Table 4.

#### L1 On-Chip Memory Bandwidth

The internal memory architecture allows programs to have four accesses at the same time to any of the four blocks, assuming no block conflicts. The total bandwidth is realized using both the DMD and PMD buses.

#### Instruction and Data Cache

The ADSP-SC58x/ADSP-2158x processors also include a traditional instruction cache (I-cache) and two data caches (D-cache) (PM and DM caches). These caches support one instruction access and two data accesses over the DM and PM buses, per CCLK cycle. The cache controllers automatically

manage the configured L1 memory. The system can configure part of the L1 memory for automatic management by the cache controllers. The sizes of these caches are independently configu- rable from 0 kB to a maximum of 128 kB each. The memory not managed by the cache controllers is directly addressable by the processors. The controllers ensure the data coherence between the two data caches. The caches provide user-controllable fea- tures such as full and partial locking, range-bound invalidation, and flushing.

#### System Event Controller (SEC) Input

The output of the system event controller (SEC) controller is forwarded to the core event controller (CEC) to respond directly to all unmasked system-based interrupts. The SEC also supports nesting including various SEC interrupt channel arbi- tration options. For all SEC channels, the processor automatically stacks the arithmetic status (ASTATx and ASTATy) registers and mode (MODE1) register in parallel with the interrupt servicing.

#### Core Memory-Mapped Registers (CMMR)

0x FFFF FFFF

| DMC1 (1GB) |  |  |
| --- | --- | --- |
| DMC0 (1GB) |  |  |
| SPI2 FLASH (512MB) |  |  |
| PCIe (256MB) |  |  |
| SMC BANK 3 (64MB) |  |  |
| SMC BANK 2 (64MB) |  |  |
| SMC BANK 1 (64MB) |  |  |
| SMC BANK 0 (64MB) |  |  |
| SYSTEM MMR |  |  |
| RESERVED |  |  |
| SHARC2 L1 MULTI-MEMORY SPACE |  |  |
| RESERVED |  |  |
| SHARC1 L1 MULTI-MEMORY SPACE |  |  |
| RESERVED |  |  |
| L2 ROM 2 (2Mb) |  |  |
| RESERVED |  |  |
| L2 BOOT ROM 2 (0.25Mb) (SHARC Cores) |  |  |
| RESERVED |  |  |
| L2 ROM 1 (2Mb) |  |  |
| RESERVED |  |  |
| L2 BOOT ROM 1 (0.25Mb) (SHARC Cores) |  |  |
| RESERVED |  |  |
| L2 SRAM (2Mb) |  |  |
| RESERVED |  |  |
| L2 BOOT ROM 0 (0.25Mb) (ARM CORE 0) |  |  |
| RESERVED | RESERVED | 0x 0x |
|  | L1 BLOCK 3 SRAM (1Mb) | 0x 0x 0x 0x 0x 0x 0x 0x |
|  | RESERVED |  |
|  | L1 BLOCK 2 SRAM (1Mb) |  |
|  | RESERVED |  |
|  | L1 BLOCK 1 SRAM (1.5Mb) |  |
| ARM L2 CONFIG REGS (4KB) | RESERVED |  |
| RESERVED | L1 BLOCK 0 SRAM (1.5Mb) |  |
| ARM BOOT (32KB) | RESERVED/CORE MMRs/ OTHER MEMORY ALIASES |  |

0x C000 0000

0x 8000 0000

0x 6000 0000

0x 5000 0000

0x 4C00 0000

0x 4800 0000

0x 4400 0000

0x 4000 0000

0x 3000 0000

0x 28F9 FFFF

0x 28A4 0000

0x 2879 FFFF

0x 2824 0000

0x 202B FFFF

0x 2028 0000

0x 2020 7FFF

0x 2020 0000

0x 201B FFFF

0x 2018 0000

0x 2010 7FFF

0x 2010 0000

0x 200B FFFF

0x 2008 0000

0x 2000 7FFF

ARM ADDRESS SPACE

0x 2000 0000

0x 1000 1000

0x 1000 0000

0x 0000 7FFF

0x 0000 0000

Figure 5. ADSP-SC58x/ADSP-2158x Memory Map

UNIFIED BYTE ADDRESS

SPACE

2000 0000

0039 FFFF

0038 0000

SHARC PRIVATE ADDRESS SPACE

0031 FFFF

0030 0000

002E FFFF

002C 0000

0026 FFFF

0024 0000

0000 0000

The core memory-mapped registers control the L1 instruction and data cache, BTB, L2 cache, parity error, system control, debug, and monitor functions.

## SHARC+ CORE ARCHITECTURE

The ADSP-SC58x/ADSP-2158x processors are code compatible at the assembly level with the ADSP-2148x, ADSP-2147x, ADSP-2146x, ADSP-2137x, ADSP-2136x, ADSP-2126x,

ADSP-2116x, and with the first-generation ADSP-2106x SHARC processors.

The ADSP-SC58x/ADSP-2158x processors share architectural features with the ADSP-2126x, ADSP-2136x, ADSP-2137x, ADSP-214xx, and ADSP-2116x SIMD SHARC processors, shown in Figure 4 and detailed in the following sections.

#### SIMD Computational Engine

The SHARC+ core contains two computational processing ele- ments that operate as a single-instruction, multiple data (SIMD) engine.

The processing elements are referred to as PEx and PEy data registers and each contain an arithmetic logic unit (ALU), mul- tiplier, shifter, and register file. PEx is always active and PEy is enabled by setting the PEYEN mode bit in the mode control register (MODE1).

Single instruction multiple data (SIMD) mode allows the pro- cessors to execute the same instruction in both processing elements, but each processing element operates on different data. This architecture efficiently executes math intensive DSP algorithms. In addition to all the features of previous generation SHARC cores, the SHARC+ core also provides a new and sim- pler way to execute an instruction only on the PEy data register.

SIMD mode also affects the way data transfers between memory and processing elements because to sustain computational operation in the processing elements requires twice the data bandwidth. Therefore, entering SIMD mode doubles the band- width between memory and the processing elements. When using the DAGs to transfer data in SIMD mode, two data values transfer with each memory or register file access.

#### Independent, Parallel Computation Units

Within each processing element is a set of pipelined computa- tional units. The computational units consist of a multiplier, arithmetic/logic unit (ALU), and shifter. These units are arranged in parallel, maximizing computational throughput. These computational units support IEEE 32-bit single-precision floating-point, 40-bit extended-precision floating-point, IEEE 64-bit double-precision floating-point, and 32-bit fixed-point data formats.

A multifunction instruction set supports parallel execution of ALU and multiplier operations. In SIMD mode, the parallel ALU and multiplier operations occur in both processing ele- ments per core.

All processing operations take one cycle to complete. For all floating-point operations, the processor takes two cycles to complete in case of data dependency. Double-precision float- ing-point data take two to six cycles to complete. The processor stalls for the appropriate number of cycles for an interlocked pipeline plus data dependency check.

#### Core Timer

Each SHARC+ processor core also has a timer. This extra timer is clocked by the internal processor clock and is typically used as a system tick clock for generating periodic operating system interrupts.

#### Data Register File

Each processing element contains a general-purpose data regis- ter file. The register files transfer data between the computation units and the data buses, and store intermediate results. These 10-port, 32-register register files (16 primary, 16 secondary), combined with the enhanced Harvard architecture of the pro- cessor, allow unconstrained data flow between computation units and internal memory. The registers in the PEx data regis- ter file are referred to as R0–R15 and in the PEy data register file as S0–S15.

#### Context Switch

Many of the registers of the processor have secondary registers that can activate during interrupt servicing for a fast context switch. The data, DAG, and multiplier result registers have sec- ondary registers. The primary registers are active at reset, while control bits in MODE1 activate the secondary registers.

#### Universal Registers (USTAT)

General-purpose tasks use the universal registers. The four USTAT registers allow easy bit manipulations (set, clear, toggle, test, XOR) for all control and status peripheral registers.

The data bus exchange register (PX) permits data to pass between the 64-bit PM data bus and the 64-bit DM data bus or between the 40-bit register file and the PM or DM data bus.

These registers contain hardware to handle the data width difference.

#### Data Address Generators With Zero-Overhead Hardware Circular Buffer Support

For indirect addressing and implementing circular data buffers in hardware, the ADSP-SC58x/ADSP-2158x processor uses the two data address generators (DAGs). Circular buffers allow effi- cient programming of delay lines and other data structures required in digital signal processing, and are commonly used in digital filters and Fourier transforms. The two DAGs of the pro- cessors contain sufficient registers to allow the creation of up to 32 circular buffers (16 primary register sets and 16 secondary sets). The DAGs automatically handle address pointer wrap- around, reduce overhead, increase performance, and simplify implementation. Circular buffers can start and end at any mem- ory location.

#### Flexible Instruction Set Architecture (ISA)

The ISA, a 48-bit instruction word, accommodates various par- allel operations for concise programming. For example, the processors can conditionally execute a multiply, an add, and a subtract in both processing elements while branching and fetch- ing up to four 32-bit values from memory—all in a single instruction. Additionally, the double-precision floating-point instruction set is an addition to the SHARC+ core.

#### Variable Instruction Set Architecture (VISA)

In addition to supporting the standard 48-bit instructions from previous SHARC processors, the SHARC+ core processors sup- port 16-bit and 32-bit opcodes for many instructions, formerly 48-bit in the ISA. This feature, called variable instruction set architecture (VISA), drops redundant or unused bits within the 48-bit instruction to create more efficient and compact code.

The program sequencer supports fetching these 16-bit and 32- bit instructions from both internal and external memories.

VISA is not an operating mode; it is only address dependent (refer to memory map ISA/VISA address spaces in Table 7). Furthermore, it allows jumps between ISA and VISA instruc- tion fetches.

#### Single-Cycle Fetch of Instructional Four Operands

The ADSP-SC58x/ADSP-2158x processors feature an enhanced Harvard architecture in which the DM bus transfers data and PM bus transfers both instructions and data.

With the separate program memory bus, data memory buses, and on-chip instruction conflict-cache, the processor can simul- taneously fetch four operands (two over each data bus) and one instruction from the conflict cache, in a single cycle.

#### Core Event Controller (CEC)

The SHARC+ core generates various core interrupts (including arithmetic and circular buffer instruction flow exceptions) and SEC events (debug/monitor and software). The core only responds to unmasked interrupts (enabled in the IMASK register).

#### Instruction Conflict-Cache

The processors include a 32-entry instruction cache that enables three-bus operation for fetching an instruction and four data values. The cache is selective—only the instructions that require fetches conflict with the PM bus data accesses cache. This cache allows full speed execution of core, looped operations, such as digital filter multiply accumulates, and fast Fourier transforms (FFT) butterfly processing. The conflict cache serves for on-chip bus conflicts only.

#### Branch Target Buffer/Branch Predictor

Implementation of a hardware-based branch predictor (BP) and branch target buffer (BTB) reduce branch delay. The program sequencer supports efficient branching using the BTB for condi- tional and unconditional instructions.

#### Addressing Spaces

In addition to traditionally supported long word, normal word, extended precision word and short word addressing aliases, the processors support byte addressing for the data and instruction accesses. The enhanced ISA/VISA provides new instructions for accessing all sizes of data from byte space as well as converting word addresses to byte and byte to word addresses.

#### Additional Features

The enhanced ISA/VISA of the ADSP-SC58x/ADSP-2158x pro- cessors also provides a memory barrier instruction for data synchronization, exclusive data access support for multicore data sharing, and exclusive data access to enable multiprocessor programming. To enhance the reliability of the application, L1 data RAMs support parity error detection logic for every byte. Additionally, the processors detect illegal opcodes. Core inter- rupts flag both errors. Master ports of the core also detect for failed external accesses.

## SYSTEM INFRASTRUCTURE

The following sections describe the system infrastructure of the ADSP-SC58x/ADSP-2158x processors.

#### System L2 Memory

A system L2 SRAM memory of 2 Mb (256 kB) and two ROM memories, each 2 Mb (256 kB), are available to both SHARC+ cores, the Arm Cortex-A5 core, and the system DMA channels (see Table 5). All L2 SRAM/ROM blocks are subdivided into eight banks to support concurrent access to the L2 memory ports. Memory accesses to the L2 memory space are multicycle accesses by both the Arm Cortex-A5 and SHARC+ cores.

The memory space is used for various cases including

- Arm Cortex-A5 to SHARC+ core data sharing and inter- core communications

- Accelerator and peripheral sources and destination mem- ory to avoid accessing data in the external memory

- A location for DMA descriptors

- Storage for additional data for either the Arm Cortex-A5 or SHARC+ cores to avoid external memory latencies and reduce external memory bandwidth

- Storage for incoming Ethernet traffic to improve performance

- Storage for data coefficient tables cached by the SHARC+ core

See the System Memory Protection Unit (SMPU) section for options in limiting access by specific cores and DMA masters.

The Arm Cortex-A5 core has an L1 instruction and data cache, each of which is 32 kB in size. The core also has an L2 cache controller of 256 kB. When enabling the caches, accesses to all other memory spaces (internal and external) go through the cache.

#### SHARC+ Core L1 Memory in Multiprocessor Space

The Arm Cortex-A5 core can access the L1 memory of the SHARC+ core. See Table 6 for the L1 memory address in multi- processor space. The SHARC+ core can access the L1 memory of the other SHARC+ core in the multiprocessor space.

#### One Time Programmable Memory (OTP)

The processors feature 7 kB of one time programmable (OTP) memory which is memory-map accessible. This memory con- tains space for programmable unique keys and supports secure boot and secure operation.

#### I/O Memory Space

The static memory controller (SMC) is programmed to control up to two blocks of external memories or memory-mapped devices, with flexible timing parameters. Each block occupies an 8 Kb segment regardless of the size of the device used. Mapped I/Os also include PCIe data and SPI2 memory address space (see Table 7).

## SYSTEM MEMORY MAP

##### Table 4. L1 Block 0, Block 1, Block 2, and Block 3 SHARC+ Addressing Memory Map (Private Address Space)

| Memory | Long Word (64 Bits) | Extended Precision/ ISA Code (48 Bits) | Normal Word (32 Bits) | Short Word/ VISA Code (16 Bits) | Byte Access (8 Bits) |
| --- | --- | --- | --- | --- | --- |
| L1 Block 0 SRAM (1.5 Mb) | 0x00048000– 0x0004DFFF | 0x00090000– 0x00097FFF | 0x00090000– 0x0009BFFF | 0x00120000– 0x00137FFF | 0x00240000– 0x0026FFFF |
| L1 Block 1 SRAM (1.5 Mb) | 0x00058000– 0x0005DFFF | 0x000B0000– 0x000B7FFF | 0x000B0000– 0x000BBFFF | 0x00160000– 0x00177FFF | 0x002C0000– 0x002EFFFF |
| L1 Block 2 SRAM (1 Mb) | 0x00060000– 0x00063FFF | 0x000C0000– 0x000C5554 | 0x000C0000– 0x000C7FFF | 0x00180000– 0x0018FFFF | 0x00300000– 0x0031FFFF |
| L1 Block 3 SRAM (1 Mb) | 0x00070000– 0x00073FFF | 0x000E0000– 0x000E5554 | 0x000E0000– 0x000E7FFF | 0x001C0000– 0x001CFFFF | 0x00380000– 0x0039FFFF |

Table 5. L2 Memory Addressing Map

| Memory1 | Byte Address Space Arm Cortex-A5: Data Access and Instruction Fetch; SHARC+: Data Access | Normal Word Address Space for Data Access SHARC+ | Instruction Fetch VISA Address Space SHARC+ | Instruction Fetch ISA Address Space SHARC+ |
| --- | --- | --- | --- | --- |
| L2 Boot ROM02 | Arm: 0x00000000–0x00007FFF SHARC+/DMA: 0x20000000–0x20007FFF | 0x08000000–0x08001FFF | 0x00B80000–0x00B83FFF | 0x00580000–0x00581555 |
| L2 RAM (2 Mb) | 0x20080000–0x200BFFFF | 0x08020000–0x0802FFFF | 0x00BA0000–0x00BBFFFF | 0x005A0000–0x005AAAAF |
| L2 Boot ROM1 | 0x20100000–0x20107FFF | 0x08040000–0x08041FFF | 0x00B00000–0x00B03FFF | 0x00500000–0x00501555 |
| L2 ROM1 | 0x20180000–0x201BFFFF | 0x08060000–0x0806FFFF | 0x00B20000–0x00B3FFFF | 0x00520000–0x0052AAAF |
| L2 Boot ROM23 | 0x20200000–0x20207FFF | 0x08080000–0x08081FFF | 0x00B40000–0x00B43FFF | 0x00540000–0x00541555 |
| L2 ROM2 | 0x20280000–0x202BFFFF | 0x080A0000–0x080AFFFF | 0x00B60000–0x00B7FFFF | 0x00560000–0x0056AAAF |

1 All L2 RAM/ROM blocks are subdivided into eight banks.

2 For ADSP-SC58x products, the L2 Boot ROM0 byte address space is 0x 0000 0000–0x 0000 7FFF.

3 L2 Boot ROM address for ADSP-2158x products.

##### Table 6. SHARC+ L1 Memory in Multiprocessor Space

|  |  | Memory Block | Byte Address Space for Arm Cortex-A5 and SHARC+ | Normal Word Address Space for SHARC+ |
| --- | --- | --- | --- | --- |
| L1 memory of SHARC1 in multiprocessor space | Address via Slave1 Port | Block 0 | 0x28240000–0x2826FFFF | 0x0A090000–0xA09BFFF |
|  |  | Block 1 | 0x282C0000–0x282EFFFF | 0x0A0B0000–0xA0BBFFF |
|  |  | Block 2 | 0x28300000–0x2831FFFF | 0x0A0C0000–0x0A0C7FFF |
|  |  | Block 3 | 0x28380000–0x2839FFFF | 0x0A0E0000–0x0A0E7FFF |
|  | Address via Slave2 Port | Block 0 | 0x28640000–0x2866FFFF | 0x0A190000–0x0A19BFFF |
|  |  | Block 1 | 0x286C0000–0x286EFFFF | 0x0A1B0000–0x0A1BBFFF |
|  |  | Block 2 | 0x28700000–0x2871FFFF | 0x0A1C0000–0x0A1C7FFF |
|  |  | Block 3 | 0x28780000–0x2879FFFF | 0x0A1E0000–0x0A1E7FFF |
| L1 memory of SHARC2 in multiprocessor space | Address via Slave1 Port | Block 0 | 0x28A40000–0x28A6FFFF | 0x0A290000–0x0A29BFFF |
|  |  | Block 1 | 0x28AC0000–0x28AEFFFF | 0x0A2B0000–0x0A2BBFFF |
|  |  | Block 2 | 0x28B00000–0x28B1FFFF | 0x0A2C0000–0x0A2C7FFF |
|  |  | Block 3 | 0x28B80000–0x28B9FFFF | 0x0A2E0000–0x0A2E7FFF |
|  | Address via Slave2 Port | Block 0 | 0x28E40000–0x28E6FFFF | 0x0A390000–0x0A39BFFF |
|  |  | Block 1 | 0x28EC0000–0x28EEFFFF | 0x0A3B0000–0x0A3BBFFF |
|  |  | Block 2 | 0x28F00000–0x28F1FFFF | 0x0A3C0000–0x0A3C7FFF |
|  |  | Block 3 | 0x28F80000–0x28F9FFFF | 0x0A3E0000–0x0A3E7FFF |

Table 7. Memory Map of Mapped I/Os1

|  | Byte Address Space Arm Cortex-A5: Data Access and Instruction Fetch; SHARC+: Data Access | Normal Word Address Space SHARC+ Data Access | VISA Address Space SHARC+ Instruction Fetch | ISA Address Space SHARC+ Instruction Fetch |
| --- | --- | --- | --- | --- |
| SMC Bank 0 (64 MB) | 0x40000000–0x43FFFFFF | 0x01000000–0x01FFFFFF | 0x00F00000–0x00F3FFFF | 0x00700000–0x0073FFFF |
| SMC Bank 1 (64 MB) | 0x44000000–0x47FFFFFF | Not applicable | Not applicable | Not applicable |
| SMC Bank 2 (64 MB) | 0x48000000–0x4BFFFFFF | Not applicable | Not applicable | Not applicable |
| SMC Bank 3 (64 MB) | 0x4C000000–0x4FFFFFFF | Not applicable | Not applicable | Not applicable |
| PCIe Data (256 MB) | 0x50000000–0x5007FFFF | 0x02000000–0x03FFFFFF | 0x00F40000–0x00F7FFFF | 0x00740000–0x0077FFFF |
|  | 0x50080000–0x5017FFFF |  | Not applicable |  |
|  | 0x50180000–0x57FFFFFF |  | Not applicable | Not applicable |
|  | 0x58000000–0x5FFFFFFF | Not applicable | Not applicable | Not applicable |
| SPI2 Memory (512 MB) | 0x60000000–0x600FFFFF | 0x04000000–0x07FFFFFF | 0x00F80000–0x00FFFFFF | 0x00780000–0x007FFFFF |
|  | 0x60100000–0x602FFFFF |  | Not applicable |  |
|  | 0x60300000–0x6FFFFFFF |  | Not applicable | Not applicable |
|  | 0x70000000–0x7FFFFFFF | Not applicable | Not applicable | Not applicable |

1 The Arm Cortex-A5 can access the entire byte address space. The SHARC+ VISA/ISA address space for instruction fetch and the normal word address space for data access do not cover the entire byte address space.

##### Table 8. DMC Memory Map1

|  | Byte Address Space Arm Cortex-A5: Data Access and Instruction Fetch; SHARC+: Data Access | Normal Word Address Space SHARC+ Data Access | VISA Address Space SHARC+ Instruction Fetch | ISA Address Space SHARC+ Instruction Fetch |
| --- | --- | --- | --- | --- |
| DMC0 (1 GB) | 0x80000000–0x805FFFFF | 0x10000000–0x17FFFFFF | Not applicable | 0x00400000–0x004FFFFF |
|  | 0x80600000–0x809FFFFF |  | Not applicable | Not applicable |
|  | 0x80A00000–0x80FFFFFF |  | 0x00800000–0x00AFFFFF | Not applicable |
|  | 0x81000000–0x9FFFFFFF |  | Not applicable | Not applicable |
|  | 0xA0000000–0xBFFFFFFF | Not applicable | Not applicable | Not applicable |
| DMC1 (1 GB) | 0xC0000000–0xC05FFFFF | 0x18000000–0x1FFFFFFF | Not applicable | 0x00600000–0x006FFFFF |
|  | 0xC0600000–0xC09FFFFF |  | Not applicable | Not applicable |
|  | 0xC0A00000–0xC0FFFFFF |  | 0x00C00000–0x00EFFFFF | Not applicable |
|  | 0xC1000000–0xDFFFFFFF |  | Not applicable | Not applicable |
|  | 0xE0000000–0xFFFFFFFF | Not applicable | Not applicable | Not applicable |

1 The Arm Cortex-A5 can access the entire byte address space. The SHARC+ VISA/ISA address space for instruction fetch and the normal word address space for data access do not cover the entire byte address space.

System Crossbars (SCBs)

The system crossbars (SCBs) are the fundamental building blocks of a switch-fabric style for on-chip system bus intercon- nection. The SCBs connect system bus masters to system bus slaves, providing concurrent data transfer between multiple bus masters and multiple bus slaves. A hierarchical model—built from multiple SCBs—provides a power and area efficient sys- tem interconnection.

The SCBs provide the following features:

- Highly efficient, pipelined bus transfer protocol for sus- tained throughput

- Full-duplex bus operation for flexibility and reduced latency

- Concurrent bus transfer support to allow multiple bus masters to access bus slaves simultaneously

- Protection model (privileged/secure) support for selective bus interconnect protection

#### Direct Memory Access (DMA)

The processors use direct memory access (DMA) to transfer data within memory spaces or between a memory space and a peripheral. The processors can specify data transfer operations and return to normal processing while the fully integrated DMA controller carries out the data transfers independent of proces- sor activity.

DMA transfers can occur between memory and a peripheral or between one memory and another memory. Each memory to memory DMA stream uses two channels: one channel is the source channel and the second is the destination channel.

All DMA channels can transport data to and from all on-chip and off-chip memories. Programs can use two types of DMA transfers: descriptor-based or register-based.

Register-based DMA allows the processors to program DMA control registers directly to initiate a DMA transfer. On comple- tion, the DMA control registers automatically update with original setup values for continuous transfer. Descriptor-based DMA transfers require a set of parameters stored within mem- ory to initiate a DMA sequence. Descriptor-based DMA transfers allow multiple DMA sequences to be chained together. Program a DMA channel to set up and start another DMA transfer automatically after the current sequence completes.

The DMA engine supports the following DMA operations:

  - A single linear buffer that stops on completion

  - A linear buffer with negative, positive, or zero stride length

  - A circular autorefreshing buffer that interrupts when each buffer becomes full

  - A similar circular buffer that interrupts on fractional buf- fers, such as at the halfway point

  - The 1D DMA uses a set of identical ping pong buffers defined by a linked ring of two-word descriptor sets, each containing a link pointer and an address

  - The 1D DMA uses a linked list of four-word descriptor sets containing a link pointer, an address, a length, and a configuration

  - The 2D DMA uses an array of one-word descriptor sets, specifying only the base DMA address

  - The 2D DMA uses a linked list of multiword descriptor sets, specifying all configurable parameters

##### Memory Direct Memory Access (MDMA)

The processor supports various MDMA operations, including,

  - Standard bandwidth MDMA channels with CRC protec- tion (32-bit bus width, run on SCLK0)

  - Enhanced bandwidth MDMA channel (32-bit bus width, runs on SYSCLK)

  - Maximum bandwidth MDMA channels (64-bit bus width, run on SYCLK, one channel can be assigned to the FFT accelerator)

##### Extended Memory DMA

Extended memory DMA supports various operating modes such as delay line (which allows processor reads and writes to external delay line buffers and to the external memory) with limited core interaction and scatter/gather DMA (writes to and from noncontiguous memory blocks).

#### Cyclic Redundant C ode (CRC) Protection

The cyclic redundant codes (CRC) protection modules allow system software to calculate the signature of code, data, or both in memory, the content of memory-mapped registers, or peri- odic communication message objects. Dedicated hardware circuitry compares the signature with precalculated values and triggers appropriate fault events.

For example, every 100 ms the system software initiates the sig- nature calculation of the entire memory contents and compares these contents with expected, precalculated values. If a mis- match occurs, a fault condition is generated through the processor core or the trigger routing unit.

The CRC is a hardware module based on a CRC32 engine that computes the CRC value of the 32-bit data-words presented to it. The source channel of the memory to memory DMA (in memory scan mode) provides data. The data can be optionally forwarded to the destination channel (memory transfer mode). The main features of the CRC peripheral are as follows:

  - Memory scan mode

  - Memory transfer mode

  - Data verify mode

  - Data fill mode

  - User-programmable CRC32 polynomial

  - Bit/byte mirroring option (endianness)

  - Fault/error interrupt mechanisms

  - 1D and 2D fill block to initialize an array with constants

  - 32-bit CRC signature of a block of a memory or an MMR block

#### Event Handling

The processors provide event handling that supports both nest- ing and prioritization. Nesting allows multiple event service routines to be active simultaneously. Prioritization ensures that servicing a higher priority event takes precedence over servicing a lower priority event.

The processors provide support for five different types of events:

  - An emulation event causes the processors to enter emula- tion mode, allowing command and control of the processors through the JTAG interface.

  - A reset event resets the processors.

  - An exceptions event occur synchronously to program flow (in other words, the exception is taken before the instruc- tion is allowed to complete). Conditions triggered on the one side by the SHARC+ core, such as data alignment (SIMD/long word) or compute violations (fixed or floating

point), and illegal instructions cause core exceptions. Con- ditions triggered on the other side by the SEC, such as error correcting codes (ECC)/parity/watchdog/system clock, cause system exceptions.

- An interrupts event occurs asynchronously to program flow. They are caused by input signals, timers, and other peripherals, as well as by an explicit software instruction.

##### System Event Controller (SEC)

Both SHARC+ cores feature a system event controller. The SEC features include the following:

- Comprehensive system event source management includ- ing interrupt enable, fault enable, priority, core mapping, and source grouping

- A distributed programming model where each system event source control and all status fields are independent of each other

- Determinism where all system events have the same propa- gation delay and provide unique identification of a specific system event source

- A slave control port that provides access to all SEC registers for configuration, status, and interrupt/fault services

- Global locking that supports a register level protection model to prevent writes to locked registers

- Fault management including fault action configuration, time out, external indication, and system reset

#### Trigger Routing Unit (TRU)

The trigger routing unit (TRU) provides system-level sequence control without core intervention. The TRU maps trigger masters (generators of triggers) to trigger slaves (receivers of triggers). Slave endpoints can be configured to respond to trig- gers in various ways. Common applications enabled by the TRU include,

- Automatically triggering the start of a DMA sequence after a sequence from another DMA channel completes

- Software triggering

- Synchronization of concurrent activities

## SECURITY FEATURES

The following sections describe the security features of the ADSP-SC58x/ADSP-2158x processors.

#### Arm TrustZone

The ADSP-SC58x processors provide TrustZone technology that is integrated into the Arm Cortex-A5 processors. The TrustZone technology enables a secure state that is extended throughout the system fabric.

#### Cryptographic Hardware Accelerators

The ADSP-SC58x/ADSP-2158x processors support standards- based hardware accelerated encryption, decryption, authentica- tion, and true random number generation.

Support for the hardware-accelerated cryptographic ciphers includes the following:

- AES in ECB, CBC, ICM, and CTR modes with 128-bit, 192-bit, and 256-bit keys

- DES in ECB and CBC mode with 56-bit key

- 3DES in ECB and CBC mode with 3x 56-bit key

- ARC4 in stateful, stateless mode, up to 128-bit key

Support for the hardware accelerated hash functions includes the following:

- SHA-1

- SHA-2 with 224-bit and 256-bit digests

- HMAC transforms for SHA-1 and SHA-2

- MD5

Public key accelerator (PKA) is available to offload computation intensive public key cryptography operations.

Both a hardware-based nondeterministic random number gen- erator and pseudorandom number generator are available.

Secure boot is also available with 224-bit elliptic curve digital signatures ensuring integrity and authenticity of the boot stream. Optionally, ensuring confidentiality through AES-128 encryption is available.

Employ secure debug to allow only trusted users to access the system with debug tools.

CAUTION

This product includes security features that can be used to protect embedded nonvolatile memory contents and prevent execution of unauthorized code. When security is enabled on this device (either by the ordering party or the subsequent receiving parties), the ability of Analog Devices to conduct failure analysis on returned devices is limited. Contact Analog Devices for details on the failure analysis limitations for this device.

#### System Protection Unit (SPU)

The system protection unit (SPU) guards against accidental or unwanted access to an MMR space of the peripheral by provid- ing a write protection mechanism. The user can choose and configure the protected peripherals as well as configure which of the four system MMR masters (two SHARC+ cores, memory DMA, and CoreSight debug) the peripherals are guarded against.

The SPU is also part of the security infrastructure. Along with providing write protection functionality, the SPU is employed to define which resources in the system are secure or nonsecure and to block access to secure resources from nonsecure masters.

#### System Memory Protection Unit (SMPU)

Synonymously, the system memory protection unit (SMPU) provides memory protection against read and/or write transac- tions to defined regions of memory. There are SMPU units in the ADSP-SC58x/ADSP-2158x processors for each memory space, except for SHARC L1 and SPI direct memory slave.

The SMPU is also part of the security infrastructure. It allows the user to protect against arbitrary read and/or write transac- tions and allows regions of memory to be defined as secure and prevent nonsecure masters from accessing those memory regions.

## SECURITY FEATURES DISCLAIMER

To our knowledge, the Security Features, when used in accor- dance with the data sheet and hardware reference manual specifications, provide a secure method of implementing code and data safeguards. However, Analog Devices does not guaran- tee that this technology provides absolute security.

ACCORDINGLY, ANALOG DEVICES HEREBY DISCLAIMS ANY AND ALL EXPRESS AND IMPLIED WARRANTIES THAT THE SECURITY FEATURES CANNOT BE BREACHED, COMPROMISED, OR OTHERWISE CIRCUM- VENTED AND IN NO EVENT SHALL ANALOG DEVICES BE LIABLE FOR ANY LOSS, DAMAGE, DESTRUCTION, OR RELEASE OF DATA, INFORMATION, PHYSICAL PROP- ERTY, OR INTELLECTUAL PROPERTY.

## SAFETY FEATURES

The ADSP-SC58x/ADSP-2158x processors are designed to sup- port functional safety applications. While the level of safety is mainly dominated by the system concept, the following primi- tives are provided by the processors to build a robust safety concept.

#### Multiparity Bit Protected SHARC+ Core L1 Memories

In the SHARC+ core L1 memory space, whether SRAM or cache, multiple parity bits protect each word to detect the single event upsets that occur in all RAMs. Parity does not protect the cache tags.

#### Error Correcting Codes (ECC) Protected L2 Memories

Error correcting codes (ECC) correct single event upsets. A sin- gle error correct-double error detect (SEC-DED) code protects the L2 memory. By default, ECC is enabled, but it can be dis- abled on a per bank basis. Single-bit errors correct transparently. If enabled, dual-bit errors can issue a system event or fault. ECC protection is fully transparent to the user, even if L2 memory is read or written by 8-bit or 16-bit entities.

#### Cyclic Redundant Code (CRC) Protected Memories

While parity bit and ECC protection mainly protect against ran- dom soft errors in L1 and L2 memory cells, the cyclic redundant code (CRC) engines can protect against systematic errors (pointer errors) and static content (instruction code) of L1, L2, and even L3 memories (DDR2, LPDDR). The processors feature two CRC engines that are embedded in the memory to memory DMA controllers.

CRC checksums can be calculated or compared automatically during memory transfers, or one or multiple memory regions can be continuously scrubbed by a single DMA work unit as per DMA descriptor chain instructions. The CRC engine also pro- tects data loaded during the boot process.

#### Signal Watchdogs

The eight general-purpose timers feature modes to monitor off- chip signals. The watchdog period mode monitors whether external signals toggle with a period within an expected range. The watchdog width mode monitors whether the pulse widths of external signals are within an expected range. Both modes help to detect undesired toggling or lack of toggling of system level signals.

#### System Event Controller (SEC)

Besides system events, the system event controller (SEC) further supports fault management including fault action configuration as timeout, internal indication by system interrupt, or external indication through the SYS_FAULT pin and system reset.

## PROCESSOR PERIPHERALS

The following sections describe the peripherals of the ADSP- SC58x/ADSP-2158x processors.

#### Dynamic Memory Controller (DMC)

The 16-bit dynamic memory controller (DMC) interfaces to:

  - LPDDR1 (JESD209A) maximum frequency 200 MHz, DDRCLK (64 Mb to 2 Gb)

  - DDR2 (JESD79-2E) maximum frequency 400 MHz, DDRCLK (256 Mb to 4 Gb)

  - DDR3 (JESD79-3E) maximum frequency 450 MHz, DDRCLK (512 Mb to 8 Gb)

  - DDR3L (1.5 V compatible only) maximum frequency 450 MHz, DDRCLK (512 Mb to 8 Gb)

See Table 8 for the DMC memory map.

#### Digital Audio Interface (DAI)

The processors support two mirrored digital audio interface (DAI) units. Each DAI can connect various peripherals to any of the DAI pins (DAI_PIN20–DAI_PIN01).

The application code makes these connections using the signal routing unit (SRU), shown in Figure 1.

The SRU is a matrix routing unit (or group of multiplexers) that enables the peripherals provided by the DAI to interconnect under software control. This functionality allows easy use of the DAI associated peripherals for a wider variety of applications by using a larger set of algorithms than is possible with nonconfig- urable signal paths.

The DAI includes the peripherals described in the following sec- tions (SPORTs, ASRC, S/PDIF, and PCG). DAI Pin Buffers 20 and 19 can change the polarity of the input signals. Most signals of the peripherals belonging to different DAIs cannot be inter- connected, with few exceptions.

The DAI_PINx pin buffers may also be used as GPIO pins. DAI input signals allow the triggering of interrupts on the rising edge, the falling edge, or both edges.

See the “Digital Audio Interface (DAI)” chapter of the ADSP- SC58x/ADSP-2158x SHARC+ Processor Hardware Reference for complete information on the use of the DAIs and SRUs.

#### Serial Ports (SPORTs)

The processors feature eight synchronous full serial ports. These ports provide an inexpensive interface to a wide variety of digi- tal and mixed-signal peripheral devices. These devices include Analog Devices AD19xx and ADAU19xx family of audio codecs, analog-to-digital converters (ADCs) and digital-to-ana- log converters (DACs). Two data lines, a clock, and frame sync make up the serial ports. The data lines can be programmed to either transmit or receive data and each data line has a dedicated DMA channel.

An individual full SPORT module consists of two inde- pendently configurable SPORT halves with identical functionality. Two bidirectional data lines—primary (0) and secondary (1)—are available per SPORT half and are configu- rable as either transmitters or receivers. Therefore, each SPORT half permits two unidirectional streams into or out of the same SPORT. This bidirectional functionality provides greater flexibility for serial communications. For full-duplex configura- tion, one half SPORT provides two transmit signals, while the other half SPORT provides the two receive signals. The frame sync and clock are shared.

Serial ports operate in the following six modes:

- Standard DSP serial mode

- Multichannel time division multiplexing (TDM) mode

- I2S mode

- Packed I2S mode

- Left justified mode

- Right justified mode

#### Asynchronous Sample Rate Converter (ASRC)

The asynchronous sample rate converter (ASRC) contains eight ASRC blocks. It is the same core in the AD1896 192 kHz stereo asynchronous sample rate converter. The ASRC provides up to 140 dB signal-to-noise ratio (SNR). The ASRC block performs synchronous or asynchronous sample rate conversion across independent stereo channels, without using internal processor resources. The ASRC blocks can also be configured to operate together to convert multichannel audio data without phase mis- matches. Finally, the ASRC can clean up audio data from jittery clock sources such as the S/PDIF receiver.

#### S/PDIF-Compatible Digital Audio Receiver/Transmitter

The Sony/Philips Digital Interface Format (S/PDIF) is a stan- dard audio data transfer format that allows the transfer of digital audio signals from one device to another without converting them to an analog signal. There are two S/PDIF transmit/receive

blocks on the processor. The digital audio interface carries three types of information: audio data, nonaudio data (compressed data), and timing information.

The S/PDIF interface supports one stereo channel or com- pressed audio streams. The S/PDIF transmitter and receiver are AES3 compliant and support the sample rate from 24 KHz to 192 KHz. The S/PDIF receiver supports professional jitter standards.

The S/PDIF receiver/transmitter has no separate DMA chan- nels. It receives audio data in serial format and converts it into a biphase encoded signal. The serial data input to the receiver/transmitter can be formatted as left justified, I2S, or right justified with word widths of 16, 18, 20, or 24 bits. The serial data, clock, and frame sync inputs to the S/PDIF receiver/transmitter are routed through the signal routing unit (SRU). They can come from various sources, such as the SPORTs, external pins, and the precision clock generators (PCGs), and are controlled by the SRU control registers.

#### Precision Clock Generators (PCG)

The precision clock generators (PCG) consist of four units: units A/B located in the DAI0 block, and units C/D located in the DAI1 block. The PCG can generate a pair of signals (clock and frame sync) derived from a clock input signal (CLKIN1-0, SCLK0, or DAI pin buffer). Each unit can also access the oppo- site DAI unit. All units are identical in functionality and operate independently of each other. The two signals generated by each unit are normally used as a serial bit clock/frame sync pair.

#### Enhanced Parallel Peripheral Interface (EPPI)

The processors provide an enhanced parallel peripheral inter- face (EPPI) that supports data widths up to 24 bits. The EPPI supports direct connection to TFT LCD panels, parallel ADCs and DACs, video encoders and decoders, image sensor mod- ules, and other general-purpose peripherals.

The features supported in the EPPI module include the following:

- Programmable data length of 8 bits, 10 bits, 12 bits, 14 bits, 16 bits, 18 bits, and 24 bits per clock.

- Various framed, nonframed, and general-purpose operat- ing modes. Frame syncs can be generated internally or can be supplied by an external device.

- ITU-656 status word error detection and correction for ITU-656 receive modes and ITU-656 preamble and status word decoding.

- Optional packing and unpacking of data to/from 32 bits from/to 8 bits, 16 bits, and 24 bits. If packing/unpacking is enabled, configure endianness to change the order of pack- ing/unpacking of the bytes/words.

- RGB888 can be converted to RGB666 or RGB565 for trans- mit modes.

- Various deinterleaving/interleaving modes for receiv- ing/transmitting 4:2:2 YCrCb data.

- Configurable LCD data enable output available on Frame Sync 3.

#### Universal Asynchronous Receiver/Transmitter (UART) Ports

The processors provide three full-duplex universal asynchro- nous receiver/transmitter (UART) ports, fully compatible with PC standard UARTs. Each UART port provides a simplified UART interface to other peripherals or hosts, supporting full- duplex, DMA supported, asynchronous transfers of serial data. A UART port includes support for five to eight data bits as well as no parity, even parity, or odd parity.

Optionally, an additional address bit can be transferred to inter- rupt only addressed nodes in multidrop bus (MDB) systems. A frame is terminated by a configurable number of stop bits.

The UART ports support automatic hardware flow control through the clear to send (CTS) input and request to send (RTS) output with programmable assertion first in, first out (FIFO) levels.

To help support the Local Interconnect Network (LIN) proto- cols, a special command causes the transmitter to queue a break command of programmable bit length into the transmit buffer. Similarly, the number of stop bits can be extended by a pro- grammable interframe space.

#### Serial Peripheral Interface (SPI) Ports

The processors have three industry-standard SPI-compatible ports that allow the processors to communicate with multiple SPI-compatible devices.

The baseline SPI peripheral is a synchronous, four-wire inter- face consisting of two data pins, one device select pin, and a gated clock pin. The two data pins allow full-duplex operation to other SPI-compatible devices. An extra two (optional) data pins are provided to support quad SPI operation. Enhanced modes of operation, such as flow control, fast mode, and dual I/O mode (DIOM), are also supported. A direct memory access (DMA) mode allows for transferring several words with mini- mal central processing unit (CPU) interaction.

With a range of configurable options, the SPI ports provide a glueless hardware interface with other SPI-compatible devices in master mode, slave mode, and multimaster environments. The SPI peripheral includes programmable baud rates, clock phase, and clock polarity. The peripheral can operate in a multi- master environment by interfacing with several other devices, acting as either a master device or a slave device. In a multimas- ter environment, the SPI peripheral uses open-drain outputs to avoid data bus contention. The flow control features enable slow slave devices to interface with fast master devices by providing an SPI ready pin (SPI_RDY) which flexibly controls the transfers.

The baud rate and clock phase/polarities of the SPI port are pro- grammable. The port has integrated DMA channels for both transmit and receive data streams.

#### Link Ports (LP)

Two 8-bit wide link ports (LP) can connect to the link ports of other DSPs or peripherals. LP are bidirectional ports that have eight data lines, an acknowledge line, and a clock line.

#### ADC Control Module (ACM) Interface

The ADC control module (ACM) provides an interface that synchronizes the controls between the processors and an ADC. The analog-to-digital conversions are initiated by the proces- sors, based on external or internal events.

The ACM allows for flexible scheduling of sampling instants and provides precise sampling signals to the ADC.

The ACM synchronizes the ADC conversion process, generat- ing the ADC controls, the ADC conversion start signal, and other signals. The actual data acquisition from the ADC is done by an internal DAI routing of the ACM with the SPORT0 block.

The processors interface directly to many ADCs without any glue logic required.

#### Phase Pulse Width Modulator (PWM) Units

The pulse width modulator (PWM) module is a flexible and programmable waveform generator. With minimal CPU inter- vention, the PWM generates complex waveforms for motor control, pulse coded modulation (PCM), DAC conversions, power switching, and power conversion. The PWM module has four PWM pairs capable of 3-phase PWM generation for source inverters for ac induction and dc brushless motors.

Each of the three 3-phase PWM generation units features the following:

  - 16-bit center-based PWM generation unit

  - Programmable PWM pulse width

  - Single update mode with an option for asymmetric duty

  - Programmable dead time and switching frequency

  - Programmable dead time per channel

  - Twos complement implementation which permits smooth transition to full on and full off states

  - Dedicated asynchronous PWM shutdown signal

#### Ethernet Media Access Controller (EMAC)

The processor features two ethernet media access controllers (EMACs): 10/100 Ethernet and 10/100/1000/AVB Ethernet with precision time protocol IEEE 1588.

The processors can directly connect to a network through embedded fast EMAC that supports 10-BaseT (10 Mb/sec),

- BaseT (100 Mb/sec) and 1000-BaseT (1 Gb/sec) operations. The 10/100 EMAC peripheral on the processors is fully compli- ant to the IEEE 802.3-2002 standard. The peripheral provides programmable features designed to minimize supervision, bus use, or message processing by the rest of the processor system.

Some standard features of the EMAC are as follows:

  - Support and RMII/RGMII protocols for external PHYs

  - Full-duplex and half-duplex modes

  - Media access management (in half-duplex operation)

  - Flow control

  - Station management, including the generation of MDC/MDIO frames for read/write access to PHY registers

Some advanced features of the EMAC are as follows:

- Automatic checksum computation of IP header and IP payload fields of receive frames

- Independent 32-bit descriptor driven receive and transmit DMA channels

- Frame status delivery to memory through DMA, including frame completion semaphores for efficient buffer queue management in software

- Transmit DMA support for separate descriptors for MAC header and payload fields to eliminate buffer copy operations

- Convenient frame alignment modes

- 47 MAC management statistics counters with selectable clear on read behavior and programmable interrupts on half maximum value

- Advanced power management

- Magic packet detection and wakeup frame filtering

- Support for 802.3Q tagged VLAN frames

- Programmable MDC clock rate and preamble suppression

##### Audio Video Bridging (AVB) Support (10/100/1000 EMAC Only)

The 10/100/1000 EMAC supports the following audio video (AVB) features:

- Separate channels or queues for AV data transfer in 100 Mbps and 1000 Mbps modes

- IEEE 802.1-Qav specified credit-based shaper (CBS) algo- rithm for the additional transmit channels

- Configuring up to two additional channels (Channel 1 and Channel 2) on the transmit and receive paths for AV traffic. Channel 0 is available by default and carries the legacy best effort Ethernet traffic on the transmit side.

- Separate DMA, transmit and receive FIFO for AVB latency class

- Programmable control to route received VLAN tagged non AV packets to channels or queues

##### Precision Time Protocol (PTP) IEEE 1588 Support

The IEEE 1588 standard is a precision clock synchronization protocol for networked measurement and control systems. The processors include hardware support for IEEE 1588 with an integrated precision time protocol synchronization engine (PTP_TSYNC).

This engine provides hardware assisted time stamping to improve the accuracy of clock synchronization between PTP nodes. The main features of the engine are as follows:

- Support for both IEEE 1588-2002 and IEEE 1588-2008 pro- tocol standards

- Hardware assisted time stamping capable of up to 12.5 ns resolution

- Lock adjustment

- Automatic detection of IPv4 and IPv6 packets, as well as PTP messages

- Multiple input clock sources (SCLK0, RGMII, RMII, RMII clock, and external clock)

- Programmable pulse per second (PPS) output

- Auxiliary snapshot to time stamp external events

#### Controller Area Network (CAN)

There are two controller area network (CAN) modules. A CAN controller implements the CAN 2.0B (active) protocol. This protocol is an asynchronous communications protocol used in both industrial and automotive control systems. The CAN pro- tocol is well suited for control applications due to the capability to communicate reliably over a network. This is because the protocol incorporates CRC checking, message error tracking, and fault node confinement.

The CAN controller offers the following features:

- 32 mailboxes (8 receive only, 8 transmit only, 16 configu- rable for receive or transmit)

- Dedicated acceptance masks for each mailbox

- Additional data filtering on the first two bytes

- Support for both the standard (11-bit) and extended (29- bit) identifier (ID) message formats

- Support for remote frames

- Active or passive network support

- Interrupts, including transmit and receive complete, error, and global

An additional crystal is not required to supply the CAN clock because it is derived from a system clock through a programma- ble divider.

#### Timers

The processors include several timers that are described in the following sections.

##### General-Purpose (GP) Timers (TIMER)

There is one general-purpose (GP) timer unit, providing eight general-purpose programmable timers. Each timer has an exter- nal pin that can be configured either as PWM or timer output, as an input to clock the timer, or as a mechanism for measuring pulse widths and periods of external events. These timers can be synchronized to an external clock input on the TM_TMR[n] pins, an external TM_CLK input pin, or to the internal SCLK0.

These timer units can be used in conjunction with the UARTs and the CAN controller to measure the width of the pulses in the data stream to provide a software autobaud detect function for the respective serial channels.

The GP timers can generate interrupts to the processor core, providing periodic events for synchronization to either the sys- tem clock or to external signals. Timer events can also trigger other peripherals via the TRU (for instance, to signal a fault). Each timer can also be started and/or stopped by any TRU mas- ter without core intervention.

##### Watchdog Timer (WDT)

Two on-chip software watchdog timers (WDT) can be used by the Arm Cortex-A5 and/or SHARC+ cores. A software watch- dog can improve system availability by forcing the processors to a known state, via a general-purpose interrupt, or a fault, if the timer expires before being reset by software.

The programmer initializes the count value of the timer, enables the appropriate interrupt, then enables the timer. Thereafter, the software must reload the counter before it counts down to zero from the programmed value, protecting the system from remaining in an unknown state where software that normally resets the timer stops running due to an external noise condi- tion or software error.

#### General-Purpose Counters (CNT)

A 32-bit counter (CNT) is provided that can operate in general- purpose up/down count modes and can sense 2-bit quadrature or binary codes as typically emitted by industrial drives or man- ual thumbwheels. Count direction is either controlled by a level- sensitive input pin or by two edge detectors.

A third counter input can provide flexible zero marker support and can input the push button signal of thumbwheel devices. All three CNT0 pins have a programmable debouncing circuit.

Internal signals forwarded to a GP timer enable this timer to measure the intervals between count events. Boundary registers enable auto-zero operation or simple system warning by inter- rupts when programmed count values are exceeded.

#### PCI Express (PCIe)

A PCI express interface (PCIe) is available on some product variants (see Table 2 and Table 3). This single, bidirectional lane can be configured to be either a root complex (RC) or end point (EP) system. The PCIe interface has the following features:

  - Designed to be compliant with the PCI Express Base Specification 3.0

  - Support for transfers at either 2.5 Gbps (Gen 1) or 5.0 Gbps (Gen 2) in each direction

  - Support for 8b/10b encode and decode

  - Lane reversal and lane polarity inversion

  - Flow control of data in both the transmit and receive directions

  - Support for removal of corrupted packets for error detec- tion and recovery

  - Maximum transaction payload of 256 bytes

#### Housekeeping Analog-to-Digital Converter (HADC)

The housekeeping analog-to-digital converter (HADC) pro- vides a general-purpose, multichannel successive approximation ADC. It supports the following set of features:

  - 12-bit ADC core with built in sample-and-hold.

  - 8 single-ended input channels that can be extended to 15 channels by adding an external channel multiplexer.

  - Throughput rates up to 1 MSPS.

  - Single external reference with analog inputs between 0 V and 3.3 V.

  - Selectable ADC clock frequency including the ability to program a prescaler.

  - Adaptable conversion type; allows single or continuous conversion with option of autoscan.

  - Autosequencing capability with up to 15 autoconversions in a single session. Each conversion can be programmed to select 1 to 15 input channels.

  - 16 data registers (individually addressable) to store conver- sion values.

#### USB 2.0 On the Go (OTG) Dual-Role Device Controller

There are two USB modules + PHY. USB0 supports HS/FS/LS USB 2.0 on the go (OTG) and USB1 supports HS/FS USB 2.0 only and can be programmed to be a host or device.

The USB 2.0 OTG dual-role device controller provides a low cost connectivity solution in industrial applications, as well as consumer mobile devices such as cell phones, digital still cam- eras, and MP3 players. The USB 2.0 controller allows these devices to transfer data using a point to point USB connection without the need for a PC host. The module can operate in a tra- ditional USB peripheral only mode as well as the host mode presented in the OTG supplement to the USB 2.0 specification.

The USB clock is provided through a dedicated external crystal or crystal oscillator.

The USB OTG dual-role device controller includes a PLL with programmable multipliers to generate the necessary internal clocking frequency for the USB.

#### Media Local Bus (Media LB)

The automotive model has a Media LB (MLB) slave interface that allows the processors to function as a media local bus device. It includes support for both 3-pin and 6-pin media local bus protocols. The MLB 3-pin configuration supports speeds up to 1024 × FS. The MLB 6-pin configuration supports a speed of 2048 × FS. The MLB also supports up to 64 logical channels with up to 468 bytes of data per MLB frame.

The MLB interface supports MOST25, MOST50, and MOST150 data rates and operates in slave mode only.

#### 2-Wire Controller Interface (TWI)

The processors include three 2-wire interface (TWI) modules that provide a simple exchange method of control data between multiple devices. The TWI module is compatible with the widely used I2C bus standard. The TWI module offers the capabilities of simultaneous master and slave operation and support for both 7-bit addressing and multimedia data arbitra- tion. The TWI interface utilizes two pins for transferring clock (TWI_SCL) and data (TWI_SDA) and supports the protocol at speeds up to 400 kb/sec. The TWI interface pins are compatible with 5 V logic levels.

Additionally, the TWI module is fully compatible with serial camera control bus (SCCB) functionality for easier control of various CMOS camera sensor devices.

#### General-Purpose I/O (GPIO)

Each general-purpose port pin can be individually controlled by manipulating the port control, status, and interrupt registers:

- GPIO direction control register specifies the direction of each individual GPIO pin as input or output.

- GPIO control and status registers have a write one to mod- ify mechanism that allows any combination of individual GPIO pins to be modified in a single instruction, without affecting the level of any other GPIO pins.

- GPIO interrupt mask registers allow each individual GPIO pin to function as an interrupt to the processors. GPIO pins defined as inputs can be configured to generate hardware interrupts, while output pins can be triggered by software interrupts.

- GPIO interrupt sensitivity registers specify whether indi- vidual pins are level or edge sensitive and specify, if edge sensitive, whether the rising edge or both the rising and falling edges of the signal are significant.

#### Pin Interrupts

Every port pin on the processors can request interrupts in either an edge sensitive or a level sensitive manner with programmable polarity. Interrupt functionality is decoupled from GPIO opera- tion. Six system-level interrupt channels (PINT0–PINT5) are reserved for this purpose. Each of these interrupt channels can manage up to 32 interrupt pins. The assignment from pin to interrupt is not performed on a pin by pin basis. Rather, groups of eight pins (half ports) can be flexibly assigned to interrupt channels.

Every pin interrupt channel features a special set of 32-bit mem- ory-mapped registers that enable half-port assignment and interrupt management. This includes masking, identification, and clearing of requests. These registers also enable access to the respective pin states and use of the interrupt latches, regardless of whether the interrupt is masked or not. Most control registers feature multiple MMR address entries to write-one-to-set or write-one-to-clear them individually.

#### Mobile Storage Interface (MSI)

The mobile storage interface (MSI) controller acts as the host interface for multimedia cards (MMC), secure digital memory cards (SD), and secure digital input/output cards (SDIO). The MSI controller has the following features:

- Support for a single MMC, SD memory, and SDIO card

- Support for 1-bit and 4-bit SD modes

- Support for 1-bit, 4-bit, and 8-bit MMC modes

- Support for eMMC 4.3 embedded NAND flash devices

- An eleven-signal external interface with clock, command, optional interrupt, and up to eight data lines

- Integrated DMA controller

- Card interface clock generation in the clock distribution unit (CDU)

- SDIO interrupt and read wait features

## SYSTEM ACCELERATION

The following sections describe the system acceleration blocks of the ADSP-SC58x/ADSP-2158x processors.

#### FFT/IFFT Accelerator

A high performance FFT/IFFT accelerator is available to improve the overall floating-point computation power of the ADSP-SC58x/ADSP-2158x processors.

The following features are available to improve the overall per- formance of the FFT/IFFT accelerator:

- Support for the IEEE-754/854 single-precision floating- point data format.

- Automatic twiddle factor generation to reduce system bandwidth.

- Support for a vector complex multiply for windowing and frequency domain filtering.

- Ability to pipeline the data flow. This allows the accelerator to bring in a new data set while the current data set is pro- cessed and the previous data set is sent out to memory. This can provide a significant system level performance improvement.

- Ability to output the result as the magnitude squared of the complex samples.

- Dedicated, high speed DMA controller with 64-bit buses that can read and write data from any memory space.

The FFT/IFFT accelerator can run concurrently with the other accelerators on the processor.

#### Finite Impulse Response (FIR) Accelerator

The finite impulse response (FIR) accelerator consists of a 1024 word coefficient memory, a 1024 word deep delay line for

the data, and four MAC units. A controller manages the acceler- ator. The FIR accelerator runs at the peripheral clock frequency. The FIR accelerator can access all memory spaces and can run concurrently with the other accelerators on the processor.

#### Infinite Impulse Response (IIR) Accelerator

The infinite impulse response (IIR) accelerator consists of a 1440 word coefficient memory for storage of biquad coeffi- cients, a data memory for storing the intermediate data, and one MAC unit. A controller manages the accelerator. The IIR accel- erator runs at the peripheral clock frequency. The IIR accelerator can access all memory spaces and run concurrently with the other accelerators on the processor.

#### Harmonic Analysis Engine (HAE)

The harmonic analysis engine (HAE) block receives 8 kHz input samples from two source signals whose frequencies are between 45 Hz and 65 Hz. The HAE processes the input samples and produces output results. The output results consist of power quality measurements of the fundamental and up to 12 addi- tional harmonics.

#### Sinus Cardinalis (SINC) Filter

The sinus cardinalis (SINC) filter module processes four bit streams using a pair of configurable SINC filters for each bit stream. The purpose of the primary SINC filter of each pair is to produce the filtered and decimated output for the pair. The out- put can decimate any integer rate between 8 and 256 times lower than the input rate. Greater decimation allows greater removal of noise, and, therefore, greater effective number of bits (ENOB).

Optional additional filtering outside the SINC module can fur- ther increase ENOB. The primary SINC filter output is accessible through transfer to processor memory, or to another peripheral, via DMA.

Each of the four channels is also provided with a low latency secondary filter with programmable positive and negative over- range detection comparators. These limit detection events can interrupt the core, generate a trigger, or signal a system fault.

Digital Transmission Content Protection (DTCP) Contact Analog Devices for more information on DTCP. SYSTEM DESIGN

The following sections provide an introduction to system design

features and power supply issues.

#### Clock Management

The processors provide three operating modes, each with a dif- ferent performance and power profile. Control of clocking to each of the processor peripherals reduces power consumption. The processors do not support any low power operation modes. Control of clocking to each of the processor peripherals can reduce the power consumption.

#### Reset Control Unit (RCU)

Reset is the initial state of the whole processor, or the core, and

The reset target is defined as the following:

  - System reset—all functional units except the RCU are set to default states.

  - Hardware reset—all functional units are set to default states without exception. History is lost.

  - Core only reset— affects the core only. When in reset state, the core is not accessed by any bus master.

The reset source is defined as the following:

  - System reset—can be triggered by software (writing to the RCU_CTL register) or by another functional unit such as the dynamic power management (DPM) unit or any of the SEC, TRU, or emulator inputs.

  - Hardware reset—the SYS_HWRST input signal asserts active (pulled down).

  - Core only reset—affects only the core. The core is not accessed by any bus master when in reset state.

  - Trigger request (peripheral).

#### Real-Time Clock (RTC)

The real-time clock (RTC) provides a robust set of digital watch features, including current time, stopwatch, and alarm. The RTC is clocked by a 32.768 kHz crystal external to the processor. Connect the RTC0_CLKIN and RTC0_XTAL pins with external components as shown in Figure 6.

The RTC peripheral has dedicated power supply pins so it can remain powered up and clocked even when the remainder of the processor is in a low power state. The RTC provides several programmable interrupt options, including interrupt per second, minute, hour, or day clock ticks; interrupt on program- mable stopwatch countdown; or interrupt at a programmed alarm time.

is the result of a hardware or software triggered event. In this state, all control registers are set to default values and functional units are idle. Exiting a full system reset starts with the core ready to boot.

The reset control unit (RCU) controls how all the functional units enter and exit reset. Differences in functional require- ments and clocking constraints define how reset signals are

RTC0_CLKIN

R1

X1

C2

C1

RTC0_XTAL

generated. Programs must guarantee that none of the reset functions put the system into an undefined state or causes resources to stall. This is particularly important when the core resets (programs must ensure that there is no pending system activity involving the core when it is reset).

From a system perspective, reset is defined by both the reset tar- get and the reset source.

NOTE: C1 AND C2 ARE SPECIFIC TO CRYSTAL SPECIFIED FOR X1. CONTACT CRYSTAL MANUFACTURER FOR DETAILS.

Figure 6. External Components for RTC

The 32.768 kHz input clock frequency is divided down to a 1 Hz signal by a prescaler. The counter function of the timer consists of four counters: a 60 second counter, a 60 minute counter, a 24 hour counter, and a 32,768 day counter. When the alarm interrupt is enabled, the alarm function generates an interrupt when the output of the timer matches the programmed value in the alarm control register (RTC_ALARM). There are two alarms: a time of day and a day and time of that day.

The stopwatch function counts down from a programmed value, with 1 sec resolution. When the stopwatch interrupt is enabled and the counter underflows, an interrupt is generated.

#### Clock Generation Unit (CGU)

The ADSP-SC58x/ADSP-2158x processors support two inde- pendent PLLs. Each PLL is part of a clock generation unit (CGU); see Figure 8. Each CGU can be either driven externally by the same clock source or each can be driven by separate sources. This provides flexibility in determining the internal clocking frequencies for each clock domain.

Frequencies generated by each CGU are derived from a com- mon multiplier with different divider values available for each output.

The CGU generates all on-chip clocks and synchronization sig- nals. Multiplication factors are programmed to define the PLLCLK frequency.

Programmable values divide the PLLCLK frequency to generate the core clock (CCLK), the system clocks, the DDR1/DDR2/ DDR3 clock (DCLK), and the output clock (OCLK). For more information on clocking, see the ADSP-SC58x/ADSP-2158x SHARC+ Processor Hardware Reference.

Writing to the CGU control registers does not affect the behav- ior of the PLL immediately. Registers are first programmed with a new value and the PLL logic executes the changes so it transi- tions smoothly from the current conditions to the new conditions.

#### System Crystal Oscillator and USB Crystal Oscillator

The processor can be clocked by an external crystal (see

Figure 7), a sine wave input, or a buffered, shaped clock derived from an external clock oscillator. If using an external clock, it should be a TTL-compatible signal and must not be halted, changed, or operated below the specified frequency during nor- mal operation. This signal is connected to the SYS_CLKINx pin and the USB_CLKIN pin of the processor. When using an external clock, the SYS_XTALx pin and the USB_XTAL pin must be left unconnected. Alternatively, because the processor includes an on-chip oscillator circuit, an external crystal can be used.

For fundamental frequency operation, use the circuit shown in Figure 7. A parallel resonant, fundamental frequency, micro- processor grade crystal is connected across the SYS_CLKINx pin and the SYS_XTALx pin. The on-chip resistance between the SYS_CLKINx pin and the SYS_XTALx pin is in the 500 kΩ range. Further parallel resistors are typically not recommended.

The two capacitors and the series resistor, shown in Figure 7, fine tune phase and amplitude of the sine frequency. The capac- itor and resistor values shown in Figure 7 are typical values only. The capacitor values are dependent upon the load capaci- tance recommendations of the crystal manufacturer and the physical layout of the printed circuit board (PCB). The resistor value depends on the drive level specified by the crystal manu- facturer. The user must verify the customized values based on careful investigations on multiple devices over the required temperature range.

SHARC PROCESSOR

TO PLL CIRCUITRY

510ȍ

SYS_CLKINx

SYS_XTALx

2Nȍ *

FOR OVERTONE OPERATION ONLY:

18 pF*

18 pF*

NOTE: VALUES MARKED WITH * MUST BE CUSTOMIZED, DEPENDING ON THE CRYSTAL AND LAYOUT. ANALYZE CAREFULLY. FOR FREQUENCIES ABOVE 33 MHz, THE SUGGESTED CAPACITOR VALUE OF 18 pF MUST BE TREATED AS A MAXIMUM.

Figure 7. External Crystal Connection

A third overtone crystal can be used for frequencies above

25 MHz. The circuit is then modified to ensure crystal operation only at the third overtone by adding a tuned inductor circuit, shown in Figure 7. A design procedure for the third overtone operation is discussed in detail in Using Third Overtone Crys- tals with the ADSP-218x DSP (EE-168). The same recommendations can be used for the USB crystal oscillator.

#### Clock Distribution Unit (CDU)

The two CGUs each provide outputs which feed a clock distri- bution unit (CDU). The clock outputs CLKO0–CLKO9 are connected to various targets. For more information, refer to the ADSP-SC58x/ADSP-2158x SHARC+ Processor Hardware Reference.

#### Power-Up

SYS_XTALx oscillations (SYS_CLKINx) start when power is applied to the VDD_EXT pins. The rising edge of SYS_HWRST starts on-chip PLL locking (PLL lock counter). The deassertion must apply only if all voltage supplies and SYS_CLKINx oscilla- tions are valid (refer to the Power-Up Reset Timing section).

#### Clock Out/External Clock

The SYS_CLKOUT output pin has programmable options to output divided-down versions of the on-chip clocks. By default, the SYS_CLKOUT pin drives a buffered version of the

SYS_ CLKIN0 input. Refer to the ADSP-SC58x/ADSP-2158x SHARC+ Processor Hardware Reference to change the default mapping of clocks.

#### Booting

The processors have several mechanisms for automatically load- ing internal and external memory after a reset. The boot mode is defined by the SYS_BMODE[n] input pins. There are two

categories of boot modes. In master boot mode, the processors actively load data from serial memories. In slave boot modes, the processors receive data from external host devices.

The boot modes are shown in Table 9. These modes are imple- mented by the SYS_BMODE[n] bits of the reset configuration register and are sampled during power-on resets and software initiated resets.

In the ADSP-SC58x processors, the Arm Cortex-A5 (Core 0) controls the boot process, including loading all internal and external memory. Likewise, in the ADSP-2158x processors, the SHARC+ (Core 1) controls the boot function. The option for secure boot is available on all models.

##### Table 9. Boot Modes

| SYS_BMODE[n] Setting | Boot Mode |
| --- | --- |
| 000 | No boot |
| 001 | SPI2 master |
| 010 | SPI2 slave |
| 011 | Reserved |
| 100 | Reserved |
| 101 | Reserved |
| 110 | Link0 slave |
| 111 | UART0 slave |

Thermal Monitoring Unit (TMU)

The thermal monitoring unit (TMU) provides on-chip tem- perature measurement which is important in applications that require substantial power consumption. The TMU is integrated into the processor die and digital infrastructure using an MMR- based system access to measure the die temperature variations in real-time.

TMU features include the following:

- On-chip temperature sensing

- Programmable over temperature and under temperature limits

- Programmable conversion rate

- Averaging feature available

#### Power Supplies

The processors have separate power supply connections for:

- Internal (VDD_INT)

- External (VDD_EXT)

- USB (VDD_USB)

- HADC/TMU (VDD_HADC)

- RTC (VDD_RTC)

- DMC (VDD_DMC)

- PCIe (VDD_PCIE, VDD_PCIE_TX and VDD_PCIE_RX)

All power supplies must meet the specifications provided in the Operating Conditions section. All external supply pins must be connected to the same power supply.

#### Power Management

As shown in Table 10, the processors support four different power domains, which maximizes flexibility while maintaining compliance with industry standards and conventions. There are no sequencing requirements for the various power domains, but all domains must be powered according to the appropriate spec- ifications (see the Specifications section for processor operating conditions). If the feature or the peripheral is not used, refer to Table 27.)

##### Table 10. Power Domains

| Power Domain | VDD Range |
| --- | --- |
| All internal logic | VDD_INT |
| DDR3/DDR2/LPDDR | VDD_DMC |
| USB | VDD_USB |
| HADC/TMU | VDD_HADC |
| RTC | VDD_RTC |
| PCIe_TX | VDD_PCIE_TX |
| PCIe_RX | VDD_PCIE_RX |
| PCIe | VDD_PCIE |
| All other I/O (includes SYS, JTAG, and port pins) | VDD_EXT |

The power dissipated by the processors is largely a function of the clock frequency and the square of the operating voltage. For example, reducing the clock frequency by 25% results in a 25% reduction in dynamic power dissipation.

#### Target Board JTAG Emulator Connector

The Analog Devices DSP tools product line of JTAG emulators uses the IEEE 1149.1 JTAG test access port of the processors to monitor and control the target board processor during emula- tion. The Analog Devices DSP tools product line of JTAG emulators provides emulation at full processor speed, allowing inspection and modification of memory, registers, and proces- sor stacks. The processor JTAG interface ensures the emulator does not affect target system loading or timing.

For information on JTAG emulator operation, see the appropri- ate emulator hardware user’s guide at SHARC Processors Software and Tools.

## SYSTEM DEBUG

The processors include various features that allow easy system debug. These are described in the following sections.

#### System Watchpoint Unit (SWU)

The system watchpoint unit (SWU) is a single module that con- nects to a single system bus and provides transaction monitoring. One SWU is attached to the bus going to each system slave. The SWU provides ports for all system bus address channel signals. Each SWU contains four match groups of registers with associated hardware. These four SWU match groups operate independently but share common event (for example, interrupt and trigger) outputs.

#### Debug Access Port (DAP)

Debug access port (DAP) provides IEEE 1149.1 JTAG interface support through the JTAG debug. The DAP provides an optional instrumentation trace for both the core and system. It provides a trace stream that conforms to MIPI System Trace Protocol version 2 (STPv2).

## DEVELOPMENT TOOLS

Analog Devices supports its processors with a complete line of software and hardware development tools, including an inte- grated development environment (CrossCore® Embedded Studio), evaluation products, emulators, and a variety of soft- ware add-ins.

#### Integrated Development Environments (IDEs)

For C/C++ software writing and editing, code generation, and debug support, Analog Devices offers the CrossCore Embedded Studio integrated development environment (IDE).

CrossCore Embedded Studio is based on the Eclipse framework. Supporting most Analog Devices processor families, it is the IDE of choice for processors, including multicore devices.

CrossCore Embedded Studio seamlessly integrates available software add-ins to support real-time operating systems, file systems, TCP/IP stacks, USB stacks, algorithmic software mod- ules, and evaluation hardware board support packages. For more information, visit www.analog.com/cces.

#### EZ-KIT Lite Evaluation Board

For processor evaluation, Analog Devices provides a wide range of EZ-KIT Lite® evaluation boards. Including the processor and key peripherals, the evaluation board also supports on-chip emulation capabilities and other evaluation and development features. Various EZ-Extenders® are also available, which are daughter cards that deliver additional specialized functionality, including audio and video processing. For more information visit www.analog.com.

#### EZ-KIT Lite Evaluation Kits

For a cost-effective way to learn more about developing with Analog Devices processors, Analog Devices offer a range of EZ- KIT Lite evaluation kits. Each evaluation kit includes an EZ-KIT Lite evaluation board, directions for downloading an evaluation version of the available IDE(s), a USB cable, and a power supply. The USB controller on the EZ-KIT Lite board connects to the USB port of the user PC, enabling the chosen IDE evaluation suite to emulate the on-board processor in-circuit.

This permits users to download, execute, and debug programs for the EZ-KIT Lite system. It also supports in circuit program- ming of the on-board Flash device to store user specific boot code, enabling standalone operation. With the full version of CrossCore Embedded Studio installed (sold separately), engi- neers can develop software for supported EZ-KITs or any custom system utilizing supported Analog Devices processors.

#### Software Add-Ins for CrossCore Embedded Studio

Analog Devices offers software add-ins which seamlessly inte- grate with CrossCore Embedded Studio to extend the capabilities and reduce development time. Add-ins include board support packages for evaluation hardware, various mid- dleware packages, and algorithmic modules. Documentation, help, configuration dialogs, and coding examples present in these add-ins are viewable through the CrossCore Embedded Studio IDE once the add-in is installed.

#### Board Support Packages for Evaluation Hardware

Software support for the EZ-KIT Lite evaluation boards and EZ- Extender daughter cards is provided by software add-ins called board support packages (BSPs). The BSPs contain the required drivers, pertinent release notes, and select example code for the given evaluation hardware. A download link for a specific BSP is located on the web page for the associated EZ-KIT or EZ- Extender product.

#### Middleware Packages

Analog Devices offers middleware add-ins such as real-time operating systems, file systems, USB stacks, and TCP/IP stacks. For more information, see the following web pages:

- www.analog.com/ucos2

- www.analog.com/ucos3

- www.analog.com/ucfs

- www.analog.com/ucusbd

- www.analog.com/ucusbh

- www.analog.com/lwip

#### Algorithmic Modules

To speed development, Analog Devices offers add-ins that per- form popular audio and video processing algorithms. These are available for use with CrossCore Embedded Studio. For more information visit www.analog.com.

#### Designing an Emulator-Compatible DSP Board (Target)

For embedded system test and debug, Analog Devices provides a family of emulators. On each JTAG DSP, Analog Devices sup- plies an IEEE 1149.1 JTAG test access port (TAP). In-circuit emulation is facilitated by use of this JTAG interface. The emulator accesses the internal features of the processor via the TAP, allowing the developer to load code, set breakpoints, and view variables, memory, and registers.

The processor must be halted to send data and commands, but once an operation is completed by the emulator, the DSP system is set to run at full speed with no impact on system timing. The emulators require the target board to include a header that sup- ports connection of the JTAG port of the DSP to the emulator.

For details on target board design issues including mechanical layout, single processor connections, signal buffering, signal ter- mination, and emulator pod logic, see Analog Devices JTAG Emulation Technical Reference (EE-68).

## ADDITIONAL INFORMATION

This data sheet provides a general overview of the ADSP- SC58x/ADSP-2158x architecture and functionality. For detailed information on the core architecture and instruction set, refer to the SHARC+ Core Programming Reference.

## RELATED SIGNAL CHAINS

A signal chain is a series of signal-conditioning electronic com- ponents that receive input (data acquired from sampling either real-time phenomena or from stored data) in tandem, with the output of one portion of the chain supplying input to the next. Signal chains are often used in signal processing applications to gather and process data or to apply system controls based on analysis of real-time phenomena.

Analog Devices eases signal processing system development by providing signal processing components that are designed to work together well. A tool for viewing relationships between specific applications and related components is available on the www.analog.com website.

The application signal chains page in the Circuits from the Lab® site (www.analog.com\circuits) provides the following:

  - Graphical circuit block diagram presentation of signal chains for a variety of circuit types and applications

  - Drill down links for components in each chain to selection guides and application information

  - Reference designs applying best practice design techniques

# ADSP-SC58x/ADSP-2158x DETAILED SIGNAL DESCRIPTIONS

Table 11 provides a detailed description of each pin.

##### Table 11. ADSP-SC58x/ADSP-2158x Detailed Signal Descriptions

| Signal Name | Direction | Description |
| --- | --- | --- |
| ACM_A[n] | Output | ADC Control Signals. Function varies by mode. |
| ACM_T[n] | Input | External Trigger n. Input for external trigger events. |
| C1_FLG[n] | InOut | SHARC+ Core 1 Flag Pin. |
| C2_FLG[n] | InOut | SHARC+ Core 2 Flag Pin. |
| CAN_RX | Input | Receive. Typically an external CAN transceiver RX output. |
| CAN_TX | Output | Transmit. Typically an external CAN transceiver TX input. |
| CNT_DG | Input | Count Down and Gate. Depending on the mode of operation, this input acts either as a count down signal or a gate signal. |
|  |  | Count down—this input causes the GP counter to decrement. |
|  |  | Gate—stops the GP counter from incrementing or decrementing. |
| CNT_UD | Input | Count Up and Direction. Depending on the mode of operation, this input acts either as a count up signal or a direction signal. Count up—this input causes the GP counter to increment. Direction—selects whether the GP counter is incrementing or decrementing. |
| CNT_ZM | Input | Count Zero Marker. Input that connects to the zero marker output of a rotary device or detects the pressing of a pushbutton. |
| DAI_PIN[nn] | InOut | Pin n. The digital applications interfaces (DAI0 and DAI1) connect various peripherals to any of the DAI0_PINnn and DAI1_PINnn pins. Programs make these connections using the signal routing unit (SRU). Both DAI units are symmetric. The shared DAIx PIN03 and DAIx_PIN04 pins allow routing between both DAI units. |
| DMC_A[nn] | Output | Address n. Address bus. |
| DMC_BA[n] | Output | Bank Address n. Defines which internal bank an activate, read, write, or precharge command is applied to on the dynamic memory. Bank Address n also defines which mode registers (MR, EMR, EMR2, and/or EMR3) load during the load mode register command. |
| DMC_CAS | Output | Column Address Strobe. Defines the operation for external dynamic memory to perform in conjunction with other DMC command signals. Connect to the CAS input of dynamic memory. |
| DMC_CK | Output | Clock. Outputs DCLK to external dynamic memory. |
| DMC_CKE | Output | Clock Enable. Active high clock enables. Connects to the dynamic memory’s CKE input. |
| DMC_CK | Output | Clock (Complement). Complement of DMC_CK. |
| DMC_CS[n] | Output | Chip Select n. Commands are recognized by the memory only when this signal is asserted. |
| DMC_DQ[nn] | InOut | Data n. Bidirectional data bus. |
| DMC_LDM | Output | Data Mask for Lower Byte. Mask for DMC_DQ07:DMC_DQ00 write data when driven high. Sampled on both edges of the data strobe by the dynamic memory. |
| DMC_LDQS | InOut | Data Strobe for Lower Byte. DMC_DQ07:DMC_DQ00 data strobe. Output with write data. Input with read data. Can be single-ended or differential depending on register settings. |
| DMC_LDQS | InOut | Data Strobe for Lower Byte (Complement). Complement of LDQS. Not used in single-ended mode. |
| DMC_ODT | Output | On-Die Termination. Enables dynamic memory termination resistances when driven high (assuming the memory is properly configured). |
| DMC_RAS | Output | Row Address Strobe. Defines the operation for external dynamic memory to perform in conjunction with other DMC command signals. Connect to the RAS input of dynamic memory. |
| DMC_RESET | Output | Reset (DDR3 Only). |
| DMC_RZQ | InOut | External Calibration Resistor Connection. |
| DMC_UDM | Output | Data Mask for Upper Byte. Mask for DMC_DQ15:DMC_DQ08 write data when driven high. Sampled on both edges of the data strobe by the dynamic memory. |
| DMC_UDQS | InOut | Data Strobe for Upper Byte. DMC_DQ15:DMC_DQ08 data strobe. Output with write data. Input with read data. Not used in single-ended mode. |

| Signal Name | Direction | Description |
| --- | --- | --- |
| DMC_UDQS | InOut | Data Strobe for Upper Byte (Complement). Complement of UDQS. Not used in single-ended mode. |
| DMC_VREF | Input | Voltage Reference. Externally driven to VDD_DMC/2. Applies to DMC0_VREF and DMC1_VREF pins. |
| DMC_WE | Output | Write Enable. Defines the operation for external dynamic memory to perform in conjunction with other DMC command signals. Connect to the WE input of dynamic memory. |
| ETH_CRS | Input | Carrier Sense/RMII Receive Data Valid. Multiplexed on alternate clock cycles. CRS— asserted by the PHY when either the transmit or receive medium is not idle. Deasserted when both are idle. RXDV—asserted by the PHY when the data on RXDn is valid. |
| ETH_MDC | Output | Management Channel Clock. Clocks the MDC input of the PHY for RMII/RGMII. |
| ETH_MDIO | InOut | Management Channel Serial Data. Bidirectional data bus for PHY control for RMII/RGMII. |
| ETH_PTPAUXIN[n] | Input | PTP Auxiliary Trigger Input. Assert this signal to take an auxiliary snapshot of the time and store it in the auxiliary time stamp FIFO. |
| ETH_PTPCLKIN[n] | Input | PTP Clock Input. Optional external PTP clock input. |
| ETH_PTPPPS[n] | Output | PTP Pulse Per Second Output. When the advanced time stamp feature enables, this signal is asserted based on the PPS mode selected. Otherwise, PTPPPS is asserted every time the seconds counter is incremented. |
| ETH_REFCLK | Input | Reference Clock. Externally supplied Ethernet clock. |
| ETH_RXCLK_REFCLK | Input | RXCLK (10/100/1000) or REFCLK (10/100). |
| ETH_RXCTL_CRS | Input | RXCTL (10/100/1000) or CRS (10/100). |
| ETH_RXD[n] | Input | Receive Data n. Receive data bus. |
| ETH_TXCLK | Output | Transmit Clock. |
| ETH_TXCTL_TXEN | Output | TXCTL (10/100/1000) or TXEN (10/100). |
| ETH_TXD[n] | Output | Transmit Data n. Transmits data bus. |
| ETH_TXEN | Output | Transmit Enable. When asserted, signal indicates the data on TXDn is valid. |
| HADC_EOC_DOUT | Output | End of Conversion/Serial Data Out. Transitions high for one cycle of the HADC internal clock at the end of every conversion. Alternatively, HADC serial data out can be seen by setting the appropriate bit in HADC_CTL. |
| HADC_MUX[n] | Input | Controls to External Multiplexer. Allows additional input channels when connected to an external multiplexer. |
| HADC_VIN[n] | Input | Analog Input at Channel n. Analog voltage inputs for digital conversion. |
| HADC_VREFN | Input | Ground Reference for ADC. Connect to an external voltage reference that meets data sheet specifications. |
| HADC_VREFP | Input | External Reference for ADC. Connect to an external voltage reference that meets data sheet specifications. |
| JTG_TCK | Input | JTAG Clock. JTAG test access port clock. |
| JTG_TDI | Input | JTAG Serial Data In. JTAG test access port data input. |
| JTG_TDO | Output | JTAG Serial Data Out. JTAG test access port data output. |
| JTG_TMS | Input | JTAG Mode Select. JTAG test access port mode select. |
| JTG_TRST | Input | JTAG Reset. JTAG test access port reset. |
| LP_ACK | InOut | Acknowledge. Provides handshaking. When the link port is configured as a receiver, ACK is an output. When the link port is configured as a transmitter, ACK is an input. |
| LP_CLK | InOut | Clock. When the link port is configured as a receiver, CLK is an input. When the link port is configured as a transmitter, CLK is an output. |
| LP_D[n] | InOut | Data n. Data bus. Input when receiving, output when transmitting. |
| MLB_CLKN | Input | Differential Clock (–). |
| MLB_CLKP | Input | Differential Clock (+). |
| MLB_DATN | InOut | Differential Data (–). |
| MLB_DATP | InOut | Differential Data (+). |
| MLB_SIGN | InOut | Differential Signal (–). |

| Signal Name | Direction | Description |
| --- | --- | --- |
| MLB_SIGP | InOut | Differential Signal (+). |
| MLB_CLK | Input | Single-Ended Clock. |
| MLB_DAT | InOut | Single-Ended Data. |
| MLB_SIG | InOut | Single-Ended Signal. |
| MLB_CLKOUT | Output | Single-Ended Clock Out. |
| MSI_CD | Input | Card Detect. Connects to a pull-up resistor and to the card detect output of an SD socket. |
| MSI_CLK | Output | Clock. The clock signal applied to the connected device from the MSI. |
| MSI_CMD | InOut | Command. Sends commands to and receives responses from the connected device. |
| MSI_D[n] | InOut | Data n. Bidirectional data bus. |
| MSI_INT | Input | eSDIO Interrupt Input. Used only for eSDIO. Connects to an eSDIO card interrupt output. An interrupt may be sampled even when the MSI clock to the card is switched off. |
| PCIE_CLKM | Input | CLK –. |
| PCIE_CLKP | Input | CLK +. |
| PCIE_REF | InOut | Reference Resistor. Attach a 200 Ω, 1%, 100 ppm/C precision resistor to ground on the board. |
| PCIE_RXM | Input | RX –. |
| PCIE_RXP | Input | RX +. |
| PCIE_TXM | Output | TX –. |
| PCIE_TXP | Output | TX +. |
| PPI_CLK | InOut | Clock. Input in external clock mode, output in internal clock mode. |
| PPI_D[nn] | InOut | Data n. Bidirectional data bus. |
| PPI_FS1 | InOut | Frame Sync 1 (HSYNC). Behavior depends on EPPI mode. See the “EPPI” chapter of the ADSP- SC58x/ADSP-2158x SHARC+ Processor Hardware Reference for more details. |
| PPI_FS2 | InOut | Frame Sync 2 (VSYNC). Behavior depends on EPPI mode. See the “EPPI” chapter of the ADSP- SC58x/ADSP-2158x SHARC+ Processor Hardware Reference for more details. |
| PPI_FS3 | InOut | Frame Sync 3 (FIELD). Behavior depends on EPPI mode. See the “EPPI” chapter of the ADSP- SC58x/ADSP-2158x SHARC+ Processor Hardware Reference for more details. |
| PWM_AH | Output | Channel A High Side. High-side drive signal. |
| PWM_AL | Output | Channel A Low Side. Low-side drive signal. |
| PWM_BH | Output | Channel B High Side. High-side drive signal. |
| PWM_BL | Output | Channel B Low Side. Low-side drive signal. |
| PWM_CH | Output | Channel C High Side. High-side drive signal. |
| PWM_CL | Output | Channel C Low Side. Low-side drive signal. |
| PWM_DH | Output | Channel D High Side. High-side drive signal. |
| PWM_DL | Output | Channel D Low Side. Low-side drive signal. |
| PWM_SYNC | Input | PWMTMR Grouped. This input is for an externally generated sync signal. If the sync signal is internally generated, no connection is necessary. |
| PWM_TRIP[n] | Input | Shutdown Input n. When asserted, the selected PWM channel outputs are shut down immediately. |
| P_[nn] | InOut | Position n. General-purpose input/output. See the “GP Ports” chapter of the ADSP-SC58x/ADSP- 2158x SHARC+ Processor Hardware Reference for more details. |
| RTC_CLKIN | Input | Crystal Input/External Oscillator Connection. Connect to an external clock source or crystal. |
| RTC_XTAL | Output | Crystal Output. Drives an external crystal. Must be left unconnected if an external clock is driving RTC_CLKIN. |
| SINC_CLK0 | Output | Clock 0. |
| SINC_D0 | Input | Data 0. |
| SINC_D1 | Input | Data 1. |
| SINC_D2 | Input | Data 2. |
| SINC_D3 | Input | Data 3. |

| Signal Name | Direction | Description |
| --- | --- | --- |
| SMC_ABE[n] | Output | Byte Enable n. Indicates whether the lower or upper byte of a memory is being accessed. When an asynchronous write is made to the upper byte of a 16-bit memory, SMC_ABE1 = 0 and SMC_ABE0 = 1. When an asynchronous write is made to the lower byte of a 16-bit memory, SMC_ABE1 = 1 and SMC_ABE0 = 0. |
| SMC_AMS[n] | Output | Memory Select n. Typically connects to the chip select of a memory device. |
| SMC_AOE | Output | Output Enable. Asserts at the beginning of the setup period of a read access. |
| SMC_ARDY | Input | Asynchronous Ready. Flow control signal used by memory devices to indicate to the SMC when further transactions may proceed. |
| SMC_ARE | Output | Read Enable. Asserts at the beginning of a read access. |
| SMC_AWE | Output | Write Enable. Asserts for the duration of a write access period. |
| SMC_A[nn] | Output | Address n. Address bus. |
| SMC_D[nn] | InOut | Data n. Bidirectional data bus. |
| SPI_CLK | InOut | Clock. Input in slave mode, output in master mode. |
| SPI_D2 | InOut | Data 2. Transfers serial data in quad mode. Open-drain when ODM mode is enabled. |
| SPI_D3 | InOut | Data 3. Transfers serial data in quad mode. Open-drain when ODM mode is enabled. |
| SPI_MISO | InOut | Master In, Slave Out. Transfers serial data. Operates in the same direction as SPI_MOSI in dual and quad modes. Open-drain when ODM mode is enabled. |
| SPI_MOSI | InOut | Master Out, Slave In. Transfers serial data. Operates in the same direction as SPI_MISO in dual and quad modes. Open-drain when ODM mode is enabled. |
| SPI_RDY | InOut | Ready. Optional flow signal. Output in slave mode, input in master mode. |
| SPI_SEL[n] | Output | Slave Select Output n. Used in master mode to enable the desired slave. |
| SPI_SS | Input | Slave Select Input. Slave mode—acts as the slave select input. Master mode—optionally serves as an error detection input for the SPI when there are multiple masters. |
| SPT_ACLK | InOut | Channel A Clock. Data and frame sync are driven/sampled with respect to this clock. This signal can be either internally or externally generated. |
| SPT_AD0 | InOut | Channel A Data 0. Primary bidirectional data I/O. This signal can be configured as an output to transmit serial data or as an input to receive serial data. |
| SPT_AD1 | InOut | Channel A Data 1. Secondary bidirectional data I/O. This signal can be configured as an output to transmit serial data or as an input to receive serial data. |
| SPT_AFS | InOut | Channel A Frame Sync. The frame sync pulse initiates shifting of the serial data. This signal is either generated internally or externally. |
| SPT_ATDV | Output | Channel A Transmit Data Valid. This signal is optional and only active when SPORT is configured in multichannel transmit mode. It is asserted during enabled slots. |
| SPT_BCLK | InOut | Channel B Clock. Data and frame sync are driven/sampled with respect to this clock. This signal can be either internally or externally generated. |
| SPT_BD0 | InOut | Channel B Data 0. Primary bidirectional data I/O. This signal can be configured as an output to transmit serial data or as an input to receive serial data. |
| SPT_BD1 | InOut | Channel B Data 1. Secondary bidirectional data I/O. This signal can be configured as an output to transmit serial data or as an input to receive serial data. |
| SPT_BFS | InOut | Channel B Frame Sync. The frame sync pulse initiates shifting of serial data. This signal is either generated internally or externally. |
| SPT_BTDV | Output | Channel B Transmit Data Valid. This signal is optional and only active when SPORT is configured in multichannel transmit mode. It is asserted during enabled slots. |
| SYS_BMODE[n] | Input | Boot Mode Control n. Selects the boot mode of the processor. |
| SYS_CLKIN0 | Input | Clock/Crystal Input. |
| SYS_CLKIN1 | Input | Clock/Crystal Input. |
| SYS_CLKOUT | Output | Processor Clock Output. Outputs internal clocks. Clocks may be divided down. See the “CGU” chapter of the ADSP-SC58x/ADSP-2158x SHARC+ Processor Hardware Reference for more details. |

| Signal Name | Direction | Description |
| --- | --- | --- |
| SYS_FAULT | InOut | Active High Fault Output. Indicates internal faults or senses external faults, depending on the operating mode. |
| SYS_FAULT | InOut | Active Low Fault Output. Indicates internal faults or senses external faults, depending on the operating mode. |
| SYS_HWRST | Input | Processor Hardware Reset Control. Resets the device when asserted. |
| SYS_RESOUT | Output | Reset Output. Indicates the device is in the reset state. |
| SYS_XTAL0 | Output | Crystal Output. |
| SYS_XTAL1 | Output | Crystal Output. |
| TM_ACI[n] | Input | Alternate Capture Input n. Provides an additional input for WIDCAP, WATCHDOG, and PININT modes. |
| TM_ACLK[n] | Input | Alternate Clock n. Provides an additional time base for an individual timer. |
| TM_CLK | Input | Clock. Provides an additional global time base for all GP timers. |
| TM_TMR[n] | InOut | Timer n. The main input/output signal for each timer. |
| TRACE_CLK | Output | Trace Clock. Clock output. |
| TRACE_D[nn] | Output | Trace Data n. Unidirectional data bus. |
| TWI_SCL | InOut | Serial Clock. Clock output when master, clock input when slave. |
| TWI_SDA | InOut | Serial Data. Receives or transmits data. |
| UART_CTS | Input | Clear to Send. Flow control signal. |
| UART_RTS | Output | Request to Send. Flow control signal. |
| UART_RX | Input | Receive. Receives input. Typically connects to a transceiver that meets the electrical requirements of the device being communicated with. |
| UART_TX | Output | Transmit. Transmits output. Typically connects to a transceiver that meets the electrical requirements of the device being communicated with. |
| USB_CLKIN | Input | Clock/Crystal Input. This clock input is multiplied by a PLL to form the USB clock. See data sheet specifications for frequency/tolerance information. |
| USB_DM | InOut | Data –. Bidirectional differential data line. |
| USB_DP | InOut | Data +. Bidirectional differential data line. |
| USB_ID | Input | OTG ID. Senses whether the controller is a host or device. This signal is pulled low when an A-type plug is sensed (signifying that the USB controller is the A device).The input is high when a B-type plug is sensed (signifying that the USB controller is the B device). |
| USB_VBC | Output | VBUS Control. Controls an external voltage source to supply VBUS when in host mode. Can be configured as open drain. Polarity is configurable as well. |
| USB_VBUS | InOut | Bus Voltage. Connects to bus voltage in host and device modes. |
| USB_XTAL | Output | Crystal. Drives an external crystal. Must be left unconnected if an external clock is driving USB_CLKIN. |

349-BALL CSP_BGA SIGNAL DESCRIPTIONS

The processor pin definitions are shown in Table 12 for the 349-ball CSP_BGA package. The columns in this table provide the following information:

- The Signal Name column includes the signal name for every pin and the GPIO multiplexed pin function, where applicable.

- The Description column provides a descriptive name for each signal.

- The Port column shows whether or not a signal is multi- plexed with other signals on a general-purpose I/O port pin.

- The Pin Name column identifies the name of the package pin (at power on reset) on which the signal is located (if a single function pin) or is multiplexed (if a general-purpose I/O pin).

- The DAI pins and their associated signal routing units (SRUs) connect inputs and outputs of the DAI peripherals (SPORT, ASRC, S/PDIF, and PCG). See the “Digital Audio Interface (DAI)” chapter of the ADSP-SC58x/ADSP-2158x SHARC+ Processor Hardware Reference for complete information on the use of the DAI and SRUs.

##### Table 12. ADSP-SC58x/ADSP-2158x 349-Ball CSP_BGA Signal Descriptions

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| ACM0_A0 | ACM0 ADC Control Signals | C | PC_13 |
| ACM0_A1 | ACM0 ADC Control Signals | C | PC_14 |
| ACM0_A2 | ACM0 ADC Control Signals | C | PC_15 |
| ACM0_A3 | ACM0 ADC Control Signals | D | PD_00 |
| ACM0_A4 | ACM0 ADC Control Signals | D | PD_01 |
| ACM0_T0 | ACM0 External Trigger n | C | PC_12 |
| C1_FLG0 | SHARC Core 1 Flag Pin | E | PE_01 |
| C1_FLG1 | SHARC Core 1 Flag Pin | E | PE_03 |
| C1_FLG2 | SHARC Core 1 Flag Pin | E | PE_05 |
| C1_FLG3 | SHARC Core 1 Flag Pin | E | PE_07 |
| C2_FLG0 | SHARC Core 2 Flag Pin | E | PE_02 |
| C2_FLG1 | SHARC Core 2 Flag Pin | E | PE_04 |
| C2_FLG2 | SHARC Core 2 Flag Pin | E | PE_06 |
| C2_FLG3 | SHARC Core 2 Flag Pin | E | PE_08 |
| CAN0_RX | CAN0 Receive | C | PC_07 |
| CAN0_TX | CAN0 Transmit | C | PC_08 |
| CAN1_RX | CAN1 Receive | B | PB_10 |
| CAN1_TX | CAN1 Transmit | B | PB_09 |
| CNT0_DG | CNT0 Count Down and Gate | B | PB_14 |
| CNT0_UD | CNT0 Count Up and Direction | B | PB_12 |
| CNT0_ZM | CNT0 Count Zero Marker | B | PB_11 |
| DAI0_PIN01 | DAI0 Pin 1 | Not Muxed | DAI0_PIN01 |
| DAI0_PIN02 | DAI0 Pin 2 | Not Muxed | DAI0_PIN02 |
| DAI0_PIN03 | DAI0 Pin 3 | Not Muxed | DAI0_PIN03 |
| DAI0_PIN04 | DAI0 Pin 4 | Not Muxed | DAI0_PIN04 |
| DAI0_PIN05 | DAI0 Pin 5 | Not Muxed | DAI0_PIN05 |
| DAI0_PIN06 | DAI0 Pin 6 | Not Muxed | DAI0_PIN06 |
| DAI0_PIN07 | DAI0 Pin 7 | Not Muxed | DAI0_PIN07 |
| DAI0_PIN08 | DAI0 Pin 8 | Not Muxed | DAI0_PIN08 |
| DAI0_PIN09 | DAI0 Pin 9 | Not Muxed | DAI0_PIN09 |
| DAI0_PIN10 | DAI0 Pin 10 | Not Muxed | DAI0_PIN10 |
| DAI0_PIN11 | DAI0 Pin 11 | Not Muxed | DAI0_PIN11 |
| DAI0_PIN12 | DAI0 Pin 12 | Not Muxed | DAI0_PIN12 |
| DAI0_PIN19 | DAI0 Pin 19 | Not Muxed | DAI0_PIN19 |
| DAI0_PIN20 | DAI0 Pin 20 | Not Muxed | DAI0_PIN20 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| DAI1_PIN01 | DAI1 Pin 1 | Not Muxed | DAI1_PIN01 |
| DAI1_PIN02 | DAI1 Pin 2 | Not Muxed | DAI1_PIN02 |
| DAI1_PIN03 | DAI1 Pin 3 | Not Muxed | DAI1_PIN03 |
| DAI1_PIN04 | DAI1 Pin 4 | Not Muxed | DAI1_PIN04 |
| DAI1_PIN05 | DAI1 Pin 5 | Not Muxed | DAI1_PIN05 |
| DAI1_PIN06 | DAI1 Pin 6 | Not Muxed | DAI1_PIN06 |
| DAI1_PIN07 | DAI1 Pin 7 | Not Muxed | DAI1_PIN07 |
| DAI1_PIN08 | DAI1 Pin 8 | Not Muxed | DAI1_PIN08 |
| DAI1_PIN09 | DAI1 Pin 9 | Not Muxed | DAI1_PIN09 |
| DAI1_PIN10 | DAI1 Pin 10 | Not Muxed | DAI1_PIN10 |
| DAI1_PIN11 | DAI1 Pin 11 | Not Muxed | DAI1_PIN11 |
| DAI1_PIN12 | DAI1 Pin 12 | Not Muxed | DAI1_PIN12 |
| DAI1_PIN19 | DAI1 Pin 19 | Not Muxed | DAI1_PIN19 |
| DAI1_PIN20 | DAI1 Pin 20 | Not Muxed | DAI1_PIN20 |
| DMC0_A00 | DMC0 Address 0 | Not Muxed | DMC0_A00 |
| DMC0_A01 | DMC0 Address 1 | Not Muxed | DMC0_A01 |
| DMC0_A02 | DMC0 Address 2 | Not Muxed | DMC0_A02 |
| DMC0_A03 | DMC0 Address 3 | Not Muxed | DMC0_A03 |
| DMC0_A04 | DMC0 Address 4 | Not Muxed | DMC0_A04 |
| DMC0_A05 | DMC0 Address 5 | Not Muxed | DMC0_A05 |
| DMC0_A06 | DMC0 Address 6 | Not Muxed | DMC0_A06 |
| DMC0_A07 | DMC0 Address 7 | Not Muxed | DMC0_A07 |
| DMC0_A08 | DMC0 Address 8 | Not Muxed | DMC0_A08 |
| DMC0_A09 | DMC0 Address 9 | Not Muxed | DMC0_A09 |
| DMC0_A10 | DMC0 Address 10 | Not Muxed | DMC0_A10 |
| DMC0_A11 | DMC0 Address 11 | Not Muxed | DMC0_A11 |
| DMC0_A12 | DMC0 Address 12 | Not Muxed | DMC0_A12 |
| DMC0_A13 | DMC0 Address 13 | Not Muxed | DMC0_A13 |
| DMC0_A14 | DMC0 Address 14 | Not Muxed | DMC0_A14 |
| DMC0_A15 | DMC0 Address 15 | Not Muxed | DMC0_A15 |
| DMC0_BA0 | DMC0 Bank Address 0 | Not Muxed | DMC0_BA0 |
| DMC0_BA1 | DMC0 Bank Address 1 | Not Muxed | DMC0_BA1 |
| DMC0_BA2 | DMC0 Bank Address 2 | Not Muxed | DMC0_BA2 |
| DMC0_CAS | DMC0 Column Address Strobe | Not Muxed | DMC0_CAS |
| DMC0_CK | DMC0 Clock | Not Muxed | DMC0_CK |
| DMC0_CKE | DMC0 Clock Enable | Not Muxed | DMC0_CKE |
| DMC0_CK | DMC0 Clock (Complement) | Not Muxed | DMC0_CK |
| DMC0_CS0 | DMC0 Chip Select 0 | Not Muxed | DMC0_CS0 |
| DMC0_DQ00 | DMC0 Data 0 | Not Muxed | DMC0_DQ00 |
| DMC0_DQ01 | DMC0 Data 1 | Not Muxed | DMC0_DQ01 |
| DMC0_DQ02 | DMC0 Data 2 | Not Muxed | DMC0_DQ02 |
| DMC0_DQ03 | DMC0 Data 3 | Not Muxed | DMC0_DQ03 |
| DMC0_DQ04 | DMC0 Data 4 | Not Muxed | DMC0_DQ04 |
| DMC0_DQ05 | DMC0 Data 5 | Not Muxed | DMC0_DQ05 |
| DMC0_DQ06 | DMC0 Data 6 | Not Muxed | DMC0_DQ06 |
| DMC0_DQ07 | DMC0 Data 7 | Not Muxed | DMC0_DQ07 |
| DMC0_DQ08 | DMC0 Data 8 | Not Muxed | DMC0_DQ08 |
| DMC0_DQ09 | DMC0 Data 9 | Not Muxed | DMC0_DQ09 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| DMC0_DQ10 | DMC0 Data 10 | Not Muxed | DMC0_DQ10 |
| DMC0_DQ11 | DMC0 Data 11 | Not Muxed | DMC0_DQ11 |
| DMC0_DQ12 | DMC0 Data 12 | Not Muxed | DMC0_DQ12 |
| DMC0_DQ13 | DMC0 Data 13 | Not Muxed | DMC0_DQ13 |
| DMC0_DQ14 | DMC0 Data 14 | Not Muxed | DMC0_DQ14 |
| DMC0_DQ15 | DMC0 Data 15 | Not Muxed | DMC0_DQ15 |
| DMC0_LDM | DMC0 Data Mask for Lower Byte | Not Muxed | DMC0_LDM |
| DMC0_LDQS | DMC0 Data Strobe for Lower Byte | Not Muxed | DMC0_LDQS |
| DMC0_LDQS | DMC0 Data Strobe for Lower Byte (Complement) | Not Muxed | DMC0_LDQS |
| DMC0_ODT | DMC0 On-Die Termination | Not Muxed | DMC0_ODT |
| DMC0_RAS | DMC0 Row Address Strobe | Not Muxed | DMC0_RAS |
| DMC0_RESET | DMC0 Reset (DDR3 Only) | Not Muxed | DMC0_RESET |
| DMC0_RZQ | DMC0 External Calibration Resistor Connection | Not Muxed | DMC0_RZQ |
| DMC0_UDM | DMC0 Data Mask for Upper Byte | Not Muxed | DMC0_UDM |
| DMC0_UDQS | DMC0 Data Strobe for Upper Byte | Not Muxed | DMC0_UDQS |
| DMC0_UDQS | DMC0 Data Strobe for Upper Byte (Complement) | Not Muxed | DMC0_UDQS |
| DMC0_VREF | DMC0 Voltage Reference | Not Muxed | DMC0_VREF |
| DMC0_WE | DMC0 Write Enable | Not Muxed | DMC0_WE |
| ETH0_CRS | ETH0 Carrier Sense/RMII Receive Data Valid | A | PA_07 |
| ETH0_MDC | ETH0 Management Channel Clock | A | PA_02 |
| ETH0_MDIO | ETH0 Management Channel Serial Data | A | PA_03 |
| ETH0_PTPAUXIN0 | ETH0 PTP Auxiliary Trigger Input 0 | B | PB_03 |
| ETH0_PTPAUXIN1 | ETH0 PTP Auxiliary Trigger Input 1 | B | PB_04 |
| ETH0_PTPAUXIN2 | ETH0 PTP Auxiliary Trigger Input 2 | B | PB_05 |
| ETH0_PTPAUXIN3 | ETH0 PTP Auxiliary Trigger Input 3 | B | PB_06 |
| ETH0_PTPCLKIN0 | ETH0 PTP Clock Input 0 | B | PB_02 |
| ETH0_PTPPPS0 | ETH0 PTP Pulse Per Second Output 0 | B | PB_01 |
| ETH0_PTPPPS1 | ETH0 PTP Pulse Per Second Output 1 | B | PB_00 |
| ETH0_PTPPPS2 | ETH0 PTP Pulse Per Second Output 2 | A | PA_15 |
| ETH0_PTPPPS3 | ETH0 PTP Pulse Per Second Output 3 | A | PA_14 |
| ETH0_RXCLK_REFCLK | ETH0 RXCLK (10/100/1000) or REFCLK (10/100) | A | PA_06 |
| ETH0_RXCTL_CRS | ETH0 RXCTL (10/100/1000) or CRS (10/100) | A | PA_07 |
| ETH0_RXD0 | ETH0 Receive Data 0 | A | PA_04 |
| ETH0_RXD1 | ETH0 Receive Data 1 | A | PA_05 |
| ETH0_RXD2 | ETH0 Receive Data 2 | A | PA_08 |
| ETH0_RXD3 | ETH0 Receive Data 3 | A | PA_09 |
| ETH0_TXCLK | ETH0 Transmit Clock | A | PA_11 |
| ETH0_TXCTL_TXEN | ETH0 TXCTL (10/100/1000) or TXEN (10/100) | A | PA_10 |
| ETH0_TXD0 | ETH0 Transmit Data 0 | A | PA_00 |
| ETH0_TXD1 | ETH0 Transmit Data 1 | A | PA_01 |
| ETH0_TXD2 | ETH0 Transmit Data 2 | A | PA_12 |
| ETH0_TXD3 | ETH0 Transmit Data 3 | A | PA_13 |
| ETH0_TXEN | ETH0 Transmit Enable | A | PA_10 |
| HADC0_VIN0 | HADC0 Analog Input at Channel 0 | Not Muxed | HADC0_VIN0 |
| HADC0_VIN1 | HADC0 Analog Input at Channel 1 | Not Muxed | HADC0_VIN1 |
| HADC0_VIN2 | HADC0 Analog Input at Channel 2 | Not Muxed | HADC0_VIN2 |
| HADC0_VIN3 | HADC0 Analog Input at Channel 3 | Not Muxed | HADC0_VIN3 |
| HADC0_VIN4 | HADC0 Analog Input at Channel 4 | Not Muxed | HADC0_VIN4 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| HADC0_VIN5 | HADC0 Analog Input at Channel 5 | Not Muxed | HADC0_VIN5 |
| HADC0_VIN6 | HADC0 Analog Input at Channel 6 | Not Muxed | HADC0_VIN6 |
| HADC0_VIN7 | HADC0 Analog Input at Channel 7 | Not Muxed | HADC0_VIN7 |
| HADC0_VREFN | HADC0 Ground Reference for ADC | Not Muxed | HADC0_VREFN |
| HADC0_VREFP | HADC0 External Reference for ADC | Not Muxed | HADC0_VREFP |
| JTG_TCK | TAPC JTAG Clock | Not Muxed | JTG_TCK |
| JTG_TDI | TAPC JTAG Serial Data In | Not Muxed | JTG_TDI |
| JTG_TDO | TAPC JTAG Serial Data Out | Not Muxed | JTG_TDO |
| JTG_TMS | TAPC JTAG Mode Select | Not Muxed | JTG_TMS |
| JTG_TRST | TAPC JTAG Reset | Not Muxed | JTG_TRST |
| LP0_ACK | LP0 Acknowledge | D | PD_11 |
| LP0_CLK | LP0 Clock | D | PD_10 |
| LP0_D0 | LP0 Data 0 | D | PD_02 |
| LP0_D1 | LP0 Data 1 | D | PD_03 |
| LP0_D2 | LP0 Data 2 | D | PD_04 |
| LP0_D3 | LP0 Data 3 | D | PD_05 |
| LP0_D4 | LP0 Data 4 | D | PD_06 |
| LP0_D5 | LP0 Data 5 | D | PD_07 |
| LP0_D6 | LP0 Data 6 | D | PD_08 |
| LP0_D7 | LP0 Data 7 | D | PD_09 |
| LP1_ACK | LP1 Acknowledge | B | PB_15 |
| LP1_CLK | LP1 Clock | C | PC_00 |
| LP1_D0 | LP1 Data 0 | B | PB_07 |
| LP1_D1 | LP1 Data 1 | B | PB_08 |
| LP1_D2 | LP1 Data 2 | B | PB_09 |
| LP1_D3 | LP1 Data 3 | B | PB_10 |
| LP1_D4 | LP1 Data 4 | B | PB_11 |
| LP1_D5 | LP1 Data 5 | B | PB_12 |
| LP1_D6 | LP1 Data 6 | B | PB_13 |
| LP1_D7 | LP1 Data 7 | B | PB_14 |
| MLB0_CLKN | MLB0 Negative Differential Clock (–) | Not Muxed | MLB0_CLKN |
| MLB0_CLKP | MLB0 Positive Differential Clock (+) | Not Muxed | MLB0_CLKP |
| MLB0_DATN | MLB0 Negative Differential Data (–) | Not Muxed | MLB0_DATN |
| MLB0_DATP | MLB0 Positive Differential Data (+) | Not Muxed | MLB0_DATP |
| MLB0_SIGN | MLB0 Negative Differential Signal (–) | Not Muxed | MLB0_SIGN |
| MLB0_SIGP | MLB0 Positive Differential Signal (+) | Not Muxed | MLB0_SIGP |
| MLB0_CLK | MLB0 Single-Ended Clock | B | PB_04 |
| MLB0_DAT | MLB0 Single-Ended Data | B | PB_06 |
| MLB0_SIG | MLB0 Single-Ended Signal | B | PB_05 |
| MLB0_CLKOUT | MLB0 Single-Ended Clock Out | D | PD_14 |
| PA_00-15 | PORTA Position 00 Through Position 15 | A | PA_00-15 |
| PB_00-15 | PORTB Position 00 Through Position 15 | B | PB_00-15 |
| PC_00-15 | PORTC Position 00 Through Position 15 | C | PC_00-15 |
| PD_00-15 | PORTD Position 00 Through Position 15 | D | PD_00-15 |
| PE_00-15 | PORTE Position 00 Through Position 15 | E | PE_00-15 |
| PPI0_CLK | EPPI0 Clock | E | PE_03 |
| PPI0_D00 | EPPI0 Data 0 | E | PE_12 |
| PPI0_D01 | EPPI0 Data 1 | E | PE_11 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| PPI0_D02 | EPPI0 Data 2 | E | PE_10 |
| PPI0_D03 | EPPI0 Data 3 | E | PE_09 |
| PPI0_D04 | EPPI0 Data 4 | E | PE_08 |
| PPI0_D05 | EPPI0 Data 5 | E | PE_07 |
| PPI0_D06 | EPPI0 Data 6 | E | PE_06 |
| PPI0_D07 | EPPI0 Data 7 | E | PE_05 |
| PPI0_D08 | EPPI0 Data 8 | E | PE_04 |
| PPI0_D09 | EPPI0 Data 9 | E | PE_00 |
| PPI0_D10 | EPPI0 Data 10 | D | PD_15 |
| PPI0_D11 | EPPI0 Data 11 | D | PD_14 |
| PPI0_D12 | EPPI0 Data 12 | B | PB_04 |
| PPI0_D13 | EPPI0 Data 13 | B | PB_05 |
| PPI0_D14 | EPPI0 Data 14 | B | PB_00 |
| PPI0_D15 | EPPI0 Data 15 | B | PB_01 |
| PPI0_D16 | EPPI0 Data 16 | B | PB_02 |
| PPI0_D17 | EPPI0 Data 17 | B | PB_03 |
| PPI0_D18 | EPPI0 Data 18 | D | PD_13 |
| PPI0_D19 | EPPI0 Data 19 | D | PD_12 |
| PPI0_D20 | EPPI0 Data 20 | E | PE_13 |
| PPI0_D21 | EPPI0 Data 21 | E | PE_14 |
| PPI0_D22 | EPPI0 Data 22 | E | PE_15 |
| PPI0_D23 | EPPI0 Data 23 | D | PD_00 |
| PPI0_FS1 | EPPI0 Frame Sync 1 (HSYNC) | E | PE_02 |
| PPI0_FS2 | EPPI0 Frame Sync 2 (VSYNC) | E | PE_01 |
| PPI0_FS3 | EPPI0 Frame Sync 3 (FIELD) | C | PC_15 |
| PWM0_AH | PWM0 Channel A High Side | B | PB_07 |
| PWM0_AL | PWM0 Channel A Low Side | B | PB_08 |
| PWM0_BH | PWM0 Channel B High Side | B | PB_06 |
| PWM0_BL | PWM0 Channel B Low Side | C | PC_00 |
| PWM0_CH | PWM0 Channel C High Side | B | PB_13 |
| PWM0_CL | PWM0 Channel C Low Side | B | PB_14 |
| PWM0_DH | PWM0 Channel D High Side | B | PB_11 |
| PWM0_DL | PWM0 Channel D Low Side | B | PB_12 |
| PWM0_SYNC | PWM0 PWMTMR Grouped | E | PE_09 |
| PWM0_TRIP0 | PWM0 Shutdown Input 0 | B | PB_15 |
| PWM1_AH | PWM1 Channel A High Side | D | PD_03 |
| PWM1_AL | PWM1 Channel A Low Side | D | PD_04 |
| PWM1_BH | PWM1 Channel B High Side | D | PD_05 |
| PWM1_BL | PWM1 Channel B Low Side | D | PD_06 |
| PWM1_CH | PWM1 Channel C High Side | D | PD_07 |
| PWM1_CL | PWM1 Channel C Low Side | D | PD_08 |
| PWM1_DH | PWM1 Channel D High Side | D | PD_09 |
| PWM1_DL | PWM1 Channel D Low Side | D | PD_10 |
| PWM1_SYNC | PWM1 PWMTMR Grouped | D | PD_11 |
| PWM1_TRIP0 | PWM1 Shutdown Input 0 | D | PD_02 |
| PWM2_CH | PWM2 Channel C High Side | D | PD_15 |
| PWM2_CL | PWM2 Channel C Low Side | E | PE_00 |
| PWM2_DH | PWM2 Channel D High Side | E | PE_04 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| PWM2_DL | PWM2 Channel D Low Side | E | PE_10 |
| PWM2_SYNC | PWM2 PWMTMR Grouped | E | PE_05 |
| PWM2_TRIP0 | PWM2 Shutdown Input 0 | D | PD_14 |
| GND | Ground | Not Muxed | GND |
| VDD_EXT | External Voltage Domain | Not Muxed | VDD_EXT |
| VDD_INT | Internal Voltage Domain | Not Muxed | VDD_INT |
| SINC0_CLK0 | SINC0 Clock 0 | B | PB_01 |
| SINC0_D0 | SINC0 Data 0 | A | PA_14 |
| SINC0_D1 | SINC0 Data 1 | A | PA_15 |
| SINC0_D2 | SINC0 Data 2 | B | PB_00 |
| SINC0_D3 | SINC0 Data 3 | B | PB_04 |
| SMC0_A01 | SMC0 Address 1 | B | PB_05 |
| SMC0_A02 | SMC0 Address 2 | B | PB_06 |
| SMC0_A03 | SMC0 Address 3 | B | PB_03 |
| SMC0_A04 | SMC0 Address 4 | B | PB_02 |
| SMC0_A05 | SMC0 Address 5 | D | PD_13 |
| SMC0_A06 | SMC0 Address 6 | D | PD_12 |
| SMC0_A07 | SMC0 Address 7 | B | PB_01 |
| SMC0_A08 | SMC0 Address 8 | B | PB_00 |
| SMC0_A09 | SMC0 Address 9 | A | PA_15 |
| SMC0_A10 | SMC0 Address 10 | A | PA_14 |
| SMC0_A11 | SMC0 Address 11 | A | PA_09 |
| SMC0_A12 | SMC0 Address 12 | A | PA_08 |
| SMC0_A13 | SMC0 Address 13 | A | PA_13 |
| SMC0_A14 | SMC0 Address 14 | A | PA_12 |
| SMC0_A15 | SMC0 Address 15 | A | PA_11 |
| SMC0_A16 | SMC0 Address 16 | A | PA_07 |
| SMC0_A17 | SMC0 Address 17 | A | PA_06 |
| SMC0_A18 | SMC0 Address 18 | A | PA_05 |
| SMC0_A19 | SMC0 Address 19 | A | PA_04 |
| SMC0_A20 | SMC0 Address 20 | A | PA_01 |
| SMC0_A21 | SMC0 Address 21 | A | PA_00 |
| SMC0_A22 | SMC0 Address 22 | A | PA_10 |
| SMC0_A23 | SMC0 Address 23 | A | PA_03 |
| SMC0_A24 | SMC0 Address 24 | A | PA_02 |
| SMC0_A25 | SMC0 Address 25 | C | PC_12 |
| SMC0_ABE0 | SMC0 Byte Enable 0 | E | PE_14 |
| SMC0_ABE1 | SMC0 Byte Enable 1 | E | PE_15 |
| SMC0_AMS0 | SMC0 Memory Select 0 | C | PC_15 |
| SMC0_AMS1 | SMC0 Memory Select 1 | E | PE_13 |
| SMC0_AMS2 | SMC0 Memory Select 2 | C | PC_07 |
| SMC0_AMS3 | SMC0 Memory Select 3 | C | PC_08 |
| SMC0_AOE | SMC0 Output Enable | D | PD_01 |
| SMC0_ARDY | SMC0 Asynchronous Ready | B | PB_04 |
| SMC0_ARE | SMC0 Read Enable | C | PC_00 |
| SMC0_AWE | SMC0 Write Enable | B | PB_15 |
| SMC0_D00 | SMC0 Data 0 | E | PE_12 |
| SMC0_D01 | SMC0 Data 1 | E | PE_11 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| SMC0_D02 | SMC0 Data 2 | E | PE_10 |
| SMC0_D03 | SMC0 Data 3 | E | PE_09 |
| SMC0_D04 | SMC0 Data 4 | E | PE_00 |
| SMC0_D05 | SMC0 Data 5 | D | PD_15 |
| SMC0_D06 | SMC0 Data 6 | D | PD_14 |
| SMC0_D07 | SMC0 Data 7 | D | PD_00 |
| SMC0_D08 | SMC0 Data 8 | B | PB_14 |
| SMC0_D09 | SMC0 Data 9 | B | PB_13 |
| SMC0_D10 | SMC0 Data 10 | B | PB_12 |
| SMC0_D11 | SMC0 Data 11 | B | PB_11 |
| SMC0_D12 | SMC0 Data 12 | B | PB_10 |
| SMC0_D13 | SMC0 Data 13 | B | PB_09 |
| SMC0_D14 | SMC0 Data 14 | B | PB_08 |
| SMC0_D15 | SMC0 Data 15 | B | PB_07 |
| SPI0_CLK | SPI0 Clock | C | PC_09 |
| SPI0_MISO | SPI0 Master In, Slave Out | C | PC_10 |
| SPI0_MOSI | SPI0 Master Out, Slave In | C | PC_11 |
| SPI0_RDY | SPI0 Ready | C | PC_12 |
| SPI0_SEL1 | SPI0 Slave Select Output 1 | C | PC_07 |
| SPI0_SEL2 | SPI0 Slave Select Output 2 | D | PD_01 |
| SPI0_SEL3 | SPI0 Slave Select Output 3 | C | PC_12 |
| SPI0_SEL4 | SPI0 Slave Select Output 4 | C | PC_00 |
| SPI0_SEL5 | SPI0 Slave Select Output 5 | E | PE_01 |
| SPI0_SEL6 | SPI0 Slave Select Output 6 | E | PE_02 |
| SPI0_SEL7 | SPI0 Slave Select Output 7 | E | PE_03 |
| SPI0_SS | SPI0 Slave Select Input | D | PD_01 |
| SPI1_CLK | SPI1 Clock | E | PE_13 |
| SPI1_MISO | SPI1 Master In, Slave Out | E | PE_14 |
| SPI1_MOSI | SPI1 Master Out, Slave In | E | PE_15 |
| SPI1_RDY | SPI1 Ready | E | PE_08 |
| SPI1_SEL1 | SPI1 Slave Select Output 1 | C | PC_13 |
| SPI1_SEL2 | SPI1 Slave Select Output 2 | E | PE_07 |
| SPI1_SEL3 | SPI1 Slave Select Output 3 | E | PE_11 |
| SPI1_SEL4 | SPI1 Slave Select Output 4 | E | PE_12 |
| SPI1_SEL5 | SPI1 Slave Select Output 5 | E | PE_08 |
| SPI1_SS | SPI1 Slave Select Input | E | PE_11 |
| SPI2_CLK | SPI2 Clock | C | PC_01 |
| SPI2_D2 | SPI2 Data 2 | C | PC_04 |
| SPI2_D3 | SPI2 Data 3 | C | PC_05 |
| SPI2_MISO | SPI2 Master In, Slave Out | C | PC_02 |
| SPI2_MOSI | SPI2 Master Out, Slave In | C | PC_03 |
| SPI2_RDY | SPI2 Ready | E | PE_12 |
| SPI2_SEL1 | SPI2 Slave Select Output 1 | C | PC_06 |
| SPI2_SEL2 | SPI2 Slave Select Output 2 | E | PE_03 |
| SPI2_SEL3 | SPI2 Slave Select Output 3 | E | PE_04 |
| SPI2_SEL4 | SPI2 Slave Select Output 4 | E | PE_05 |
| SPI2_SEL5 | SPI2 Slave Select Output 5 | E | PE_06 |
| SPI2_SS | SPI2 Slave Select Input | C | PC_06 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| SYS_BMODE0 | Boot Mode Control n | Not Muxed | SYS_BMODE0 |
| SYS_BMODE1 | Boot Mode Control n | Not Muxed | SYS_BMODE1 |
| SYS_BMODE2 | Boot Mode Control n | Not Muxed | SYS_BMODE2 |
| SYS_CLKIN0 | Clock/Crystal Input | Not Muxed | SYS_CLKIN0 |
| SYS_CLKIN1 | Clock/Crystal Input | Not Muxed | SYS_CLKIN1 |
| SYS_CLKOUT | Processor Clock Output | Not Muxed | SYS_CLKOUT |
| SYS_FAULT | Active High Fault Output | Not Muxed | SYS_FAULT |
| SYS_FAULT | Active Low Fault Output | Not Muxed | SYS_FAULT |
| SYS_HWRST | Processor Hardware Reset Control | Not Muxed | SYS_HWRST |
| SYS_RESOUT | Reset Output | Not Muxed | SYS_RESOUT |
| SYS_XTAL0 | Crystal Output | Not Muxed | SYS_XTAL0 |
| SYS_XTAL1 | Crystal Output | Not Muxed | SYS_XTAL1 |
| TM0_ACI0 | TIMER0 Alternate Capture Input 0 | C | PC_14 |
| TM0_ACI1 | TIMER0 Alternate Capture Input 1 | B | PB_03 |
| TM0_ACI2 | TIMER0 Alternate Capture Input 2 | D | PD_13 |
| TM0_ACI3 | TIMER0 Alternate Capture Input 3 | C | PC_07 |
| TM0_ACI4 | TIMER0 Alternate Capture Input 4 | B | PB_10 |
| TM0_ACLK1 | TIMER0 Alternate Clock 1 | D | PD_08 |
| TM0_ACLK2 | TIMER0 Alternate Clock 2 | D | PD_09 |
| TM0_ACLK3 | TIMER0 Alternate Clock 3 | B | PB_00 |
| TM0_ACLK4 | TIMER0 Alternate Clock 4 | B | PB_01 |
| TM0_CLK | TIMER0 Clock | C | PC_11 |
| TM0_TMR0 | TIMER0 Timer 0 | E | PE_09 |
| TM0_TMR1 | TIMER0 Timer 1 | B | PB_15 |
| TM0_TMR2 | TIMER0 Timer 2 | B | PB_10 |
| TM0_TMR3 | TIMER0 Timer 3 | B | PB_07 |
| TM0_TMR4 | TIMER0 Timer 4 | B | PB_08 |
| TM0_TMR5 | TIMER0 Timer 5 | B | PB_14 |
| TRACE0_CLK | TRACE0 Trace Clock | D | PD_10 |
| TRACE0_D00 | TRACE0 Trace Data 0 | D | PD_02 |
| TRACE0_D01 | TRACE0 Trace Data 1 | D | PD_03 |
| TRACE0_D02 | TRACE0 Trace Data 2 | D | PD_04 |
| TRACE0_D03 | TRACE0 Trace Data 3 | D | PD_05 |
| TRACE0_D04 | TRACE0 Trace Data 4 | D | PD_06 |
| TRACE0_D05 | TRACE0 Trace Data 5 | D | PD_07 |
| TRACE0_D06 | TRACE0 Trace Data 6 | D | PD_08 |
| TRACE0_D07 | TRACE0 Trace Data 7 | D | PD_09 |
| TWI0_SCL | TWI0 Serial Clock | Not Muxed | TWI0_SCL |
| TWI0_SDA | TWI0 Serial Data | Not Muxed | TWI0_SDA |
| TWI1_SCL | TWI1 Serial Clock | Not Muxed | TWI1_SCL |
| TWI1_SDA | TWI1 Serial Data | Not Muxed | TWI1_SDA |
| TWI2_SCL | TWI2 Serial Clock | Not Muxed | TWI2_SCL |
| TWI2_SDA | TWI2 Serial Data | Not Muxed | TWI2_SDA |
| UART0_CTS | UART0 Clear to Send | D | PD_00 |
| UART0_RTS | UART0 Request to Send | C | PC_15 |
| UART0_RX | UART0 Receive | C | PC_14 |
| UART0_TX | UART0 Transmit | C | PC_13 |
| UART1_CTS | UART1 Clear to Send | E | PE_01 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| UART1_RTS | UART1 Request to Send | E | PE_02 |
| UART1_RX | UART1 Receive | B | PB_03 |
| UART1_TX | UART1 Transmit | B | PB_02 |
| UART2_CTS | UART2 Clear to Send | E | PE_11 |
| UART2_RTS | UART2 Request to Send | E | PE_10 |
| UART2_RX | UART2 Receive | D | PD_13 |
| UART2_TX | UART2 Transmit | D | PD_12 |
| USB0_CLKIN | USB0 Clock/Crystal Input | Not Muxed | USB_CLKIN |
| USB0_DM | USB0 Negative Data (–) | Not Muxed | USB0_DM |
| USB0_DP | USB0 Positive Data (+) | Not Muxed | USB0_DP |
| USB0_ID | USB0 OTG ID | Not Muxed | USB0_ID |
| USB0_VBC | USB0 VBUS Control | Not Muxed | USB0_VBC |
| USB0_VBUS | USB0 Bus Voltage | Not Muxed | USB0_VBUS |
| USB0_XTAL | USB0 Crystal | Not Muxed | USB_XTAL |
| VDD_DMC | DMC VDD | Not Muxed | VDD_DMC |
| VDD_HADC | HADC/TMU VDD | Not Muxed | VDD_HADC |
| VDD_USB | USB VDD | Not Muxed | VDD_USB |

GPIO MULTIPLEXING FOR THE 349-BALL CSP_BGA PACKAGE

Table 13 through Table 17 identify the pin functions that are multiplexed on the general-purpose I/O pins of the 349-ball CSP_BGA package.

##### Table 13.  Signal Multiplexing for Port A

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PA_00 | ETH0_TXD0 |  |  | SMC0_A21 |  |
| PA_01 | ETH0_TXD1 |  |  | SMC0_A20 |  |
| PA_02 | ETH0_MDC |  |  | SMC0_A24 |  |
| PA_03 | ETH0_MDIO |  |  | SMC0_A23 |  |
| PA_04 | ETH0_RXD0 |  |  | SMC0_A19 |  |
| PA_05 | ETH0_RXD1 |  |  | SMC0_A18 |  |
| PA_06 | ETH0_RXCLK_REFCLK |  |  | SMC0_A17 |  |
| PA_07 | ETH0_CRS |  |  | SMC0_A16 |  |
| PA_08 | ETH0_RXD2 |  |  | SMC0_A12 |  |
| PA_09 | ETH0_RXD3 |  |  | SMC0_A11 |  |
| PA_10 | ETH0_TXEN |  |  | SMC0_A22 |  |
| PA_11 | ETH0_TXCLK |  |  | SMC0_A15 |  |
| PA_12 | ETH0_TXD2 |  |  | SMC0_A14 |  |
| PA_13 | ETH0_TXD3 |  |  | SMC0_A13 |  |
| PA_14 | ETH0_PTPPPS3 | SINC0_D0 |  | SMC0_A10 |  |
| PA_15 | ETH0_PTPPPS2 | SINC0_D1 |  | SMC0_A09 |  |

Table 14.  Signal Multiplexing for Port B

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PB_00 PB_01 PB_02 PB_03 PB_04 PB_05 PB_06 PB_07 PB_08 PB_09 PB_10 PB_11 PB_12 PB_13 PB_14 PB_15 | ETH0_PTPPPS1 ETH0_PTPPPS0 ETH0_PTPCLKIN0 ETH0_PTPAUXIN0 MLB0_CLK MLB0_SIG MLB0_DAT LP1_D0 LP1_D1 LP1_D2 LP1_D3 LP1_D4 LP1_D5 LP1_D6 LP1_D7 LP1_ACK | SINC0_D2 SINC0_CLK0 UART1_TX UART1_RX SINC0_D3 PWM0_AH PWM0_AL TM0_TMR2 TM0_TMR5 PWM0_TRIP0 | PPI0_D14 PPI0_D15 PPI0_D16 PPI0_D17 PPI0_D12 PPI0_D13 PWM0_BH TM0_TMR3 TM0_TMR4 CAN1_TX CAN1_RX PWM0_DH PWM0_DL PWM0_CH PWM0_CL TM0_TMR1 | SMC0_A08 SMC0_A07 SMC0_A04 SMC0_A03 SMC0_ARDY SMC0_A01 SMC0_A02 SMC0_D15 SMC0_D14 SMC0_D13 SMC0_D12 SMC0_D11 SMC0_D10 SMC0_D09 SMC0_D08 SMC0_AWE | TM0_ACLK3 TM0_ACLK4 TM0_ACI1 ETH0_PTPAUXIN1 ETH0_PTPAUXIN2 ETH0_PTPAUXIN3 TM0_ACI4 CNT0_ZM CNT0_UD CNT0_DG |

Table 15.  Signal Multiplexing for Port C

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PC_00 PC_01 PC_02 PC_03 PC_04 PC_05 PC_06 PC_07 PC_08 PC_09 PC_10 PC_11 PC_12 PC_13 PC_14 PC_15 | LP1_CLK SPI2_CLK SPI2_MISO SPI2_MOSI SPI2_D2 SPI2_D3 SPI2_SEL1 CAN0_RX CAN0_TX SPI0_CLK SPI0_MISO SPI0_MOSI SPI0_SEL3 UART0_TX UART0_RX UART0_RTS | PWM0_BL SPI0_SEL1 SPI0_RDY SPI1_SEL1 PPI0_FS3 | SPI0_SEL4 ACM0_T0 ACM0_A0 ACM0_A1 ACM0_A2 | SMC0_ARE SMC0_AMS2 SMC0_AMS3 SMC0_A25 SMC0_AMS0 | SPI2_SS TM0_ACI3 TM0_CLK TM0_ACI0 |

Table 16. Signal Multiplexing for Port D

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PD_00 PD_01 PD_02 PD_03 PD_04 PD_05 PD_06 PD_07 PD_08 PD_09 PD_10 PD_11 PD_12 PD_13 PD_14 PD_15 | UART0_CTS SPI0_SEL2 LP0_D0 LP0_D1 LP0_D2 LP0_D3 LP0_D4 LP0_D5 LP0_D6 LP0_D7 LP0_CLK LP0_ACK UART2_TX UART2_RX PPI0_D11 PPI0_D10 | PPI0_D23 PWM1_TRIP0 PWM1_AH PWM1_AL PWM1_BH PWM1_BL PWM1_CH PWM1_CL PWM1_DH PWM1_DL PWM1_SYNC PWM2_TRIP0 PWM2_CH | ACM0_A3 ACM0_A4 TRACE0_D00 TRACE0_D01 TRACE0_D02 TRACE0_D03 TRACE0_D04 TRACE0_D05 TRACE0_D06 TRACE0_D07 TRACE0_CLK PPI0_D19 PPI0_D18 MLB0_CLKOUT | SMC0_D07 SMC0_AOE SMC0_A06 SMC0_A05 SMC0_D06 SMC0_D05 | SPI0_SS TM0_ACLK1 TM0_ACLK2 TM0_ACI2 |

Table 17. Signal Multiplexing for Port E

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PE_00 PE_01 PE_02 PE_03 PE_04 PE_05 PE_06 PE_07 PE_08 PE_09 PE_10 PE_11 PE_12 PE_13 PE_14 PE_15 | PPI0_D09 PPI0_FS2 PPI0_FS1 PPI0_CLK PPI0_D08 PPI0_D07 PPI0_D06 PPI0_D05 PPI0_D04 PPI0_D03 PPI0_D02 PPI0_D01 PPI0_D00 SPI1_CLK SPI1_MISO SPI1_MOSI | PWM2_CL SPI0_SEL5 SPI0_SEL6 SPI0_SEL7 PWM2_DH PWM2_SYNC SPI1_SEL5 PWM0_SYNC PWM2_DL SPI1_SEL3 SPI1_SEL4 | UART1_CTS UART1_RTS SPI2_SEL2 SPI2_SEL3 SPI2_SEL4 SPI2_SEL5 SPI1_SEL2 SPI1_RDY TM0_TMR0 UART2_RTS UART2_CTS SPI2_RDY PPI0_D20 PPI0_D21 PPI0_D22 | SMC0_D04 C1_FLG0 C2_FLG0 C1_FLG1 C2_FLG1 C1_FLG2 C2_FLG2 C1_FLG3 C2_FLG3 SMC0_D03 SMC0_D02 SMC0_D01 SMC0_D00 SMC0_AMS1 SMC0_ABE0 SMC0_ABE1 | SPI1_SS |

Table 18 shows the internal timer signal routing. This table applies to both the 349-ball and 529-ball CSP_BGA packages.

##### Table 18. Internal Timer Signal Routing

| Timer Input Signal | Internal Source |
| --- | --- |
| TM0_ACLK0 | SYS_CLKIN1 |
| TM0_ACI5 | DAI0_CRS_PB04_O |
| TM0_ACLK5 | DAI0_CRS_PB03_O |
| TM0_ACI6 | DAI1_CRS_PB04_O |
| TM0_ACLK6 | DAI1_CRS_PB03_O |
| TM0_ACI7 | CNT0_TO |
| TM0_ACLK7 | SYS_CLKIN0 |

529-BALL CSP_BGA SIGNAL DESCRIPTIONS

The processor pin definitions are shown Table 19 for the

529-ball CSP_BGA package. The columns in this table provide the following information:

- The Signal Name column includes the signal name for every pin and the GPIO multiplexed pin function, where applicable.

- The Description column provides a descriptive name for each signal.

- The Port column shows whether or not a signal is multi- plexed with other signals on a general-purpose I/O port pin.

- The Pin Name column identifies the name of the package pin (at power on reset) on which the signal is located (if a single function pin) or is multiplexed (if a general-purpose I/O pin).

- The DAI pins and their associated signal routing units (SRUs) connect inputs and outputs of the DAI peripherals (SPORT, ASRC, S/PDIF, and PCG). See the “Digital Audio Interface (DAI)” chapter of the ADSP-SC58x/ADSP-2158x SHARC+ Processor Hardware Reference for complete information on the use of the DAIs and SRUs.

##### Table 19. ADSP-SC58x/ADSP-2158x 529-Ball CSP_BGA Signal Descriptions

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| ACM0_A0 | ACM0 ADC Control Signals | C | PC_13 |
| ACM0_A1 | ACM0 ADC Control Signals | C | PC_14 |
| ACM0_A2 | ACM0 ADC Control Signals | C | PC_15 |
| ACM0_A3 | ACM0 ADC Control Signals | D | PD_00 |
| ACM0_A4 | ACM0 ADC Control Signals | D | PD_01 |
| ACM0_T0 | ACM0 External Trigger n | C | PC_12 |
| C1_FLG0 | SHARC Core 1 Flag Pin | E | PE_01 |
| C1_FLG1 | SHARC Core 1 Flag Pin | E | PE_03 |
| C1_FLG2 | SHARC Core 1 Flag Pin | E | PE_05 |
| C1_FLG3 | SHARC Core 1 Flag Pin | E | PE_07 |
| C2_FLG0 | SHARC Core 2 Flag Pin | E | PE_02 |
| C2_FLG1 | SHARC Core 2 Flag Pin | E | PE_04 |
| C2_FLG2 | SHARC Core 2 Flag Pin | E | PE_06 |
| C2_FLG3 | SHARC Core 2 Flag Pin | E | PE_08 |
| CAN0_RX | CAN0 Receive | C | PC_07 |
| CAN0_TX | CAN0 Transmit | C | PC_08 |
| CAN1_RX | CAN1 Receive | B | PB_10 |
| CAN1_TX | CAN1 Transmit | B | PB_09 |
| CNT0_DG | CNT0 Count Down and Gate | B | PB_14 |
| CNT0_UD | CNT0 Count Up and Direction | B | PB_12 |
| CNT0_ZM | CNT0 Count Zero Marker | B | PB_11 |
| DAI0_PIN01 | DAI0 Pin 1 | Not Muxed | DAI0_PIN01 |
| DAI0_PIN02 | DAI0 Pin 2 | Not Muxed | DAI0_PIN02 |
| DAI0_PIN03 | DAI0 Pin 3 | Not Muxed | DAI0_PIN03 |
| DAI0_PIN04 | DAI0 Pin 4 | Not Muxed | DAI0_PIN04 |
| DAI0_PIN05 | DAI0 Pin 5 | Not Muxed | DAI0_PIN05 |
| DAI0_PIN06 | DAI0 Pin 6 | Not Muxed | DAI0_PIN06 |
| DAI0_PIN07 | DAI0 Pin 7 | Not Muxed | DAI0_PIN07 |
| DAI0_PIN08 | DAI0 Pin 8 | Not Muxed | DAI0_PIN08 |
| DAI0_PIN09 | DAI0 Pin 9 | Not Muxed | DAI0_PIN09 |
| DAI0_PIN10 | DAI0 Pin 10 | Not Muxed | DAI0_PIN10 |
| DAI0_PIN11 | DAI0 Pin 11 | Not Muxed | DAI0_PIN11 |
| DAI0_PIN12 | DAI0 Pin 12 | Not Muxed | DAI0_PIN12 |
| DAI0_PIN13 | DAI0 Pin 13 | Not Muxed | DAI0_PIN13 |
| DAI0_PIN14 | DAI0 Pin 14 | Not Muxed | DAI0_PIN14 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| DAI0_PIN15 | DAI0 Pin 15 | Not Muxed | DAI0_PIN15 |
| DAI0_PIN16 | DAI0 Pin 16 | Not Muxed | DAI0_PIN16 |
| DAI0_PIN17 | DAI0 Pin 17 | Not Muxed | DAI0_PIN17 |
| DAI0_PIN18 | DAI0 Pin 18 | Not Muxed | DAI0_PIN18 |
| DAI0_PIN19 | DAI0 Pin 19 | Not Muxed | DAI0_PIN19 |
| DAI0_PIN20 | DAI0 Pin 20 | Not Muxed | DAI0_PIN20 |
| DAI1_PIN01 | DAI1 Pin 1 | Not Muxed | DAI1_PIN01 |
| DAI1_PIN02 | DAI1 Pin 2 | Not Muxed | DAI1_PIN02 |
| DAI1_PIN03 | DAI1 Pin 3 | Not Muxed | DAI1_PIN03 |
| DAI1_PIN04 | DAI1 Pin 4 | Not Muxed | DAI1_PIN04 |
| DAI1_PIN05 | DAI1 Pin 5 | Not Muxed | DAI1_PIN05 |
| DAI1_PIN06 | DAI1 Pin 6 | Not Muxed | DAI1_PIN06 |
| DAI1_PIN07 | DAI1 Pin 7 | Not Muxed | DAI1_PIN07 |
| DAI1_PIN08 | DAI1 Pin 8 | Not Muxed | DAI1_PIN08 |
| DAI1_PIN09 | DAI1 Pin 9 | Not Muxed | DAI1_PIN09 |
| DAI1_PIN10 | DAI1 Pin 10 | Not Muxed | DAI1_PIN10 |
| DAI1_PIN11 | DAI1 Pin 11 | Not Muxed | DAI1_PIN11 |
| DAI1_PIN12 | DAI1 Pin 12 | Not Muxed | DAI1_PIN12 |
| DAI1_PIN13 | DAI1 Pin 13 | Not Muxed | DAI1_PIN13 |
| DAI1_PIN14 | DAI1 Pin 14 | Not Muxed | DAI1_PIN14 |
| DAI1_PIN15 | DAI1 Pin 15 | Not Muxed | DAI1_PIN15 |
| DAI1_PIN16 | DAI1 Pin 16 | Not Muxed | DAI1_PIN16 |
| DAI1_PIN17 | DAI1 Pin 17 | Not Muxed | DAI1_PIN17 |
| DAI1_PIN18 | DAI1 Pin 18 | Not Muxed | DAI1_PIN18 |
| DAI1_PIN19 | DAI1 Pin 19 | Not Muxed | DAI1_PIN19 |
| DAI1_PIN20 | DAI1 Pin 20 | Not Muxed | DAI1_PIN20 |
| DMC0_A00 | DMC0 Address 0 | Not Muxed | DMC0_A00 |
| DMC0_A01 | DMC0 Address 1 | Not Muxed | DMC0_A01 |
| DMC0_A02 | DMC0 Address 2 | Not Muxed | DMC0_A02 |
| DMC0_A03 | DMC0 Address 3 | Not Muxed | DMC0_A03 |
| DMC0_A04 | DMC0 Address 4 | Not Muxed | DMC0_A04 |
| DMC0_A05 | DMC0 Address 5 | Not Muxed | DMC0_A05 |
| DMC0_A06 | DMC0 Address 6 | Not Muxed | DMC0_A06 |
| DMC0_A07 | DMC0 Address 7 | Not Muxed | DMC0_A07 |
| DMC0_A08 | DMC0 Address 8 | Not Muxed | DMC0_A08 |
| DMC0_A09 | DMC0 Address 9 | Not Muxed | DMC0_A09 |
| DMC0_A10 | DMC0 Address 10 | Not Muxed | DMC0_A10 |
| DMC0_A11 | DMC0 Address 11 | Not Muxed | DMC0_A11 |
| DMC0_A12 | DMC0 Address 12 | Not Muxed | DMC0_A12 |
| DMC0_A13 | DMC0 Address 13 | Not Muxed | DMC0_A13 |
| DMC0_A14 | DMC0 Address 14 | Not Muxed | DMC0_A14 |
| DMC0_A15 | DMC0 Address 15 | Not Muxed | DMC0_A15 |
| DMC0_BA0 | DMC0 Bank Address 0 | Not Muxed | DMC0_BA0 |
| DMC0_BA1 | DMC0 Bank Address 1 | Not Muxed | DMC0_BA1 |
| DMC0_BA2 | DMC0 Bank Address 2 | Not Muxed | DMC0_BA2 |
| DMC0_CAS | DMC0 Column Address Strobe | Not Muxed | DMC0_CAS |
| DMC0_CK | DMC0 Clock | Not Muxed | DMC0_CK |
| DMC0_CKE | DMC0 Clock Enable | Not Muxed | DMC0_CKE |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| DMC0_CK | DMC0 Clock (Complement) | Not Muxed | DMC0_CK |
| DMC0_CS0 | DMC0 Chip Select 0 | Not Muxed | DMC0_CS0 |
| DMC0_DQ00 | DMC0 Data 0 | Not Muxed | DMC0_DQ00 |
| DMC0_DQ01 | DMC0 Data 1 | Not Muxed | DMC0_DQ01 |
| DMC0_DQ02 | DMC0 Data 2 | Not Muxed | DMC0_DQ02 |
| DMC0_DQ03 | DMC0 Data 3 | Not Muxed | DMC0_DQ03 |
| DMC0_DQ04 | DMC0 Data 4 | Not Muxed | DMC0_DQ04 |
| DMC0_DQ05 | DMC0 Data 5 | Not Muxed | DMC0_DQ05 |
| DMC0_DQ06 | DMC0 Data 6 | Not Muxed | DMC0_DQ06 |
| DMC0_DQ07 | DMC0 Data 7 | Not Muxed | DMC0_DQ07 |
| DMC0_DQ08 | DMC0 Data 8 | Not Muxed | DMC0_DQ08 |
| DMC0_DQ09 | DMC0 Data 9 | Not Muxed | DMC0_DQ09 |
| DMC0_DQ10 | DMC0 Data 10 | Not Muxed | DMC0_DQ10 |
| DMC0_DQ11 | DMC0 Data 11 | Not Muxed | DMC0_DQ11 |
| DMC0_DQ12 | DMC0 Data 12 | Not Muxed | DMC0_DQ12 |
| DMC0_DQ13 | DMC0 Data 13 | Not Muxed | DMC0_DQ13 |
| DMC0_DQ14 | DMC0 Data 14 | Not Muxed | DMC0_DQ14 |
| DMC0_DQ15 | DMC0 Data 15 | Not Muxed | DMC0_DQ15 |
| DMC0_LDM | DMC0 Data Mask for Lower Byte | Not Muxed | DMC0_LDM |
| DMC0_LDQS | DMC0 Data Strobe for Lower Byte | Not Muxed | DMC0_LDQS |
| DMC0_LDQS | DMC0 Data Strobe for Lower Byte (Complement) | Not Muxed | DMC0_LDQS |
| DMC0_ODT | DMC0 On-Die Termination | Not Muxed | DMC0_ODT |
| DMC0_RAS | DMC0 Row Address Strobe | Not Muxed | DMC0_RAS |
| DMC0_RESET | DMC0 Reset (DDR3 Only) | Not Muxed | DMC0_RESET |
| DMC0_RZQ | DMC0 External Calibration Resistor Connection | Not Muxed | DMC0_RZQ |
| DMC0_UDM | DMC0 Data Mask for Upper Byte | Not Muxed | DMC0_UDM |
| DMC0_UDQS | DMC0 Data Strobe for Upper Byte | Not Muxed | DMC0_UDQS |
| DMC0_UDQS | DMC0 Data Strobe for Upper Byte (Complement) | Not Muxed | DMC0_UDQS |
| DMC0_VREF | DMC0 Voltage Reference | Not Muxed | DMC0_VREF |
| DMC0_WE | DMC0 Write Enable | Not Muxed | DMC0_WE |
| DMC1_A00 | DMC1 Address 0 | Not Muxed | DMC1_A00 |
| DMC1_A01 | DMC1 Address 1 | Not Muxed | DMC1_A01 |
| DMC1_A02 | DMC1 Address 2 | Not Muxed | DMC1_A02 |
| DMC1_A03 | DMC1 Address 3 | Not Muxed | DMC1_A03 |
| DMC1_A04 | DMC1 Address 4 | Not Muxed | DMC1_A04 |
| DMC1_A05 | DMC1 Address 5 | Not Muxed | DMC1_A05 |
| DMC1_A06 | DMC1 Address 6 | Not Muxed | DMC1_A06 |
| DMC1_A07 | DMC1 Address 7 | Not Muxed | DMC1_A07 |
| DMC1_A08 | DMC1 Address 8 | Not Muxed | DMC1_A08 |
| DMC1_A09 | DMC1 Address 9 | Not Muxed | DMC1_A09 |
| DMC1_A10 | DMC1 Address 10 | Not Muxed | DMC1_A10 |
| DMC1_A11 | DMC1 Address 11 | Not Muxed | DMC1_A11 |
| DMC1_A12 | DMC1 Address 12 | Not Muxed | DMC1_A12 |
| DMC1_A13 | DMC1 Address 13 | Not Muxed | DMC1_A13 |
| DMC1_A14 | DMC1 Address 14 | Not Muxed | DMC1_A14 |
| DMC1_A15 | DMC1 Address 15 | Not Muxed | DMC1_A15 |
| DMC1_BA0 | DMC1 Bank Address 0 | Not Muxed | DMC1_BA0 |
| DMC1_BA1 | DMC1 Bank Address 1 | Not Muxed | DMC1_BA1 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| DMC1_BA2 | DMC1 Bank Address 2 | Not Muxed | DMC1_BA2 |
| DMC1_CAS | DMC1 Column Address Strobe | Not Muxed | DMC1_CAS |
| DMC1_CK | DMC1 Clock | Not Muxed | DMC1_CK |
| DMC1_CKE | DMC1 Clock Enable | Not Muxed | DMC1_CKE |
| DMC1_CK | DMC1 Clock (Complement) | Not Muxed | DMC1_CK |
| DMC1_CS0 | DMC1 Chip Select 0 | Not Muxed | DMC1_CS0 |
| DMC1_DQ00 | DMC1 Data 0 | Not Muxed | DMC1_DQ00 |
| DMC1_DQ01 | DMC1 Data 1 | Not Muxed | DMC1_DQ01 |
| DMC1_DQ02 | DMC1 Data 2 | Not Muxed | DMC1_DQ02 |
| DMC1_DQ03 | DMC1 Data 3 | Not Muxed | DMC1_DQ03 |
| DMC1_DQ04 | DMC1 Data 4 | Not Muxed | DMC1_DQ04 |
| DMC1_DQ05 | DMC1 Data 5 | Not Muxed | DMC1_DQ05 |
| DMC1_DQ06 | DMC1 Data 6 | Not Muxed | DMC1_DQ06 |
| DMC1_DQ07 | DMC1 Data 7 | Not Muxed | DMC1_DQ07 |
| DMC1_DQ08 | DMC1 Data 8 | Not Muxed | DMC1_DQ08 |
| DMC1_DQ09 | DMC1 Data 9 | Not Muxed | DMC1_DQ09 |
| DMC1_DQ10 | DMC1 Data 10 | Not Muxed | DMC1_DQ10 |
| DMC1_DQ11 | DMC1 Data 11 | Not Muxed | DMC1_DQ11 |
| DMC1_DQ12 | DMC1 Data 12 | Not Muxed | DMC1_DQ12 |
| DMC1_DQ13 | DMC1 Data 13 | Not Muxed | DMC1_DQ13 |
| DMC1_DQ14 | DMC1 Data 14 | Not Muxed | DMC1_DQ14 |
| DMC1_DQ15 | DMC1 Data 15 | Not Muxed | DMC1_DQ15 |
| DMC1_LDM | DMC1 Data Mask for Lower Byte | Not Muxed | DMC1_LDM |
| DMC1_LDQS | DMC1 Data Strobe for Lower Byte | Not Muxed | DMC1_LDQS |
| DMC1_LDQS | DMC1 Data Strobe for Lower Byte (Complement) | Not Muxed | DMC1_LDQS |
| DMC1_ODT | DMC1 On-Die Termination | Not Muxed | DMC1_ODT |
| DMC1_RAS | DMC1 Row Address Strobe | Not Muxed | DMC1_RAS |
| DMC1_RESET | DMC1 Reset (DDR3 Only) | Not Muxed | DMC1_RESET |
| DMC1_RZQ | DMC1 External Calibration Resistor Connection | Not Muxed | DMC1_RZQ |
| DMC1_UDM | DMC1 Data Mask for Upper Byte | Not Muxed | DMC1_UDM |
| DMC1_UDQS | DMC1 Data Strobe for Upper Byte | Not Muxed | DMC1_UDQS |
| DMC1_UDQS | DMC1 Data Strobe for Upper Byte (Complement) | Not Muxed | DMC1_UDQS |
| DMC1_VREF | DMC1 Voltage Reference | Not Muxed | DMC1_VREF |
| DMC1_WE | DMC1 Write Enable | Not Muxed | DMC1_WE |
| ETH0_CRS | ETH0 Carrier Sense/RMII Receive Data Valid | A | PA_07 |
| ETH0_MDC | ETH0 Management Channel Clock | A | PA_02 |
| ETH0_MDIO | ETH0 Management Channel Serial Data | A | PA_03 |
| ETH0_PTPAUXIN0 | ETH0 PTP Auxiliary Trigger Input 0 | B | PB_03 |
| ETH0_PTPAUXIN1 | ETH0 PTP Auxiliary Trigger Input 1 | B | PB_04 |
| ETH0_PTPAUXIN2 | ETH0 PTP Auxiliary Trigger Input 2 | B | PB_05 |
| ETH0_PTPAUXIN3 | ETH0 PTP Auxiliary Trigger Input 3 | B | PB_06 |
| ETH0_PTPCLKIN0 | ETH0 PTP Clock Input 0 | B | PB_02 |
| ETH0_PTPPPS0 | ETH0 PTP Pulse-Per-Second Output 0 | B | PB_01 |
| ETH0_PTPPPS1 | ETH0 PTP Pulse-Per-Second Output 1 | B | PB_00 |
| ETH0_PTPPPS2 | ETH0 PTP Pulse-Per-Second Output 2 | A | PA_15 |
| ETH0_PTPPPS3 | ETH0 PTP Pulse-Per-Second Output 3 | A | PA_14 |
| ETH0_RXCLK_REFCLK | ETH0 RXCLK (10/100/1000) or REFCLK (10/100) | A | PA_06 |
| ETH0_RXCTL_CRS | ETH0 RXCTL (10/100/1000) or CRS (10/100) | A | PA_07 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| ETH0_RXD0 | ETH0 Receive Data 0 | A | PA_04 |
| ETH0_RXD1 | ETH0 Receive Data 1 | A | PA_05 |
| ETH0_RXD2 | ETH0 Receive Data 2 | A | PA_08 |
| ETH0_RXD3 | ETH0 Receive Data 3 | A | PA_09 |
| ETH0_TXCLK | ETH0 Transmit Clock | A | PA_11 |
| ETH0_TXCTL_TXEN | ETH0 TXCTL (10/100/1000) or TXEN (10/100) | A | PA_10 |
| ETH0_TXD0 | ETH0 Transmit Data 0 | A | PA_00 |
| ETH0_TXD1 | ETH0 Transmit Data 1 | A | PA_01 |
| ETH0_TXD2 | ETH0 Transmit Data 2 | A | PA_12 |
| ETH0_TXD3 | ETH0 Transmit Data 3 | A | PA_13 |
| ETH0_TXEN | ETH0 Transmit Enable | A | PA_10 |
| ETH1_CRS | ETH1 Carrier Sense/RMII Receive Data Valid | F | PF_13 |
| ETH1_MDC | ETH1 Management Channel Clock | F | PF_14 |
| ETH1_MDIO | ETH1 Management Channel Serial Data | F | PF_15 |
| ETH1_REFCLK | ETH1 Reference Clock | G | PG_00 |
| ETH1_RXD0 | ETH1 Receive Data 0 | G | PG_04 |
| ETH1_RXD1 | ETH1 Receive Data 1 | G | PG_05 |
| ETH1_TXD0 | ETH1 Transmit Data 0 | G | PG_02 |
| ETH1_TXD1 | ETH1 Transmit Data 1 | G | PG_03 |
| ETH1_TXEN | ETH1 Transmit Enable | G | PG_01 |
| HADC0_EOC_DOUT | HADC0 End of Conversion/Serial Data Out | F | PF_02 |
| HADC0_MUX0 | HADC0 Controls to External Multiplexer | F | PF_05 |
| HADC0_MUX1 | HADC0 Controls to External Multiplexer | F | PF_04 |
| HADC0_MUX2 | HADC0 Controls to External Multiplexer | F | PF_03 |
| HADC0_VIN0 | HADC0 Analog Input at Channel 0 | Not Muxed | HADC0_VIN0 |
| HADC0_VIN1 | HADC0 Analog Input at Channel 1 | Not Muxed | HADC0_VIN1 |
| HADC0_VIN2 | HADC0 Analog Input at Channel 2 | Not Muxed | HADC0_VIN2 |
| HADC0_VIN3 | HADC0 Analog Input at Channel 3 | Not Muxed | HADC0_VIN3 |
| HADC0_VIN4 | HADC0 Analog Input at Channel 4 | Not Muxed | HADC0_VIN4 |
| HADC0_VIN5 | HADC0 Analog Input at Channel 5 | Not Muxed | HADC0_VIN5 |
| HADC0_VIN6 | HADC0 Analog Input at Channel 6 | Not Muxed | HADC0_VIN6 |
| HADC0_VIN7 | HADC0 Analog Input at Channel 7 | Not Muxed | HADC0_VIN7 |
| HADC0_VREFN | HADC0 Ground Reference for ADC | Not Muxed | HADC0_VREFN |
| HADC0_VREFP | HADC0 External Reference for ADC | Not Muxed | HADC0_VREFP |
| JTG_TCK | TAPC JTAG Clock | Not Muxed | JTG_TCK |
| JTG_TDI | TAPC JTAG Serial Data In | Not Muxed | JTG_TDI |
| JTG_TDO | TAPC JTAG Serial Data Out | Not Muxed | JTG_TDO |
| JTG_TMS | TAPC JTAG Mode Select | Not Muxed | JTG_TMS |
| JTG_TRST | TAPC JTAG Reset | Not Muxed | JTG_TRST |
| LP0_ACK | LP0 Acknowledge | D | PD_11 |
| LP0_CLK | LP0 Clock | D | PD_10 |
| LP0_D0 | LP0 Data 0 | D | PD_02 |
| LP0_D1 | LP0 Data 1 | D | PD_03 |
| LP0_D2 | LP0 Data 2 | D | PD_04 |
| LP0_D3 | LP0 Data 3 | D | PD_05 |
| LP0_D4 | LP0 Data 4 | D | PD_06 |
| LP0_D5 | LP0 Data 5 | D | PD_07 |
| LP0_D6 | LP0 Data 6 | D | PD_08 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| LP0_D7 | LP0 Data 7 | D | PD_09 |
| LP1_ACK | LP1 Acknowledge | B | PB_15 |
| LP1_CLK | LP1 Clock | C | PC_00 |
| LP1_D0 | LP1 Data 0 | B | PB_07 |
| LP1_D1 | LP1 Data 1 | B | PB_08 |
| LP1_D2 | LP1 Data 2 | B | PB_09 |
| LP1_D3 | LP1 Data 3 | B | PB_10 |
| LP1_D4 | LP1 Data 4 | B | PB_11 |
| LP1_D5 | LP1 Data 5 | B | PB_12 |
| LP1_D6 | LP1 Data 6 | B | PB_13 |
| LP1_D7 | LP1 Data 7 | B | PB_14 |
| MLB0_CLKN | MLB0 Differential Clock (–) | Not Muxed | MLB0_CLKN |
| MLB0_CLKP | MLB0 Differential Clock (+) | Not Muxed | MLB0_CLKP |
| MLB0_DATN | MLB0 Differential Data (–) | Not Muxed | MLB0_DATN |
| MLB0_DATP | MLB0 Differential Data (+) | Not Muxed | MLB0_DATP |
| MLB0_SIGN | MLB0 Differential Signal (–) | Not Muxed | MLB0_SIGN |
| MLB0_SIGP | MLB0 Differential Signal (+) | Not Muxed | MLB0_SIGP |
| MLB0_CLK | MLB0 Single-Ended Clock | B | PB_04 |
| MLB0_DAT | MLB0 Single-Ended Data | B | PB_06 |
| MLB0_SIG | MLB0 Single-Ended Signal | B | PB_05 |
| MLB0_CLKOUT | MLB0 Single-Ended Clock Out | D | PD_14 |
| MSI0_CD | MSI0 Card Detect | F | PF_12 |
| MSI0_CLK | MSI0 Clock | F | PF_11 |
| MSI0_CMD | MSI0 Command | F | PF_10 |
| MSI0_D0 | MSI0 Data 0 | F | PF_02 |
| MSI0_D1 | MSI0 Data 1 | F | PF_03 |
| MSI0_D2 | MSI0 Data 2 | F | PF_04 |
| MSI0_D3 | MSI0 Data 3 | F | PF_05 |
| MSI0_D4 | MSI0 Data 4 | F | PF_06 |
| MSI0_D5 | MSI0 Data 5 | F | PF_07 |
| MSI0_D6 | MSI0 Data 6 | F | PF_08 |
| MSI0_D7 | MSI0 Data 7 | F | PF_09 |
| MSI0_INT | MSI0 eSDIO Interrupt Input | F | PF_13 |
| PA_00-15 | PORTA Position 00 Through Position 15 | A | PA_00-15 |
| PB_00-15 | PORTB Position 00 Through Position 15 | B | PB_00-15 |
| PCIE0_CLKM | PCIE0 CLK − | Not Muxed | PCIE0_CLKM |
| PCIE0_CLKP | PCIE0 CLK + | Not Muxed | PCIE0_CLKP |
| PCIE0_REF | PCIE0 Reference | Not Muxed | PCIE0_REF |
| PCIE0_RXM | PCIE0 RX − | Not Muxed | PCIE0_RXM |
| PCIE0_RXP | PCIE0 RX + | Not Muxed | PCIE0_RXP |
| PCIE0_TXM | PCIE0 TX − | Not Muxed | PCIE0_TXM |
| PCIE0_TXP | PCIE0 TX + | Not Muxed | PCIE0_TXP |
| PC_00-15 | PORTC Position 00 Through Position 15 | C | PC_00-15 |
| PD_00-15 | PORTD Position 00 Through Position 15 | D | PD_00-15 |
| PE_00-15 | PORTE Position 00 Through Position 15 | E | PE_00-15 |
| PF_00-15 | PORTF Position 00 Through Position 15 | F | PF_00-15 |
| PG_00-5 | PORTG Position 00 Through Position 5 | G | PG_00-5 |
| PPI0_CLK | EPPI0 Clock | E | PE_03 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| PPI0_D00 | EPPI0 Data 0 | E | PE_12 |
| PPI0_D01 | EPPI0 Data 1 | E | PE_11 |
| PPI0_D02 | EPPI0 Data 2 | E | PE_10 |
| PPI0_D03 | EPPI0 Data 3 | E | PE_09 |
| PPI0_D04 | EPPI0 Data 4 | E | PE_08 |
| PPI0_D05 | EPPI0 Data 5 | E | PE_07 |
| PPI0_D06 | EPPI0 Data 6 | E | PE_06 |
| PPI0_D07 | EPPI0 Data 7 | E | PE_05 |
| PPI0_D08 | EPPI0 Data 8 | E | PE_04 |
| PPI0_D09 | EPPI0 Data 9 | E | PE_00 |
| PPI0_D10 | EPPI0 Data 10 | D | PD_15 |
| PPI0_D11 | EPPI0 Data 11 | D | PD_14 |
| PPI0_D12 | EPPI0 Data 12 | B | PB_04 |
| PPI0_D13 | EPPI0 Data 13 | B | PB_05 |
| PPI0_D14 | EPPI0 Data 14 | B | PB_00 |
| PPI0_D15 | EPPI0 Data 15 | B | PB_01 |
| PPI0_D16 | EPPI0 Data 16 | B | PB_02 |
| PPI0_D17 | EPPI0 Data 17 | B | PB_03 |
| PPI0_D18 | EPPI0 Data 18 | D | PD_13 |
| PPI0_D19 | EPPI0 Data 19 | D | PD_12 |
| PPI0_D20 | EPPI0 Data 20 | E | PE_13 |
| PPI0_D21 | EPPI0 Data 21 | E | PE_14 |
| PPI0_D22 | EPPI0 Data 22 | E | PE_15 |
| PPI0_D23 | EPPI0 Data 23 | D | PD_00 |
| PPI0_FS1 | EPPI0 Frame Sync 1 (HSYNC) | E | PE_02 |
| PPI0_FS2 | EPPI0 Frame Sync 2 (VSYNC) | E | PE_01 |
| PPI0_FS3 | EPPI0 Frame Sync 3 (FIELD) | C | PC_15 |
| PWM0_AH | PWM0 Channel A High Side | B | PB_07 |
| PWM0_AL | PWM0 Channel A Low Side | B | PB_08 |
| PWM0_BH | PWM0 Channel B High Side | B | PB_06 |
| PWM0_BL | PWM0 Channel B Low Side | C | PC_00 |
| PWM0_CH | PWM0 Channel C High Side | B | PB_13 |
| PWM0_CL | PWM0 Channel C Low Side | B | PB_14 |
| PWM0_DH | PWM0 Channel D High Side | B | PB_11 |
| PWM0_DL | PWM0 Channel D Low Side | B | PB_12 |
| PWM0_SYNC | PWM0 PWMTMR Grouped | E | PE_09 |
| PWM0_TRIP0 | PWM0 Shutdown Input 0 | B | PB_15 |
| PWM1_AH | PWM1 Channel A High Side | D | PD_03 |
| PWM1_AL | PWM1 Channel A Low Side | D | PD_04 |
| PWM1_BH | PWM1 Channel B High Side | D | PD_05 |
| PWM1_BL | PWM1 Channel B Low Side | D | PD_06 |
| PWM1_CH | PWM1 Channel C High Side | D | PD_07 |
| PWM1_CL | PWM1 Channel C Low Side | D | PD_08 |
| PWM1_DH | PWM1 Channel D High Side | D | PD_09 |
| PWM1_DL | PWM1 Channel D Low Side | D | PD_10 |
| PWM1_SYNC | PWM1 PWMTMR Grouped | D | PD_11 |
| PWM1_TRIP0 | PWM1 Shutdown Input 0 | D | PD_02 |
| PWM2_AH | PWM2 Channel A High Side | F | PF_07 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| PWM2_AL | PWM2 Channel A Low Side | F | PF_06 |
| PWM2_BH | PWM2 Channel B High Side | F | PF_09 |
| PWM2_BL | PWM2 Channel B Low Side | F | PF_08 |
| PWM2_CH | PWM2 Channel C High Side | D | PD_15 |
| PWM2_CL | PWM2 Channel C Low Side | E | PE_00 |
| PWM2_DH | PWM2 Channel D High Side | E | PE_04 |
| PWM2_DL | PWM2 Channel D Low Side | E | PE_10 |
| PWM2_SYNC | PWM2 PWMTMR Grouped | E | PE_05 |
| PWM2_TRIP0 | PWM2 Shutdown Input 0 | D | PD_14 |
| GND | Ground | Not Muxed | GND |
| VDD_EXT | External Voltage Domain | Not Muxed | VDD_EXT |
| VDD_INT | Internal Voltage Domain | Not Muxed | VDD_INT |
| RTC0_CLKIN | RTC0 Crystal Input/External Oscillator Connection | Not Muxed | RTC0_CLKIN |
| RTC0_XTAL | RTC0 Crystal Output | Not Muxed | RTC0_XTAL |
| SINC0_CLK0 | SINC0 Clock 0 | B | PB_01 |
| SINC0_D0 | SINC0 Data 0 | A | PA_14 |
| SINC0_D1 | SINC0 Data 1 | A | PA_15 |
| SINC0_D2 | SINC0 Data 2 | B | PB_00 |
| SINC0_D3 | SINC0 Data 3 | B | PB_04 |
| SMC0_A01 | SMC0 Address 1 | B | PB_05 |
| SMC0_A02 | SMC0 Address 2 | B | PB_06 |
| SMC0_A03 | SMC0 Address 3 | B | PB_03 |
| SMC0_A04 | SMC0 Address 4 | B | PB_02 |
| SMC0_A05 | SMC0 Address 5 | D | PD_13 |
| SMC0_A06 | SMC0 Address 6 | D | PD_12 |
| SMC0_A07 | SMC0 Address 7 | B | PB_01 |
| SMC0_A08 | SMC0 Address 8 | B | PB_00 |
| SMC0_A09 | SMC0 Address 9 | A | PA_15 |
| SMC0_A10 | SMC0 Address 10 | A | PA_14 |
| SMC0_A11 | SMC0 Address 11 | A | PA_09 |
| SMC0_A12 | SMC0 Address 12 | A | PA_08 |
| SMC0_A13 | SMC0 Address 13 | A | PA_13 |
| SMC0_A14 | SMC0 Address 14 | A | PA_12 |
| SMC0_A15 | SMC0 Address 15 | A | PA_11 |
| SMC0_A16 | SMC0 Address 16 | A | PA_07 |
| SMC0_A17 | SMC0 Address 17 | A | PA_06 |
| SMC0_A18 | SMC0 Address 18 | A | PA_05 |
| SMC0_A19 | SMC0 Address 19 | A | PA_04 |
| SMC0_A20 | SMC0 Address 20 | A | PA_01 |
| SMC0_A21 | SMC0 Address 21 | A | PA_00 |
| SMC0_A22 | SMC0 Address 22 | A | PA_10 |
| SMC0_A23 | SMC0 Address 23 | A | PA_03 |
| SMC0_A24 | SMC0 Address 24 | A | PA_02 |
| SMC0_A25 | SMC0 Address 25 | C | PC_12 |
| SMC0_ABE0 | SMC0 Byte Enable 0 | E | PE_14 |
| SMC0_ABE1 | SMC0 Byte Enable 1 | E | PE_15 |
| SMC0_AMS0 | SMC0 Memory Select 0 | C | PC_15 |
| SMC0_AMS1 | SMC0 Memory Select 1 | E | PE_13 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| SMC0_AMS2 | SMC0 Memory Select 2 | C | PC_07 |
| SMC0_AMS3 | SMC0 Memory Select 3 | C | PC_08 |
| SMC0_AOE | SMC0 Output Enable | D | PD_01 |
| SMC0_ARDY | SMC0 Asynchronous Ready | B | PB_04 |
| SMC0_ARE | SMC0 Read Enable | C | PC_00 |
| SMC0_AWE | SMC0 Write Enable | B | PB_15 |
| SMC0_D00 | SMC0 Data 0 | E | PE_12 |
| SMC0_D01 | SMC0 Data 1 | E | PE_11 |
| SMC0_D02 | SMC0 Data 2 | E | PE_10 |
| SMC0_D03 | SMC0 Data 3 | E | PE_09 |
| SMC0_D04 | SMC0 Data 4 | E | PE_00 |
| SMC0_D05 | SMC0 Data 5 | D | PD_15 |
| SMC0_D06 | SMC0 Data 6 | D | PD_14 |
| SMC0_D07 | SMC0 Data 7 | D | PD_00 |
| SMC0_D08 | SMC0 Data 8 | B | PB_14 |
| SMC0_D09 | SMC0 Data 9 | B | PB_13 |
| SMC0_D10 | SMC0 Data 10 | B | PB_12 |
| SMC0_D11 | SMC0 Data 11 | B | PB_11 |
| SMC0_D12 | SMC0 Data 12 | B | PB_10 |
| SMC0_D13 | SMC0 Data 13 | B | PB_09 |
| SMC0_D14 | SMC0 Data 14 | B | PB_08 |
| SMC0_D15 | SMC0 Data 15 | B | PB_07 |
| SPI0_CLK | SPI0 Clock | C | PC_09 |
| SPI0_MISO | SPI0 Master In, Slave Out | C | PC_10 |
| SPI0_MOSI | SPI0 Master Out, Slave In | C | PC_11 |
| SPI0_RDY | SPI0 Ready | C | PC_12 |
| SPI0_SEL1 | SPI0 Slave Select Output 1 | C | PC_07 |
| SPI0_SEL2 | SPI0 Slave Select Output 2 | D | PD_01 |
| SPI0_SEL3 | SPI0 Slave Select Output 3 | C | PC_12 |
| SPI0_SEL4 | SPI0 Slave Select Output 4 | C | PC_00 |
| SPI0_SEL5 | SPI0 Slave Select Output 5 | E | PE_01 |
| SPI0_SEL6 | SPI0 Slave Select Output 6 | E | PE_02 |
| SPI0_SEL7 | SPI0 Slave Select Output 7 | E | PE_03 |
| SPI0_SS | SPI0 Slave Select Input | D | PD_01 |
| SPI1_CLK | SPI1 Clock | E | PE_13 |
| SPI1_MISO | SPI1 Master In, Slave Out | E | PE_14 |
| SPI1_MOSI | SPI1 Master Out, Slave In | E | PE_15 |
| SPI1_RDY | SPI1 Ready | E | PE_08 |
| SPI1_SEL1 | SPI1 Slave Select Output 1 | C | PC_13 |
| SPI1_SEL2 | SPI1 Slave Select Output 2 | E | PE_07 |
| SPI1_SEL3 | SPI1 Slave Select Output 3 | E | PE_11 |
| SPI1_SEL4 | SPI1 Slave Select Output 4 | E | PE_12 |
| SPI1_SEL5 | SPI1 Slave Select Output 5 | E | PE_08 |
| SPI1_SEL6 | SPI1 Slave Select Output 6 | F | PF_00 |
| SPI1_SEL7 | SPI1 Slave Select Output 7 | F | PF_01 |
| SPI1_SS | SPI1 Slave Select Input | E | PE_11 |
| SPI2_CLK | SPI2 Clock | C | PC_01 |
| SPI2_D2 | SPI2 Data 2 | C | PC_04 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| SPI2_D3 | SPI2 Data 3 | C | PC_05 |
| SPI2_MISO | SPI2 Master In, Slave Out | C | PC_02 |
| SPI2_MOSI | SPI2 Master Out, Slave In | C | PC_03 |
| SPI2_RDY | SPI2 Ready | E | PE_12 |
| SPI2_SEL1 | SPI2 Slave Select Output 1 | C | PC_06 |
| SPI2_SEL2 | SPI2 Slave Select Output 2 | E | PE_03 |
| SPI2_SEL3 | SPI2 Slave Select Output 3 | E | PE_04 |
| SPI2_SEL4 | SPI2 Slave Select Output 4 | E | PE_05 |
| SPI2_SEL5 | SPI2 Slave Select Output 5 | E | PE_06 |
| SPI2_SS | SPI2 Slave Select Input | C | PC_06 |
| SYS_BMODE0 | Boot Mode Control 0 | Not Muxed | SYS_BMODE0 |
| SYS_BMODE1 | Boot Mode Control 1 | Not Muxed | SYS_BMODE1 |
| SYS_BMODE2 | Boot Mode Control 2 | Not Muxed | SYS_BMODE2 |
| SYS_CLKIN0 | Clock/Crystal Input | Not Muxed | SYS_CLKIN0 |
| SYS_CLKIN1 | Clock/Crystal Input | Not Muxed | SYS_CLKIN1 |
| SYS_CLKOUT | Processor Clock Output | Not Muxed | SYS_CLKOUT |
| SYS_FAULT | Active High Fault Output | Not Muxed | SYS_FAULT |
| SYS_FAULT | Active Low Fault Output | Not Muxed | SYS_FAULT |
| SYS_HWRST | Processor Hardware Reset Control | Not Muxed | SYS_HWRST |
| SYS_RESOUT | Reset Output | Not Muxed | SYS_RESOUT |
| SYS_XTAL0 | Crystal Output | Not Muxed | SYS_XTAL0 |
| SYS_XTAL1 | Crystal Output | Not Muxed | SYS_XTAL1 |
| TM0_ACI0 | TIMER0 Alternate Capture Input 0 | C | PC_14 |
| TM0_ACI1 | TIMER0 Alternate Capture Input 1 | B | PB_03 |
| TM0_ACI2 | TIMER0 Alternate Capture Input 2 | D | PD_13 |
| TM0_ACI3 | TIMER0 Alternate Capture Input 3 | C | PC_07 |
| TM0_ACI4 | TIMER0 Alternate Capture Input 4 | B | PB_10 |
| TM0_ACLK1 | TIMER0 Alternate Clock 1 | D | PD_08 |
| TM0_ACLK2 | TIMER0 Alternate Clock 2 | D | PD_09 |
| TM0_ACLK3 | TIMER0 Alternate Clock 3 | B | PB_00 |
| TM0_ACLK4 | TIMER0 Alternate Clock 4 | B | PB_01 |
| TM0_CLK | TIMER0 Clock | C | PC_11 |
| TM0_TMR0 | TIMER0 Timer 0 | E | PE_09 |
| TM0_TMR1 | TIMER0 Timer 1 | B | PB_15 |
| TM0_TMR2 | TIMER0 Timer 2 | B | PB_10 |
| TM0_TMR3 | TIMER0 Timer 3 | B | PB_07 |
| TM0_TMR4 | TIMER0 Timer 4 | B | PB_08 |
| TM0_TMR5 | TIMER0 Timer 5 | B | PB_14 |
| TM0_TMR6 | TIMER0 Timer 6 | F | PF_00 |
| TM0_TMR7 | TIMER0 Timer 7 | F | PF_01 |
| TRACE0_CLK | TRACE0 Trace Clock (First Instance) | G | PG_00 |
| TRACE0_CLK | TRACE0 Trace Clock (Second Instance) | D | PD_10 |
| TRACE0_D00 | TRACE0 Trace Data (First Instance) | F | PF_13 |
| TRACE0_D00 | TRACE0 Trace Data 0 (Second Instance) | D | PD_02 |
| TRACE0_D01 | TRACE0 Trace Data 1 (First Instance) | D | PD_03 |
| TRACE0_D01 | TRACE0 Trace Data (Second Instance) | F | PF_14 |
| TRACE0_D02 | TRACE0 Trace Data (First Instance) | F | PF_15 |
| TRACE0_D02 | TRACE0 Trace Data 2 (Second Instance) | D | PD_04 |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| TRACE0_D03 | TRACE0 Trace Data (First Instance) | G | PG_01 |
| TRACE0_D03 | TRACE0 Trace Data 3 (Second Instance) | D | PD_05 |
| TRACE0_D04 | TRACE0 Trace Data (First Instance) | G | PG_02 |
| TRACE0_D04 | TRACE0 Trace Data 4 (Second Instance) | D | PD_06 |
| TRACE0_D05 | TRACE0 Trace Data 5 (First Instance) | D | PD_07 |
| TRACE0_D05 | TRACE0 Trace Data (Second Instance) | G | PG_03 |
| TRACE0_D06 | TRACE0 Trace Data (First Instance) | G | PG_04 |
| TRACE0_D06 | TRACE0 Trace Data 6 (Second Instance) | D | PD_08 |
| TRACE0_D07 | TRACE0 Trace Data (First Instance) | G | PG_05 |
| TRACE0_D07 | TRACE0 Trace Data 7 (Second Instance) | D | PD_09 |
| TRACE0_D08 | TRACE0 Trace Data 8 | F | PF_13 |
| TRACE0_D09 | TRACE0 Trace Data 9 | F | PF_14 |
| TRACE0_D10 | TRACE0 Trace Data 10 | F | PF_15 |
| TRACE0_D11 | TRACE0 Trace Data 11 | G | PG_01 |
| TRACE0_D12 | TRACE0 Trace Data 12 | G | PG_02 |
| TRACE0_D13 | TRACE0 Trace Data 13 | G | PG_03 |
| TRACE0_D14 | TRACE0 Trace Data 14 | G | PG_04 |
| TRACE0_D15 | TRACE0 Trace Data 15 | G | PG_05 |
| TWI0_SCL | TWI0 Serial Clock | Not Muxed | TWI0_SCL |
| TWI0_SDA | TWI0 Serial Data | Not Muxed | TWI0_SDA |
| TWI1_SCL | TWI1 Serial Clock | Not Muxed | TWI1_SCL |
| TWI1_SDA | TWI1 Serial Data | Not Muxed | TWI1_SDA |
| TWI2_SCL | TWI2 Serial Clock | Not Muxed | TWI2_SCL |
| TWI2_SDA | TWI2 Serial Data | Not Muxed | TWI2_SDA |
| UART0_CTS | UART0 Clear to Send | D | PD_00 |
| UART0_RTS | UART0 Request to Send | C | PC_15 |
| UART0_RX | UART0 Receive | C | PC_14 |
| UART0_TX | UART0 Transmit | C | PC_13 |
| UART1_CTS | UART1 Clear to Send | E | PE_01 |
| UART1_RTS | UART1 Request to Send | E | PE_02 |
| UART1_RX | UART1 Receive | B | PB_03 |
| UART1_TX | UART1 Transmit | B | PB_02 |
| UART2_CTS | UART2 Clear to Send | E | PE_11 |
| UART2_RTS | UART2 Request to Send | E | PE_10 |
| UART2_RX | UART2 Receive | D | PD_13 |
| UART2_TX | UART2 Transmit | D | PD_12 |
| USB0_CLKIN | USB0 Clock/Crystal Input | Not Muxed | USB_CLKIN |
| USB0_DM | USB0 Data − | Not Muxed | USB0_DM |
| USB0_DP | USB0 Data + | Not Muxed | USB0_DP |
| USB0_ID | USB0 OTG ID | Not Muxed | USB0_ID |
| USB0_VBC | USB0 VBUS Control | Not Muxed | USB0_VBC |
| USB0_VBUS | USB0 Bus Voltage | Not Muxed | USB0_VBUS |
| USB0_XTAL | USB0 Crystal | Not Muxed | USB_XTAL |
| USB1_DM | USB1 Data − | Not Muxed | USB1_DM |
| USB1_DP | USB1 Data + | Not Muxed | USB1_DP |
| USB1_VBUS | USB1 Bus Voltage | Not Muxed | USB1_VBUS |
| VDD_DMC | DMC VDD | Not Muxed | VDD_DMC |
| VDD_HADC | HADC/TMU VDD | Not Muxed | VDD_HADC |

| Signal Name | Description | Port | Pin Name |
| --- | --- | --- | --- |
| VDD_PCIE | PCIE Supply Voltage | Not Muxed | VDD_PCIE |
| VDD_PCIE_RX | PCIE RX Supply Voltage | Not Muxed | VDD_PCIE_RX |
| VDD_PCIE_TX | PCIE TX Supply Voltage | Not Muxed | VDD_PCIE_TX |
| VDD_RTC | RTC VDD | Not Muxed | VDD_RTC |
| VDD_USB | USB VDD | Not Muxed | VDD_USB |

GPIO MULTIPLEXING FOR THE 529-BALL CSP_BGA PACKAGE

Table 20 through Table 26 identify the pin functions that are multiplexed on the general-purpose I/O pins of the 529-ball CSP_BGA package.

##### Table 20.  Signal Multiplexing for Port A

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PA_00 | ETH0_TXD0 |  |  | SMC0_A21 |  |
| PA_01 | ETH0_TXD1 |  |  | SMC0_A20 |  |
| PA_02 | ETH0_MDC |  |  | SMC0_A24 |  |
| PA_03 | ETH0_MDIO |  |  | SMC0_A23 |  |
| PA_04 | ETH0_RXD0 |  |  | SMC0_A19 |  |
| PA_05 | ETH0_RXD1 |  |  | SMC0_A18 |  |
| PA_06 | ETH0_RXCLK_REFCLK |  |  | SMC0_A17 |  |
| PA_07 | ETH0_CRS |  |  | SMC0_A16 |  |
| PA_08 | ETH0_RXD2 |  |  | SMC0_A12 |  |
| PA_09 | ETH0_RXD3 |  |  | SMC0_A11 |  |
| PA_10 | ETH0_TXEN |  |  | SMC0_A22 |  |
| PA_11 | ETH0_TXCLK |  |  | SMC0_A15 |  |
| PA_12 | ETH0_TXD2 |  |  | SMC0_A14 |  |
| PA_13 | ETH0_TXD3 |  |  | SMC0_A13 |  |
| PA_14 | ETH0_PTPPPS3 | SINC0_D0 |  | SMC0_A10 |  |
| PA_15 | ETH0_PTPPPS2 | SINC0_D1 |  | SMC0_A09 |  |

Table 21.  Signal Multiplexing for Port B

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PB_00 PB_01 PB_02 PB_03 PB_04 PB_05 PB_06 PB_07 PB_08 PB_09 PB_10 PB_11 PB_12 PB_13 PB_14 PB_15 | ETH0_PTPPPS1 ETH0_PTPPPS0 ETH0_PTPCLKIN0 ETH0_PTPAUXIN0 MLB0_CLK MLB0_SIG MLB0_DAT LP1_D0 LP1_D1 LP1_D2 LP1_D3 LP1_D4 LP1_D5 LP1_D6 LP1_D7 LP1_ACK | SINC0_D2 SINC0_CLK0 UART1_TX UART1_RX SINC0_D3 PWM0_AH PWM0_AL TM0_TMR2 TM0_TMR5 PWM0_TRIP0 | PPI0_D14 PPI0_D15 PPI0_D16 PPI0_D17 PPI0_D12 PPI0_D13 PWM0_BH TM0_TMR3 TM0_TMR4 CAN1_TX CAN1_RX PWM0_DH PWM0_DL PWM0_CH PWM0_CL TM0_TMR1 | SMC0_A08 SMC0_A07 SMC0_A04 SMC0_A03 SMC0_ARDY SMC0_A01 SMC0_A02 SMC0_D15 SMC0_D14 SMC0_D13 SMC0_D12 SMC0_D11 SMC0_D10 SMC0_D09 SMC0_D08 SMC0_AWE | TM0_ACLK3 TM0_ACLK4 TM0_ACI1 ETH0_PTPAUXIN1 ETH0_PTPAUXIN2 ETH0_PTPAUXIN3 TM0_ACI4 CNT0_ZM CNT0_UD CNT0_DG |

Table 22.  Signal Multiplexing for Port C

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PC_00 PC_01 PC_02 PC_03 PC_04 PC_05 PC_06 PC_07 PC_08 PC_09 PC_10 PC_11 PC_12 PC_13 PC_14 PC_15 | LP1_CLK SPI2_CLK SPI2_MISO SPI2_MOSI SPI2_D2 SPI2_D3 SPI2_SEL1 CAN0_RX CAN0_TX SPI0_CLK SPI0_MISO SPI0_MOSI SPI0_SEL3 UART0_TX UART0_RX UART0_RTS | PWM0_BL SPI0_SEL1 SPI0_RDY SPI1_SEL1 PPI0_FS3 | SPI0_SEL4 ACM0_T0 ACM0_A0 ACM0_A1 ACM0_A2 | SMC0_ARE SMC0_AMS2 SMC0_AMS3 SMC0_A25 SMC0_AMS0 | SPI2_SS TM0_ACI3 TM0_CLK TM0_ACI0 |

Table 23. Signal Multiplexing for Port D

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PD_00 PD_01 PD_02 PD_03 PD_04 PD_05 PD_06 PD_07 PD_08 PD_09 PD_10 PD_11 PD_12 PD_13 PD_14 PD_15 | UART0_CTS SPI0_SEL2 LP0_D0 LP0_D1 LP0_D2 LP0_D3 LP0_D4 LP0_D5 LP0_D6 LP0_D7 LP0_CLK LP0_ACK UART2_TX UART2_RX PPI0_D11 PPI0_D10 | PPI0_D23 PWM1_TRIP0 PWM1_AH PWM1_AL PWM1_BH PWM1_BL PWM1_CH PWM1_CL PWM1_DH PWM1_DL PWM1_SYNC PWM2_TRIP0 PWM2_CH | ACM0_A3 ACM0_A4 TRACE0_D00 TRACE0_D01 TRACE0_D02 TRACE0_D03 TRACE0_D04 TRACE0_D05 TRACE0_D06 TRACE0_D07 TRACE0_CLK PPI0_D19 PPI0_D18 MLB0_CLKOUT | SMC0_D07 SMC0_AOE SMC0_A06 SMC0_A05 SMC0_D06 SMC0_D05 | SPI0_SS TM0_ACLK1 TM0_ACLK2 TM0_ACI2 |

Table 24.  Signal Multiplexing for Port E

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PE_00 PE_01 PE_02 | PPI0_D09 PPI0_FS2 PPI0_FS1 | PWM2_CL SPI0_SEL5 SPI0_SEL6 | UART1_CTS UART1_RTS | SMC0_D04 C1_FLG0 C2_FLG0 |  |

Table 24. Signal Multiplexing for Port E (Continued)

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PE_03 PE_04 PE_05 PE_06 PE_07 PE_08 PE_09 PE_10 PE_11 PE_12 PE_13 PE_14 PE_15 | PPI0_CLK PPI0_D08 PPI0_D07 PPI0_D06 PPI0_D05 PPI0_D04 PPI0_D03 PPI0_D02 PPI0_D01 PPI0_D00 SPI1_CLK SPI1_MISO SPI1_MOSI | SPI0_SEL7 PWM2_DH PWM2_SYNC SPI1_SEL5 PWM0_SYNC PWM2_DL SPI1_SEL3 SPI1_SEL4 | SPI2_SEL2 SPI2_SEL3 SPI2_SEL4 SPI2_SEL5 SPI1_SEL2 SPI1_RDY TM0_TMR0 UART2_RTS UART2_CTS SPI2_RDY PPI0_D20 PPI0_D21 PPI0_D22 | C1_FLG1 C2_FLG1 C1_FLG2 C2_FLG2 C1_FLG3 C2_FLG3 SMC0_D03 SMC0_D02 SMC0_D01 SMC0_D00 SMC0_AMS1 SMC0_ABE0 SMC0_ABE1 | SPI1_SS |

Table 25.  Signal Multiplexing for Port F

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PF_00 PF_01 PF_02 PF_03 PF_04 PF_05 PF_06 PF_07 PF_08 PF_09 PF_10 PF_11 PF_12 PF_13 PF_14 PF_15 | TM0_TMR6 TM0_TMR7 MSI0_D0 MSI0_D1 MSI0_D2 MSI0_D3 MSI0_D4 MSI0_D5 MSI0_D6 MSI0_D7 MSI0_CMD MSI0_CLK MSI0_CD ETH1_CRS ETH1_MDC ETH1_MDIO | SPI1_SEL6 SPI1_SEL7 HADC0_EOC_DOUT HADC0_MUX2 HADC0_MUX1 HADC0_MUX0 PWM2_AL PWM2_AH PWM2_BL PWM2_BH TRACE0_D08 TRACE0_D09 TRACE0_D10 | TRACE0_D00 TRACE0_D01 TRACE0_D02 | MSI0_INT |  |

Table 26.  Signal Multiplexing for Port G

| Signal Name | Multiplexed Function 0 | Multiplexed Function 1 | Multiplexed Function 2 | Multiplexed Function 3 | Multiplexed Function Input Tap |
| --- | --- | --- | --- | --- | --- |
| PG_00 | ETH1_REFCLK | TRACE0_CLK |  |  |  |
| PG_01 | ETH1_TXEN | TRACE0_D11 | TRACE0_D03 |  |  |
| PG_02 | ETH1_TXD0 | TRACE0_D12 | TRACE0_D04 |  |  |
| PG_03 | ETH1_TXD1 | TRACE0_D13 | TRACE0_D05 |  |  |
| PG_04 | ETH1_RXD0 | TRACE0_D14 | TRACE0_D06 |  |  |
| PG_05 | ETH1_RXD1 | TRACE0_D15 | TRACE0_D07 |  |  |

ADSP-SC58X/ADSP-2158X DESIGNER QUICK REFERENCE

Table 27 provides a quick reference summary of pin related information for circuit board design. The columns in this table provide the following information:

- The Signal Name column includes the signal name for every pin and the GPIO multiplexed pin function, where applicable.

- The Type column identifies the I/O type or supply type of the pin. The abbreviations used in this column are a (ana- log), s (supply), g (ground) and Input, Output, and InOut.

- The Driver Type column identifies the driver type used by the corresponding pin. The driver types are defined in the Output Drive Currents section of this data sheet.

- The Internal Term column specifies the termination pres- ent when the processor is not in the reset state.

- The Reset Term column specifies the termination present when the processor is in the reset state.

- The Reset Drive column specifies the active drive on the signal when the processor is in the reset state.

- The Power Domain column specifies the power supply domain in which the signal resides.

- The Description and Notes column identifies any special requirements or characteristics for a signal. These recom- mendations apply whether or not the hardware block associated with the signal is featured on the product. If no special requirements are listed, the signal can be left uncon- nected if it is not used. For multiplexed general-purpose I/O pins, this column identifies the functions available on the pin.

##### Table 27. ADSP-SC58x/ADSP-2158x Designer Quick Reference

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DAI0_PIN01 | InOut | A | PullDown | none | none | VDD_EXT | Desc: DAI0 Pin 1 Notes: No notes Desc: DAI0 Pin 2 Notes: No notes Desc: DAI0 Pin 3 Notes: No notes Desc: DAI0 Pin 4 Notes: No notes Desc: DAI0 Pin 5 Notes: No notes Desc: DAI0 Pin 6 Notes: No notes Desc: DAI0 Pin 7 Notes: No notes Desc: DAI0 Pin 8 Notes: No notes Desc: DAI0 Pin 9 Notes: No notes Desc: DAI0 Pin 10 Notes: No notes Desc: DAI0 Pin 11 Notes: No notes Desc: DAI0 Pin 12 Notes: No notes Desc: DAI0 Pin 13 Notes: No notes Desc: DAI0 Pin 14 Notes: No notes Desc: DAI0 Pin 15 Notes: No notes |
| DAI0_PIN02 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN03 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN04 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN05 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN06 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN07 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN08 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN09 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN10 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN11 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN12 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN13 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN14 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN15 | InOut | A | PullDown | none | none | VDD_EXT |  |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DAI0_PIN16 | InOut | A | PullDown | none | none | VDD_EXT | Desc: DAI0 Pin 16 Notes: No notes Desc: DAI0 Pin 17 Notes: No notes Desc: DAI0 Pin 18 Notes: No notes Desc: DAI0 Pin 19 Notes: No notes Desc: DAI0 Pin 20 Notes: No notes Desc: DAI1 Pin 1 Notes: No notes Desc: DAI1 Pin 2 Notes: No notes Desc: DAI1 Pin 3 Notes: No notes Desc: DAI1 Pin 4 Notes: No notes Desc: DAI1 Pin 5 Notes: No notes Desc: DAI1 Pin 6 Notes: No notes Desc: DAI1 Pin 7 Notes: No notes Desc: DAI1 Pin 8 Notes: No notes Desc: DAI1 Pin 9 Notes: No notes Desc: DAI1 Pin 10 Notes: No notes Desc: DAI1 Pin 11 Notes: No notes Desc: DAI1 Pin 12 Notes: No notes Desc: DAI1 Pin 13 Notes: No notes Desc: DAI1 Pin 14 Notes: No notes Desc: DAI1 Pin 15 Notes: No notes Desc: DAI1 Pin 16 Notes: No notes Desc: DAI1 Pin 17 Notes: No notes Desc: DAI1 Pin 18 Notes: No notes Desc: DAI1 Pin 19 Notes: No notes |
| DAI0_PIN17 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN18 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN19 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI0_PIN20 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN01 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN02 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN03 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN04 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN05 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN06 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN07 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN08 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN09 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN10 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN11 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN12 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN13 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN14 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN15 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN16 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN17 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN18 | InOut | A | PullDown | none | none | VDD_EXT |  |
| DAI1_PIN19 | InOut | A | PullDown | none | none | VDD_EXT |  |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DAI1_PIN20 | InOut | A | PullDown | none | none | VDD_EXT | Desc: DAI1 Pin 20 Notes: No notes Desc: DMC0 Address 0 Notes: No notes Desc: DMC0 Address 1 Notes: No notes Desc: DMC0 Address 2 Notes: No notes Desc: DMC0 Address 3 Notes: No notes Desc: DMC0 Address 4 Notes: No notes Desc: DMC0 Address 5 Notes: No notes Desc: DMC0 Address 6 Notes: No notes Desc: DMC0 Address 7 Notes: No notes Desc: DMC0 Address 8 Notes: No notes Desc: DMC0 Address 9 Notes: No notes Desc: DMC0 Address 10 Notes: No notes Desc: DMC0 Address 11 Notes: No notes Desc: DMC0 Address 12 Notes: No notes Desc: DMC0 Address 13 Notes: No notes Desc: DMC0 Address 14 Notes: No notes Desc: DMC0 Address 15 Notes: No notes Desc: DMC0 Bank Address Input 0 Notes: No notes Desc: DMC0 Bank Address Input 1 Notes: No notes Desc: DMC0 Bank Address Input 2 Notes: No notes Desc: DMC0 Column Address Strobe Notes: No notes Desc: DMC0 Clock Notes: No notes Desc: DMC0 Clock Enable Notes: No notes Desc: DMC0 Clock (Complement) Notes: No notes |
| DMC0_A00 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A01 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A02 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A03 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A04 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A05 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A06 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A07 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A08 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A09 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A10 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A11 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A12 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A13 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A14 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_A15 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_BA0 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_BA1 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_BA2 | Output | B | none | none | none | VDD_DMC |  |
| DMC0_CAS | Output | B | none | none | none | VDD_DMC |  |
| DMC0_CK | Output | C | none | none | L | VDD_DMC |  |
| DMC0_CKE | Output | B | none | none | L | VDD_DMC |  |
| DMC0_CK | Output | C | none | none | L | VDD_DMC |  |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMC0_CS0 | Output | B | none | none | none | VDD_DMC | Desc: DMC0 Chip Select 0 |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC0_DQ00 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 0 Notes: No notes |
| DMC0_DQ01 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 1 Notes: No notes |
| DMC0_DQ02 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 2 Notes: No notes |
| DMC0_DQ03 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 3 Notes: No notes |
| DMC0_DQ04 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 4 Notes: No notes |
| DMC0_DQ05 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 5 Notes: No notes |
| DMC0_DQ06 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 6 Notes: No notes |
| DMC0_DQ07 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 7 Notes: No notes |
| DMC0_DQ08 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 8 Notes: No notes |
| DMC0_DQ09 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 9 Notes: No notes |
| DMC0_DQ10 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 10 Notes: No notes |
| DMC0_DQ11 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 11 Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMC0_DQ12 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 12 Notes: No notes |
| DMC0_DQ13 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 13 Notes: No notes |
| DMC0_DQ14 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 14 Notes: No notes |
| DMC0_DQ15 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data 15 Notes: No notes |
| DMC0_LDM | Output | B | none | none | none | VDD_DMC | Desc: DMC0 Data Mask for Lower Byte |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC0_LDQS | InOut | C | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data Strobe for Lower Byte (Complement) Notes: No notes |
| DMC0_LDQS | InOut | C | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data Strobe for Lower Byte Notes: External weak pull-down required in LPDDR mode |
| DMC0_ODT | Output | B | none | none | none | VDD_DMC | Desc: DMC0 On-Die Termination |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC0_RAS | Output | B | none | none | none | VDD_DMC | Desc: DMC0 Row Address Strobe |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC0_RESET | Output | B | none | none | none | VDD_DMC | Desc: DMC0 Reset (DDR3 Only) |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC0_RZQ | a | B | none | none | none | VDD_DMC | Desc: DMC0 External Calibration Resistor Connection |
|  |  |  |  |  |  |  | Notes: Applicable for DDR2 and DDR3 only. External pull-down of 34 Ω need to be added. |
| DMC0_UDM | Output | B | none | none | none | VDD_DMC | Desc: DMC0 Data Mask for Upper Byte |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC0_UDQS | InOut | C | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data Strobe for Upper Byte Notes: External weak pull-down required in LPDDR mode |
| DMC0_UDQS | InOut | C | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC0 Data Strobe for Upper Byte (Complement) Notes: No notes |
| DMC0_VREF | a |  | none | none | none | VDD_DMC | Desc: DMC0 Voltage Reference |
|  |  |  |  |  |  |  | Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMC0_WE | Output | B | none | none | none | VDD_DMC | Desc: DMC0 Write Enable Notes: No notes Desc: DMC1 Address 0 Notes: No notes Desc: DMC1 Address 1 Notes: No notes Desc: DMC1 Address 2 Notes: No notes Desc: DMC1 Address 3 Notes: No notes Desc: DMC1 Address 4 Notes: No notes Desc: DMC1 Address 5 Notes: No notes Desc: DMC1 Address 6 Notes: No notes Desc: DMC1 Address 7 Notes: No notes Desc: DMC1 Address 8 Notes: No notes Desc: DMC1 Address 9 Notes: No notes Desc: DMC1 Address 10 Notes: No notes Desc: DMC1 Address 11 Notes: No notes Desc: DMC1 Address 12 Notes: No notes Desc: DMC1 Address 13 Notes: No notes Desc: DMC1 Address 14 Notes: No notes Desc: DMC1 Address 15 Notes: No notes Desc: DMC1 Bank Address Input 0 Notes: No notes Desc: DMC1 Bank Address Input 1 Notes: No notes Desc: DMC1 Bank Address Input 2 Notes: No notes Desc: DMC1 Column Address Strobe Notes: No notes Desc: DMC1 Clock Notes: No notes Desc: DMC1 Clock Enable Notes: No notes Desc: DMC1 Clock (Complement) Notes: No notes |
| DMC1_A00 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A01 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A02 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A03 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A04 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A05 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A06 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A07 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A08 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A09 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A10 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A11 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A12 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A13 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A14 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_A15 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_BA0 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_BA1 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_BA2 | Output | B | none | none | none | VDD_DMC |  |
| DMC1_CAS | Output | B | none | none | none | VDD_DMC |  |
| DMC1_CK | Output | C | none | none | L | VDD_DMC |  |
| DMC1_CKE | Output | B | none | none | L | VDD_DMC |  |
| DMC1_CK | Output | C | none | none | L | VDD_DMC |  |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMC1_CS0 | Output | B | none | none | none | VDD_DMC | Desc: DMC1 Chip Select 0 |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC1_DQ00 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 0 Notes: No notes |
| DMC1_DQ01 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 1 Notes: No notes |
| DMC1_DQ02 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 2 Notes: No notes |
| DMC1_DQ03 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 3 Notes: No notes |
| DMC1_DQ04 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 4 Notes: No notes |
| DMC1_DQ05 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 5 Notes: No notes |
| DMC1_DQ06 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 6 Notes: No notes |
| DMC1_DQ07 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 7 Notes: No notes |
| DMC1_DQ08 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 8 Notes: No notes |
| DMC1_DQ09 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 9 Notes: No notes |
| DMC1_DQ10 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 10 Notes: No notes |
| DMC1_DQ11 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 11 Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMC1_DQ12 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 12 Notes: No notes |
| DMC1_DQ13 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 13 Notes: No notes |
| DMC1_DQ14 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 14 Notes: No notes |
| DMC1_DQ15 | InOut | B | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data 15 Notes: No notes |
| DMC1_LDM | Output | B | none | none | none | VDD_DMC | Desc: DMC1 Data Mask for Lower Byte |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC1_LDQS | InOut | C | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data Strobe for Lower Byte Notes: External weak pull-down required in LPDDR mode |
| DMC1_LDQS | InOut | C | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data Strobe for Lower Byte (Complement) Notes: No notes |
| DMC1_ODT | Output | B | none | none | none | VDD_DMC | Desc: DMC1 On-Die Termination |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC1_RAS | Output | B | none | none | none | VDD_DMC | Desc: DMC1 Row Address Strobe |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC1_RESET | InOut | B | none | none | none | VDD_DMC | Desc: DMC1 Reset (DDR3 Only) |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC1_RZQ | a | B | none | none | none | VDD_DMC | Desc: DMC1 External Calibration Resistor Connection |
|  |  |  |  |  |  |  | Notes: Applicable for DDR2 and DDR3 only. External pull-down of 34 Ω need to be added. |
| DMC1_UDM | Output | B | none | none | none | VDD_DMC | Desc: DMC1 Data Mask for Upper Byte |
|  |  |  |  |  |  |  | Notes: No notes |
| DMC1_UDQS | InOut | C | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data Strobe for Upper Byte Notes: External weak pull-down required in LPDDR mode |
| DMC1_UDQS | InOut | C | Internal logic ensures that input signal does not float | none | none | VDD_DMC | Desc: DMC1 Data Strobe for Upper Byte (Complement) Notes: No notes |
| DMC1_VREF | a |  | none | none | none | VDD_DMC | Desc: DMC1 Voltage Reference |
|  |  |  |  |  |  |  | Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMC1_WE | Output | B | none | none | none |  | Desc: DMC1 Write Enable |
|  |  |  |  |  |  |  | Notes: No notes |
| GND | g | NA | none | none | none |  | Desc: Ground |
|  |  |  |  |  |  |  | Notes: No notes |
| HADC0_VIN0 | a | NA | none | none | none | VDD_HADC | Desc: HADC0 Analog Input at Channel 0 |
|  |  |  |  |  |  |  | Notes: If Input not used connect to GND |
| HADC0_VIN1 | a | NA | none | none | none | VDD_HADC | Desc: HADC0 Analog Input at Channel 1 |
|  |  |  |  |  |  |  | Notes: If Input not used connect to GND |
| HADC0_VIN2 | a | NA | none | none | none | VDD_HADC | Desc: HADC0 Analog Input at Channel 2 |
|  |  |  |  |  |  |  | Notes: If Input not used connect to GND |
| HADC0_VIN3 | a | NA | none | none | none | VDD_HADC | Desc: HADC0 Analog Input at Channel 3 |
|  |  |  |  |  |  |  | Notes: If Input not used connect to GND |
| HADC0_VIN4 | a | NA | none | none | none | VDD_HADC | Desc: HADC0 Analog Input at Channel 4 |
|  |  |  |  |  |  |  | Notes: If Input not used connect to GND |
| HADC0_VIN5 | a | NA | none | none | none | VDD_HADC | Desc: HADC0 Analog Input at Channel 5 |
|  |  |  |  |  |  |  | Notes: If Input not used connect to GND |
| HADC0_VIN6 | a | NA | none | none | none | VDD_HADC | Desc: HADC0 Analog Input at Channel 6 |
|  |  |  |  |  |  |  | Notes: If Input not used connect to GND |
| HADC0_VIN7 | a | NA | none | none | none | VDD_HADC | Desc: HADC0 Analog Input at Channel 7 |
|  |  |  |  |  |  |  | Notes: If Input not used connect to GND |
| HADC0_VREFN | s | NA | none | none | none | VDD_HADC | Desc: HADC0 Ground Reference for ADC |
|  |  |  |  |  |  |  | Notes: Can be left floating if HADC and TMU are not used |
| HADC0_VREFP | s | NA | none | none | none | VDD_HADC | Desc: HADC0 External Reference for ADC |
|  |  |  |  |  |  |  | Notes: Can be left floating if HADC and TMU are not used |
| JTG_TCK | Input |  | PullUp | none | none | VDD_EXT | Desc: JTAG Clock |
|  |  |  |  |  |  |  | Notes: No notes |
| JTG_TDI | Input |  | PullUp | none | none | VDD_EXT | Desc: JTAG Serial Data In |
|  |  |  |  |  |  |  | Notes: No notes |
| JTG_TDO | Output | A | none | none | none | VDD_EXT | Desc: JTAG Serial Data Out |
|  |  |  |  |  |  |  | Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JTG_TMS | InOut | A | PullUp | none | none | VDD_EXT | Desc: JTAG Mode Select |
|  |  |  |  |  |  |  | Notes: No notes |
| JTG_TRST | Input |  | PullDown | none | none | VDD_EXT | Desc: JTAG Reset |
|  |  |  |  |  |  |  | Notes: No notes |
| MLB0_CLKN | Input | NA | Internal logic ensures that input signal does not float | none | none | VDD_EXT | Desc: MLB0 Differential Clock (−) Notes: No notes |
| MLB0_CLKP | Input | NA | Internal logic ensures that input signal does not float | none | none | VDD_EXT | Desc: MLB0 Differential Clock (+) Notes: No notes |
| MLB0_DATN | InOut | I | Internal logic ensures that input signal does not float | none | none | VDD_EXT | Desc: MLB0 Differential Data (−) Notes: No notes |
| MLB0_DATP | InOut | I | Internal logic ensures that input signal does not float | none | none | VDD_EXT | Desc: MLB0 Differential Data (+) Notes: No notes |
| MLB0_SIGN | InOut | I | Internal logic ensures that input signal does not float | none | none | VDD_EXT | Desc: MLB0 Differential Signal (−) Notes: No notes |
| MLB0_SIGP | InOut | I | Internal logic ensures that input signal does not float | none | none | VDD_EXT | Desc: MLB0 Differential Signal (+) Notes: No notes |
| PA_00 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 0 ¦ EMAC0 Transmit Data 0 ¦ SMC0 Address 21 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_01 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 1 ¦ EMAC0 Transmit Data 1 ¦ SMC0 Address 20 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_02 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 2 ¦ EMAC0 Management Channel Clock ¦ SMC0 Address 24 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_03 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 3 ¦ EMAC0 Management Channel Serial Data ¦ SMC0 Address 23 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_04 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 4 ¦ EMAC0 Receive Data 0 ¦ SMC0 Address 19 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_05 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 5 ¦ EMAC0 Receive Data 1 ¦ SMC0 Address 18 |
|  |  |  |  |  |  |  | Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PA_06 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 6 ¦ EMAC0 RXCLK (10/100/1000) or REFCLK (10/100) ¦ SMC0 Address 17 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_07 | InOut | A | PullDown | none | none | VDD_EXT | Desc: EMAC0 RXCTL (10/100/1000) or CRS (10/100) ¦ PORTA Position 7 ¦ EMAC0 Carrier Sense/RMII Receive Data Valid ¦ SMC0 Address 16 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_08 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 8 ¦ EMAC0 Receive Data 2 ¦ SMC0 Address 12 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_09 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 9 ¦ EMAC0 Receive Data 3 ¦ SMC0 Address 11 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_10 | InOut | A | PullDown | none | none | VDD_EXT | Desc: EMAC0 TXCTL (10/100/1000) or TXEN (10/100) ¦ PORTA Position 10 ¦ EMAC0 Transmit Enable ¦ SMC0 Address 22 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_11 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 11 ¦ EMAC0 Transmit Clock ¦ SMC0 Address 15 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_12 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 12 ¦ EMAC0 Transmit Data 2 ¦ SMC0 Address 14 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_13 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 13 ¦ EMAC0 Transmit Data 3 ¦ SMC0 Address 13 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_14 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 14 ¦ EMAC0 PTP Pulse-Per-Second Output 3 ¦ SINC0 Data 0 ¦ SMC0 Address 10 |
|  |  |  |  |  |  |  | Notes: No notes |
| PA_15 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTA Position 15 ¦ EMAC0 PTP Pulse-Per-Second Output 2 ¦ SINC0 Data 1 ¦ SMC0 Address 9 |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_00 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 0 ¦ EMAC0 PTP Pulse-Per-Second Output 1 ¦ EPPI0 Data 14 ¦ SINC0 Data 2 ¦ SMC0 Address 8 ¦ TIMER0 Alternate Clock 3 |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_01 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 1 ¦ EMAC0 PTP Pulse-Per-Second Output 0 ¦ EPPI0 Data 15 ¦ SINC0 Clock 0 ¦ SMC0 Address 7 ¦ TIMER0 Alternate Clock 4 |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_02 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 2 ¦ EMAC0 PTP Clock Input 0 ¦ EPPI0 Data 16 ¦ SMC0 Address 4 ¦ UART1 Transmit |
|  |  |  |  |  |  |  | Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB_03 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 3 ¦ EMAC0 PTP Auxiliary Trigger Input 0 ¦ EPPI0 Data 17 ¦ SMC0 Address 3 ¦ UART1 Receive ¦ TIMER0 Alternate Capture Input 1 |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_04 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 4 ¦ EPPI0 Data 12 ¦ MLB0 Single-Ended Clock ¦ SINC0 Data 3 ¦ SMC0 Asynchronous Ready ¦ EMAC0 PTP Auxiliary Trigger Input 1 |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_05 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 5 ¦ EPPI0 Data 13 ¦ MLB0 Single-Ended Signal ¦ SMC0 Address 1 ¦ EMAC0 PTP Auxiliary Trigger Input 2 |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_06 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 6 ¦ MLB0 Single-Ended Data ¦ PWM0 Channel B High Side ¦ SMC0 Address 2 ¦ EMAC0 PTP Auxiliary Trigger Input 3 |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_07 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 7 ¦ LP1 Data 0 ¦ PWM0 Channel A High Side ¦ SMC0 Data 15 ¦ TIMER0 Timer 3 |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_08 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 8 ¦ LP1 Data 1 ¦ PWM0 Channel A Low Side ¦ SMC0 Data 14 ¦ TIMER0 Timer 4 |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_09 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 9 ¦ CAN1 Transmit ¦ LP1 Data 2 ¦ SMC0 Data 13 |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_10 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 10 ¦ CAN1 Receive ¦ LP1 Data 3 ¦ SMC0 Data 12 ¦ TIMER0 Timer 2 ¦ TIMER0 Alternate Capture Input 4 |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_11 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 11 ¦ LP1 Data 4 ¦ PWM0 Channel D High Side ¦ SMC0 Data 11 ¦ CNT0 Count Zero Marker |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_12 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 12 ¦ LP1 Data 5 ¦ PWM0 Channel D Low Side ¦ SMC0 Data 10 ¦ CNT0 Count Up and Direction |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_13 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 13 ¦ LP1 Data 6 ¦ PWM0 Channel C High Side ¦ SMC0 Data 9 |
|  |  |  |  |  |  |  | Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PB_14 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 14 ¦ LP1 Data 7 ¦ PWM0 Channel C Low Side ¦ SMC0 Data 8 ¦ TIMER0 Timer 5 ¦ CNT0 Count Down and Gate |
|  |  |  |  |  |  |  | Notes: No notes |
| PB_15 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTB Position 15 ¦ LP1 Acknowledge ¦ PWM0 Shutdown Input 0 ¦ SMC0 Write Enable ¦ TIMER0 Timer 1 |
|  |  |  |  |  |  |  | Notes: No notes |
| PCIE0_CLKM | Input | NA | PullDown | none | none | VDD_PCIE | Desc: PCIE0 CLK – |
|  |  |  |  |  |  |  | Notes: No notes |
| PCIE0_CLKP | Input | NA | PullDown | none | none | VDD_PCIE | Desc: PCIE0 CLK + |
|  |  |  |  |  |  |  | Notes: No notes |
| PCIE0_REF | a | NA | PullDown | none | none | VDD_PCIE | Desc: PCIE0 Reference |
|  |  |  |  |  |  |  | Notes: No notes |
| PCIE0_RXM | Input | NA | PullDown | none | none | VDD_PCIE_RX | Desc: PCIE0 RX – |
|  |  |  |  |  |  |  | Notes: No notes |
| PCIE0_RXP | Input | NA | PullDown | none | none | VDD_PCIE_RX | Desc: PCIE0 RX + |
|  |  |  |  |  |  |  | Notes: No notes |
| PCIE0_TXM | InOut | J | PullDown | none | none | VDD_PCIE_TX | Desc: PCIE0 TX – |
|  |  |  |  |  |  |  | Notes: No notes |
| PCIE0_TXP | InOut | J | PullDown | none | none | VDD_PCIE_TX | Desc: PCIE0 TX + |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_00 | InOut | H | PullDown | none | none | VDD_EXT | Desc: PORTC Position 0 ¦ LP1 Clock ¦ PWM0 Channel B Low Side ¦ SMC0 Read Enable ¦ SPI0 Slave Select Output 4 |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_01 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 1 ¦ SPI2 Clock |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_02 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 2 ¦ SPI2 Master In, Slave Out |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_03 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 3 ¦ SPI2 Master Out, Slave In |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_04 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 4 ¦ SPI2 Data 2 |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_05 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 5 ¦ SPI2 Data 3 |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_06 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 6 ¦ SPI2 Slave Select Output 1 ¦ SPI2 Slave Select Input |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_07 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 7 ¦ CAN0 Receive ¦ SMC0 Memory Select 2 ¦ SPI0 Slave Select Output 1 ¦ TIMER0 Alternate Capture Input 3 |
|  |  |  |  |  |  |  | Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PC_08 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 8 ¦ CAN0 Transmit ¦ SMC0 Memory Select 3 |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_09 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 9 ¦ SPI0 Clock |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_10 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 10 ¦ SPI0 Master In, Slave Out |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_11 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 11 ¦ SPI0 Master Out, Slave In ¦ TIMER0 Clock |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_12 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 12 ¦ ACM0 External Trigger n ¦ SMC0 Address 25 ¦ SPI0 Ready ¦ SPI0 Slave Select Output 3 |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_13 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 13 ¦ ACM0 ADC Control Signals ¦ SPI1 Slave Select Output 1 ¦ UART0 Transmit |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_14 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 14 ¦ ACM0 ADC Control Signals ¦ UART0 Receive ¦ TIMER0 Alternate Capture Input 0 |
|  |  |  |  |  |  |  | Notes: No notes |
| PC_15 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTC Position 15 ¦ ACM0 ADC Control Signals ¦ EPPI0 Frame Sync 3 (FIELD) ¦ SMC0 Memory Select 0 ¦ UART0 Request to Send |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_00 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 0 ¦ ACM0 ADC Control Signals ¦ EPPI0 Data 23 ¦ SMC0 Data 7 ¦ UART0 Clear to Send |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_01 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 1 ¦ ACM0 ADC Control Signals ¦ SMC0 Output Enable ¦ SPI0 Slave Select Output 2 ¦ SPI0 Slave Select Input |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_02 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 2 ¦ LP0 Data 0 ¦ PWM1 Shutdown Input 0 ¦ TRACE0 Trace Data 0 |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_03 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 3 ¦ LP0 Data 1 ¦ PWM1 Channel A High Side ¦ TRACE0 Trace Data 1 |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_04 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 4 ¦ LP0 Data 2 ¦ PWM1 Channel A Low Side ¦ TRACE0 Trace Data 2 |
|  |  |  |  |  |  |  | Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PD_05 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 5 ¦ LP0 Data 3 ¦ PWM1 Channel B High Side ¦ TRACE0 Trace Data 3 |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_06 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 6 ¦ LP0 Data 4 ¦ PWM1 Channel B Low Side ¦ TRACE0 Trace Data 4 |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_07 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 7 ¦ LP0 Data 5 ¦ PWM1 Channel C High Side ¦ TRACE0 Trace Data 5 |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_08 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 8 ¦ LP0 Data 6 ¦ PWM1 Channel C Low Side ¦ TRACE0 Trace Data 6 ¦ TIMER0 Alternate Clock 1 |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_09 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 9 ¦ LP0 Data 7 ¦ PWM1 Channel D High Side ¦ TRACE0 Trace Data 7 ¦ TIMER0 Alternate Clock 2 |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_10 | InOut | H | PullDown | none | none | VDD_EXT | Desc: PORTD Position 10 ¦ LP0 Clock ¦ PWM1 Channel D Low Side ¦ TRACE0 Trace Clock |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_11 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 11 ¦ LP0 Acknowledge ¦ PWM1 PWMTMR Grouped |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_12 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 12 ¦ EPPI0 Data 19 ¦ SMC0 Address 6 ¦ UART2 Transmit |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_13 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 13 ¦ EPPI0 Data 18 ¦ SMC0 Address 5 ¦ UART2 Receive ¦ TIMER0 Alternate Capture Input 2 |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_14 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 14 ¦ EPPI0 Data 11 ¦ MLB0 Single-Ended Clock Out ¦ PWM2 Shutdown Input 0 ¦ SMC0 Data 6 |
|  |  |  |  |  |  |  | Notes: No notes |
| PD_15 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTD Position 15 ¦ EPPI0 Data 10 ¦ PWM2 Channel C High Side ¦ SMC0 Data 5 |
|  |  |  |  |  |  |  | Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PE_00 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 0 ¦ EPPI0 Data 9 ¦ PWM2 Channel C Low Side ¦ SMC0 Data 4 |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_01 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 1 ¦ EPPI0 Frame Sync 2 (VSYNC) ¦ SPI0 Slave Select Output 5 ¦ SHARC Core 1 Flag Pin ¦ UART1 Clear to Send |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_02 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 2 ¦ EPPI0 Frame Sync 1 (HSYNC) ¦ SPI0 Slave Select Output 6 ¦ SHARC Core 2 Flag Pin ¦ UART1 Request to Send |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_03 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 3 ¦ EPPI0 Clock ¦ SPI0 Slave Select Output 7 ¦ SPI2 Slave Select Output 2 ¦ SHARC Core 1 Flag Pin |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_04 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 4 ¦ EPPI0 Data 8 ¦ PWM2 Channel D High Side ¦ SPI2 Slave Select Output 3 ¦ SHARC Core 2 Flag Pin |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_05 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 5 ¦ EPPI0 Data 7 ¦ PWM2 PWMTMR Grouped ¦ SPI2 Slave Select Output 4 ¦ SHARC Core 1 Flag Pin |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_06 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 6 ¦ EPPI0 Data 6 ¦ SPI2 Slave Select Output 5 ¦ SHARC Core 2 Flag Pin |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_07 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 7 ¦ EPPI0 Data 5 ¦ SPI1 Slave Select Output 2 ¦ SHARC Core 1 Flag Pin |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_08 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 8 ¦ EPPI0 Data 4 ¦ SPI1 Ready ¦ SPI1 Slave Select Output 5 ¦ SHARC Core 2 Flag Pin |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_09 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 9 ¦ EPPI0 Data 3 ¦ PWM0 PWMTMR Grouped ¦ SMC0 Data 3 ¦ TIMER0 Timer 0 |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_10 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 10 ¦ EPPI0 Data 2 ¦ PWM2 Channel D Low Side ¦ SMC0 Data 2 ¦ UART2 Request to Send |
|  |  |  |  |  |  |  | Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PE_11 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 11 ¦ EPPI0 Data 1 ¦ SMC0 Data 1 ¦ SPI1 Slave Select Output 3 ¦ UART2 Clear to Send ¦ SPI1 Slave Select Input |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_12 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 12 ¦ EPPI0 Data 0 ¦ SMC0 Data 0 ¦ SPI1 Slave Select Output 4 ¦ SPI2 Ready |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_13 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 13 ¦ EPPI0 Data 20 ¦ SMC0 Memory Select 1 ¦ SPI1 Clock |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_14 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 14 ¦ EPPI0 Data 21 ¦ SMC0 Byte Enable 0 ¦ SPI1 Master In, Slave Out |
|  |  |  |  |  |  |  | Notes: No notes |
| PE_15 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTE Position 15 ¦ EPPI0 Data 22 ¦ SMC0 Byte Enable 1 ¦ SPI1 Master Out, Slave In |
|  |  |  |  |  |  |  | Notes: No notes |
| PF_00 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTF Position 0 ¦ SPI1 Slave Select Output 6 ¦ TIMER0 Timer 6 |
|  |  |  |  |  |  |  | Notes: No notes |
| PF_01 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTF Position 1 ¦ SPI1 Slave Select Output 7 ¦ TIMER0 Timer 7 |
|  |  |  |  |  |  |  | Notes: No notes |
| PF_02 | InOut | A | PullDown/ Programmable PullUp | none | none | VDD_EXT | Desc: PORTF Position 2 ¦ HADC0 End of Conversion/Serial Data Out ¦ MSI0 Data 0 |
|  |  |  |  |  |  |  | Notes: No notes |
| PF_03 | InOut | A | PullDown/ Programmable PullUp | none | none | VDD_EXT | Desc: PORTF Position 3 ¦ HADC0 Controls to External Multiplexer ¦ MSI0 Data 1 |
|  |  |  |  |  |  |  | Notes: No notes |
| PF_04 | InOut | A | PullDown/ Programmable PullUp | none | none | VDD_EXT | Desc: PORTF Position 4 ¦ HADC0 Controls to External Multiplexer ¦ MSI0 Data 2 |
|  |  |  |  |  |  |  | Notes: No notes |
| PF_05 | InOut | A | PullDown/ Programmable PullUp | none | none | VDD_EXT | Desc: PORTF Position 5 ¦ HADC0 Controls to External Multiplexer ¦ MSI0 Data 3 |
|  |  |  |  |  |  |  | Notes: No notes |
| PF_06 | InOut | A | PullDown/ Programmable PullUp | none | none | VDD_EXT | Desc: PORTF Position 6 ¦ MSI0 Data 4 ¦ PWM2 Channel A Low Side Notes: No notes |
| PF_07 | InOut | A | PullDown/ Programmable PullUp | none | none | VDD_EXT | Desc: PORTF Position 7 ¦ MSI0 Data 5 ¦ PWM2 Channel A High Side Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PF_08 | InOut | A | PullDown/ Programmable PullUp | none | none | VDD_EXT | Desc: PORTF Position 8 ¦ MSI0 Data 6 ¦ PWM2 Channel B Low Side Notes: No notes |
| PF_09 | InOut | A | PullDown/ Programmable PullUp | none | none | VDD_EXT | Desc: PORTF Position 9 ¦ MSI0 Data 7 ¦ PWM2 Channel B High Side Notes: No notes |
| PF_10 | InOut | A | PullDown/ Programmable PullUp | none | none | VDD_EXT | Desc: PORTF Position 10 ¦ MSI0 Command Notes: No notes |
| PF_11 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTF Position 11 ¦ MSI0 Clock |
|  |  |  |  |  |  |  | Notes: No notes |
| PF_12 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTF Position 12 ¦ MSI0 Card Detect |
|  |  |  |  |  |  |  | Notes: No notes |
| PF_13 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTF Position 13 ¦ EMAC1 Carrier Sense/RMII Receive Data Valid ¦ MSI0 eSDIO Interrupt Input ¦ TRACE0 Trace Data ¦ TRACE0 Trace Data 8 |
|  |  |  |  |  |  |  | Notes: No notes |
| PF_14 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTF Position 14 ¦ EMAC1 Management Channel Clock ¦ TRACE0 Trace Data ¦ TRACE0 Trace Data 9 |
|  |  |  |  |  |  |  | Notes: No notes |
| PF_15 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTF Position 15 ¦ EMAC1 Management Channel Serial Data ¦ TRACE0 Trace Data ¦ TRACE0 Trace Data 10 |
|  |  |  |  |  |  |  | Notes: No notes |
| PG_00 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTG Position 0 ¦ EMAC1 Reference Clock ¦ TRACE0 Trace Clock |
|  |  |  |  |  |  |  | Notes: No notes |
| PG_01 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTG Position 1 ¦ EMAC1 Transmit Enable ¦ TRACE0 Trace Data ¦ TRACE0 Trace Data 11 |
|  |  |  |  |  |  |  | Notes: No notes |
| PG_02 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTG Position 2 ¦ EMAC1 Transmit Data 0 ¦ TRACE0 Trace Data ¦ TRACE0 Trace Data 12 |
|  |  |  |  |  |  |  | Notes: No notes |
| PG_03 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTG Position 3 ¦ EMAC1 Transmit Data 1 ¦ TRACE0 Trace Data ¦ TRACE0 Trace Data 13 |
|  |  |  |  |  |  |  | Notes: No notes |
| PG_04 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTG Position 4 ¦ EMAC1 Receive Data 0 ¦ TRACE0 Trace Data ¦ TRACE0 Trace Data 14 |
|  |  |  |  |  |  |  | Notes: No notes |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PG_05 | InOut | A | PullDown | none | none | VDD_EXT | Desc: PORTG Position 5 ¦ EMAC1 Receive Data 1 ¦ TRACE0 Trace Data ¦ TRACE0 Trace Data 15 |
|  |  |  |  |  |  |  | Notes: No notes |
| RTC0_CLKIN | a | NA | none | none | none | VDD_RTC | Desc: RTC0 Crystal Input/External Oscillator Connection |
|  |  |  |  |  |  |  | Notes: Connect to GND if not used |
| RTC0_XTAL | a | NA | none | none | none | VDD_RTC | Desc: RTC0 Crystal output |
|  |  |  |  |  |  |  | Notes: No notes |
| SYS_BMODE0 | Input | NA | PullDown | none | none | VDD_EXT | Desc: Boot Mode Control n |
|  |  |  |  |  |  |  | Notes: No notes |
| SYS_BMODE1 | Input | NA | PullDown | none | none | VDD_EXT | Desc: Boot Mode Control n |
|  |  |  |  |  |  |  | Notes: No notes |
| SYS_BMODE2 | Input | NA | PullDown | none | none | VDD_EXT | Desc: Boot Mode Control n |
|  |  |  |  |  |  |  | Notes: No notes |
| SYS_CLKIN0 | a | NA | none | none | none | VDD_EXT | Desc: Clock/Crystal Input |
|  |  |  |  |  |  |  | Notes: No notes |
| SYS_CLKIN1 | a | NA | none | none | none | VDD_EXT | Desc: Clock/Crystal Input |
|  |  |  |  |  |  |  | Notes: Connect to GND if not used |
| SYS_CLKOUT | a | A | none | none | none |  | Desc: Processor Clock Output |
|  |  |  |  |  |  |  | Notes: No notes |
| SYS_FAULT | InOut | A | none | none | none |  | Desc: Active-High Fault Output |
|  |  |  |  |  |  |  | Notes: External pull-down required to keep signal in de-asserted state |
| SYS_FAULT | InOut | A | none | none | none |  | Desc: Active-Low Fault Output |
|  |  |  |  |  |  |  | Notes: External pull-up required to keep signal in de-asserted state |
| SYS_HWRST | Input | NA | none | none | none | VDD_EXT | Desc: Processor Hardware Reset Control |
|  |  |  |  |  |  |  | Notes: No notes |
| SYS_RESOUT | Output | A | none | none | L | VDD_EXT | Desc: Reset Output |
|  |  |  |  |  |  |  | Notes: No notes |
| SYS_XTAL0 | a | NA | none | none | none | VDD_EXT | Desc: Crystal Output |
|  |  |  |  |  |  |  | Notes: No notes |
| SYS_XTAL1 | a | NA | none | none | none | VDD_EXT | Desc: Crystal Output |
|  |  |  |  |  |  |  | Notes: No notes |
| TWI0_SCL | InOut | D | none | none | none | VDD_EXT | Desc: TWI0 Serial Clock |
|  |  |  |  |  |  |  | Notes: Add external pull-up if used. Can be pulled low when not used. |
| TWI0_SDA | InOut | D | none | none | none | VDD_EXT | Desc: TWI0 Serial Data |
|  |  |  |  |  |  |  | Notes: Add external pull-up if used. Can be pulled low when not used. |
| TWI1_SCL | InOut | D | none | none | none | VDD_EXT | Desc: TWI1 Serial Clock |
|  |  |  |  |  |  |  | Notes: Add external pull-up if used. Can be pulled low when not used. |
| TWI1_SDA | InOut | D | none | none | none | VDD_EXT | Desc: TWI1 Serial Data |
|  |  |  |  |  |  |  | Notes: Add external pull-up if used. Can be pulled low when not used. |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TWI2_SCL | InOut | D | none | none | none | VDD_EXT | Desc: TWI2 Serial Clock |
|  |  |  |  |  |  |  | Notes: Add external pull-up if used. Can be pulled low when not used. |
| TWI2_SDA | InOut | D | none | none | none | VDD_EXT | Desc: TWI2 Serial Data |
|  |  |  |  |  |  |  | Notes: Add external pull-up if used. Can be pulled low when not used. |
| USB0_DM | InOut | F | none | none | none | VDD_USB | Desc: USB0 Data − |
|  |  |  |  |  |  |  | Notes: Add external pull-down if not used1 |
| USB0_DP | InOut | F | none | none | none | VDD_USB | Desc: USB0 Data + |
|  |  |  |  |  |  |  | Notes: Add external pull-down if not used1 |
| USB0_ID | InOut |  | none | none | none | VDD_USB | Desc: USB0 OTG ID |
|  |  |  |  |  |  |  | Notes: Connect to GND when USB is not used1 |
| USB0_VBC | InOut | E | none | none | none | VDD_USB | Desc: USB0 VBUS Control |
|  |  |  |  |  |  |  | Notes: Add external pull-down if not used1 |
| USB0_VBUS | InOut | G | none | none | none | VDD_USB | Desc: USB0 Bus Voltage |
|  |  |  |  |  |  |  | Notes: Connect to GND if not used1 |
| USB1_DM | InOut | F | none | none | none | VDD_USB | Desc: USB1 Data − |
|  |  |  |  |  |  |  | Notes: Add external pull-down if not used1 |
| USB1_DP | InOut | F | none | none | none | VDD_USB | Desc: USB1 Data + |
|  |  |  |  |  |  |  | Notes: Add external pull-down if not used1 |
| USB1_VBUS | InOut | G | none | none | none | VDD_USB | Desc: USB1 Bus Voltage |
|  |  |  |  |  |  |  | Notes: Connect to GND if not used1 |
| USB_CLKIN | a |  | none | none | none |  | Desc: USB0/USB1 Clock/Crystal Input |
|  |  |  |  |  |  |  | Notes: Services both USB0 and USB1. Connect to GND if not used.1 |
| USB_XTAL | a |  | none | none | none |  | Desc: USB0/USB1 Crystal |
|  |  |  |  |  |  |  | Notes: Services both USB0 and USB1 |
| VDD_DMC | s | NA | none | none | none |  | Desc: DMC VDD |
|  |  |  |  |  |  |  | Notes: No notes |
| VDD_EXT | s | NA | none | none | none |  | Desc: External Voltage Domain |
|  |  |  |  |  |  |  | Notes: No notes |
| VDD_HADC | s | NA | none | none | none |  | Desc: HADC/TMU VDD |
|  |  |  |  |  |  |  | Notes: Can be left floating if HADC and TMU are not used |
| VDD_INT | s | NA | none | none | none |  | Desc: Internal Voltage Domain |
|  |  |  |  |  |  |  | Notes: No notes |
| VDD_PCIE | s | NA | none | none | none |  | Desc: PCIE Supply Voltage |
|  |  |  |  |  |  |  | Notes: Connect to GND if not used1, 2 |
| VDD_PCIE_RX | s | NA | none | none | none |  | Desc: PCIE RX Supply Voltage |
|  |  |  |  |  |  |  | Notes: Connect to GND if not used1, 2 |
| VDD_PCIE_TX | s | NA | none | none | none |  | Desc: PCIE TX Supply Voltage |
|  |  |  |  |  |  |  | Notes: Connect to GND if not used1, 2 |

| Signal Name | Type | Driver Type | Internal Term | Reset Term | Reset Drive | Power Domain | Description and Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VDD_RTC | s | NA | none | none | none |  | Desc: RTC VDD |
|  |  |  |  |  |  |  | Notes: No notes |
| VDD_USB | s | NA | none | none | none |  | Desc: USB VDD |
|  |  |  |  |  |  |  | Notes: Connect to VDD_EXT when USB is not used |

1 Guidance also applies to models that do not feature the associated hardware block. See Table 2 or Table 3 for further information.

2 For boundary scan to work, PCIE power supplies must be connected as per Specifications.

# SPECIFICATIONS

Specifications are subject to change without notice. For information about product specifications, contact your Analog Devices, Inc., representative.

## OPERATING CONDITIONS

| Parameter |  | Conditions | Min | Nominal | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VDD_INT VDD_EXT VDD_HADC VDD_DM 1 C VDD_USB2 VDD_RTC VDD_PCIE_TX VDD_PCIE_RX VDD_PCIE VDDR_VREF3 VHADC_REF4 VHADC0_VINx V 5 IH VIHTW 6, 7 I V 5 IL VILTW 6, 7 I VIL_DDR 8 2 VIL_DDR 8 3 VIH_DDR 8 2 VIH_DDR 8 3 VIL_LPDDR9 VIH_LPD 9 DR TJ TJ TJ TJ TJ TJ TJ TJ TJ TJ | Internal (Core) Supply Voltage External (I/O) Supply Voltage Analog Power Supply Voltage DDR2/LPDDR Controller Supply Voltage DDR3 Controller Supply Voltage USB Supply Voltage RTC Voltage PCIe Core Transmit Voltage PCIe Core Receive Voltage PCIe Voltage DDR2/DDR3 Reference Voltage HADC Reference Voltage HADC Input Voltage High Level Input Voltage High Level Input Voltage Low Level Input Voltage Low Level Input Voltage Low Level Input Voltage Low Level Input Voltage High Level Input Voltage High Level Input Voltage Low Level Input Voltage High Level Input Voltage Junction Temperature 349-Lead CSP_BGA Junction Temperature 349-Lead CSP_BGA Junction Temperature 349-Lead CSP_BGA Junction Temperature 529-Lead CSP_BGA Junction Temperature 529-Lead CSP_BGA Junction Temperature 349-Lead CSP_BGA Junction Temperature 349-Lead CSP_BGA Junction Temperature 349-Lead CSP_BGA Junction Temperature 529-Lead CSP_BGA Junction Temperature 529-Lead CSP_BGA | CCLK ≤ 450 MHz | 1.05 | 1.10 | 1.15 | V |
|  |  | CCLK ≤ 500 MHz | 1.10 | 1.15 | 1.20 | V |
|  |  |  | 3.13 | 3.3 | 3.47 | V |
|  |  |  | 3.13 | 3.3 | 3.47 | V |
|  |  |  | 1.7 | 1.8 | 1.9 | V |
|  |  |  | 1.425 | 1.5 | 1.575 | V |
|  |  |  | 3.13 | 3.3 | 3.47 | V |
|  |  |  | 2.0 | 3.3 | 3.60 | V |
|  |  |  | 1.05 | 1.1 | 1.15 | V |
|  |  |  | 1.05 | 1.1 | 1.15 | V |
|  |  |  | 3.13 | 3.3 | 3.47 | V |
|  |  |  | 0.49 × VDD_DMC | 0.50 × VDD_DMC | 0.51 × VDD_DMC | V |
|  |  |  | 2.5 | 3.30 | VDD_HADC | V |
|  |  |  | 0 |  | VHADC_REF + 0.2 | V |
|  |  | VDD_EXT = 3.47 V | 2.0 |  |  | V |
|  |  | VDD_EXT = 3.47 V | 0.7 × VBUSTWI |  | VBUSTWI | V |
|  |  | VDD_EXT = 3.13 V |  |  | 0.8 | V |
|  |  | VDD_EXT = 3.13 V |  |  | 0.3 × VBUSTWI | V |
|  |  | VDD_DMC = 1.7 V |  |  | VDDR_VREF – 0.25 | V |
|  |  | VDD_DMC = 1.425 V |  |  | VDDR_VREF – 0.175 | V |
|  |  | VDD_DMC = 1.9 V | VDDR_VREF + 0.25 |  |  | V |
|  |  | VDD_DMC = 1.575 V | VDDR_VREF + 0.175 |  |  | V |
|  |  | VDD_DMC = 1.7 V |  |  | 0.2 × VDD_DMC | V |
|  |  | VDD_DMC = 1.9 V | 0.8 × VDD_DMC |  |  | V |
|  |  | TAMBIENT = 0°C to 70°C | 0 |  | 100 | °C |
|  |  | CCLK ≤ 450 MHz |  |  |  |  |
|  |  | TAMBIENT = –40°C to +85°C | –40 |  | +110 | °C |
|  |  | CCLK ≤ 450 MHz |  |  |  |  |
|  |  | TAMBIENT = –40°C to +95°C | –40 |  | +125 | °C |
|  |  | CCLK ≤ 450 MHz |  |  |  |  |
|  |  | TAMBIENT = 0°C to 70°C | 0 |  | 110 | °C |
|  |  | CCLK ≤ 450 MHz |  |  |  |  |
|  |  | TAMBIENT = –40°C to +85°C | –40 |  | +125 | °C |
|  |  | CCLK ≤ 450 MHz |  |  |  |  |
|  |  | TAMBIENT = 0°C to 70°C | 0 |  | 105 | °C |
|  |  | CCLK ≤ 500 MHz |  |  |  |  |
|  |  | TAMBIENT = –40°C to +85°C | –40 |  | +120 | °C |
|  |  | CCLK ≤ 500 MHz |  |  |  |  |
|  |  | TAMBIENT = –40°C to +90°C | –40 |  | +125 | °C |
|  |  | CCLK ≤ 500 MHz |  |  |  |  |
|  |  | TAMBIENT = 0°C to 70°C | 0 |  | 115 | °C |
|  |  | CCLK ≤ 500 MHz |  |  |  |  |
|  |  | TAMBIENT = –40°C to +80°C | –40 |  | +125 | °C |
|  |  | CCLK ≤ 500 MHz |  |  |  |  |

| Parameter | Conditions | Min | Nominal | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| AUTOMOTIVE USE ONLY TJ Junction Temperature 349-Lead CSP_BGA (Automotive Grade)10 TJ Junction Temperature 529-Lead CSP_BGA (Automotive Grade)10 TJ Junction Temperature 349-Lead CSP_BGA (Automotive Grade)10 TJ Junction Temperature 529-Lead CSP_BGA (Automotive Grade)10 | TAMBIENT = –40°C to +105°C | –40 | +133 |  | °C |
|  | CCLK ≤ 450 MHz |  |  |  |  |
|  | TAMBIENT = –40°C to +90°C | –40 | +133 |  | °C |
|  | CCLK ≤ 450 MHz |  |  |  |  |
|  | TAMBIENT = –40°C to +100°C | –40 | +133 |  | °C |
|  | CCLK ≤ 500 MHz |  |  |  |  |
|  | TAMBIENT = –40°C to +85°C | –40 | +133 |  | °C |
|  | CCLK ≤ 500 MHz |  |  |  |  |

1 Applies to DDR2/DDR3/LPDDR signals.

2 If not used, VDD_USB must be connected to 3.3 V. 3 Applies to DMC0_VREF and DMC1_VREF pins. 4 VHADC_VREF must always be less than VDD_HADC.

5 Parameter value applies to all input and bidirectional pins except the TWI, DMC, USB, PCIe, and MLB pins.

6 Parameter applies to TWI signals.

7 TWI signals are pulled up to VBUSTWI. See Table 28.

8 This parameter applies to all DMC0/1 signals in DDR2/DDR3 mode.

9 This parameter applies to DMC0/1 signals in LPDDR mode.

10Automotive application use profile only. Not supported for nonautomotive use. Contact Analog Devices for more information.

Table 28. TWIxVSEL1 Settings and VDD_EXT/VBUSTWI

| TWIxVSEL | VDD_EXT Nominal | VBUSTWI |  |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  | Min | Nominal | Max |  |
| 02 | 3.30 | 3.13 | 3.30 | 3.47 | V |
| 1 | 3.30 | 4.75 | 5.00 | 5.25 | V |

1 TWIxVSEL are the TWI voltage select bits in the PADS_PCFG0 register. See the ADSP-SC58x/ADSP-2158x SHARC+ Processor Hardware Reference.

2 Designs must comply with the VDD_EXT and VBUSTWI voltages specified for the default TWIxVSEL setting for correct JTAG boundary scan operation during reset.

#### Clock Related Operating Conditions

Table 29 describes the core clock, system clock, and peripheral clock timing requirements. The data presented in the table applies to all speed grades except where noted.

##### Table 29. Clock Operating Conditions

| Parameter |  | Restriction | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fCCLK | Core Clock Frequency | fCCLK ≥ fSYSCLK | 100 |  | 500 | MHz |
| fSYSCLK | SYSCLK Frequency1 |  |  |  | 250 | MHz |
| fSCLK0 | SCLK0 Frequency2 | fSYSCLK ≥ fSCLK0 | 30 |  | 125 | MHz |
| fSCLK1 | SCLK1 Frequency | fSYSCLK ≥ fSCLK1 |  |  | 125 | MHz |
| fDCLK | LPDDR Clock Frequency |  |  |  | 200 | MHz |
| fDCLK | DDR2 Clock Frequency |  |  |  | 400 | MHz |
| fDCLK | DDR3 Clock Frequency |  |  |  | 450 | MHz |
| fOCLK | Output Clock Frequency3 |  |  |  | 250 | MHz |
| fSYS_CLKOUTJ | SYS_CLKOUT Period Jitter4, 5 |  |  | ±2 |  | % |
| fPCLKPROG | Programmed PPI Clock When Transmitting Data and Frame Sync |  |  |  | 75 | MHz |
| fPCLKPROG | Programmed PPI Clock When Receiving Data or Frame Sync |  |  |  | 45 | MHz |
| fPCLKEXT | External PPI Clock When Receiving Data and Frame Sync6, 7 | fPCLKEXT ≤ fSCLK1 |  |  | 75 | MHz |
| fPCLKEXT | External PPI Clock Transmitting Data or Frame Sync6, 7 | fPCLKEXT ≤ fSCLK1 |  |  | 45 | MHz |
| fLCLKTPROG | Programmed Link Port Transmit Clock |  |  |  | 150 | MHz |
| fLCLKREXT | External Link Port Receive Clock6, 7 | fLCLKEXT ≤ fCLKO8 |  |  | 150 | MHz |
| fSPTCLKPROG | Programmed SPT Clock When Transmitting Data and Frame Sync |  |  |  | 62.5 | MHz |
| fSPTCLKPROG | Programmed SPT Clock When Receiving Data or Frame Sync |  |  |  | 31.25 | MHz |
| fSPTCLKEXT | External SPT Clock When Receiving Data and Frame Sync6, 7 | fSPTCLKEXT ≤ fSCLK0 |  |  | 62.5 | MHz |
| fSPTCLKEXT | External SPT Clock Transmitting Data or Frame Sync6, 7 | fSPTCLKEXT ≤ fSCLK0 |  |  | 31.25 | MHz |
| fSPICLKPROG | Programmed SPI Clock When Transmitting Data |  |  |  | 75 | MHz |
| fSPICLKPROG | Programmed SPI Clock When Receiving Data |  |  |  | 75 | MHz |
| fSPICLKEXT | External SPI Clock When Receiving Data6, 7 | fSPICLKEXT ≤ fSCLK1 |  |  | 75 | MHz |
| fSPICLKEXT | External SPI Clock When Transmitting Data6, 7 | fSPICLKEXT ≤ fSCLK1 |  |  | 45 | MHz |
| fACLKPROG | Programmed ACM Clock |  |  |  | 62.5 | MHz |

1 When using MLB, there is a requirement that the fSYSCLK value must be a minimum of 100 MHz for both 3-pin and 6-pin modes and for all supported speeds.

2 The minimum frequency for SCLK0 applies only when using the USB.

3 fOCLK must not exceed fSCLK0 when selected as SYS_CLKOUT.

4 SYS_CLKOUT jitter is dependent on the application system design including pin switching activity, board layout, and the jitter characteristics of the SYS_CLKIN source. Due to the dependency on these factors, the measured jitter may be higher or lower than this typical specification for each end application.

5 The value in the Typ field is the percentage of the SYS_CLKOUT period.

6 The maximum achievable frequency for any peripheral in external clock mode is dependent on the ability to meet the setup and hold times in the ac timing specifications section for that peripheral.

7 The peripheral external clock frequency must also be less than or equal to the fSCLK (fSCLK0 or fSCLK1) that clocks the peripheral.

##### Table 30. Phase-Locked Loop (PLL) Operating Conditions

| Parameter | Min Max | Unit |
| --- | --- | --- |
| fPLLCLK PLL Clock Frequency | 200 1000 | MHz |

PLLCLK

OSEL (1-127)

DSEL (1-31)

PLL

CSEL (1-31)

CCLK

SYSCLK

S1SEL (1-7)

SYSSEL (1-31)

S0SEL (1-7)

SCLK0

SYS_CLKIN

DCLK

SCLK1

OUTCLK

Figure 8. Clock Relationships and Divider Values

## ELECTRICAL CHARACTERISTICS

| Parameter |  | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VOH1 VOL1 VOH_DDR23 VOL_DDR23 VOH_DDR23 VOL_DDR23 VOH_DDR34 VOL_DDR34 VOH_DDR34 VOL_DDR34 VOH_LPDDR5 VOL_LPDDR5 I 6, 7 IH I 6 IL IIL_P 7 U IIH_P 8 D IOZ 9 H IOZL9 C 10 IN IDD_IDLE | High Level Output Voltage Low Level Output Voltage High Level Output Voltage for DDR2 DS = 40 Ω Low Level Output Voltage for DDR2 DS = 40 Ω High Level Output Voltage for DDR2 DS = 60 Ω Low Level Output Voltage for DDR2 DS = 60 Ω High Level Output Voltage for DDR3 DS = 40 Ω Low Level Output Voltage for DDR3 DS = 40 Ω High Level Output Voltage for DDR3 DS = 60 Ω Low Level Output Voltage for DDR3 DS = 60 Ω High Level Output Voltage for LPDDR Low Level Output Voltage for LPDDR High Level Input Current Low Level Input Current Low Level Input Current Pull-Up High Level Input Current Pull-Down Three-State Leakage Current Three-State Leakage Current Input Capacitance VDD_INT Current in Idle | At VDD_EXT = minimum, IOH = –1.0 mA2 | 2.4 |  |  | V |
|  |  | At VDD_EXT = minimum, IOL = 1.0 mA2 |  |  | 0.4 | V |
|  |  | At VDD_DDR = minimum, IOH = –5.8 mA | 1.38 |  |  | V |
|  |  | At VDD_DDR = minimum, IOL = 5.8 mA |  |  | 0.32 | V |
|  |  | At VDD_DDR = minimum, IOH = –3.4 mA | 1.38 |  |  | V |
|  |  | At VDD_DDR = minimum, IOL = 3.4 mA |  |  | 0.32 | V |
|  |  | At VDD_DDR = minimum, IOH = –5.8 mA | 1.105 |  |  | V |
|  |  | At VDD_DDR = minimum, IOL = 5.8 mA |  |  | 0.32 | V |
|  |  | At VDD_DDR = minimum, IOH = –3.4 mA | 1.105 |  |  | V |
|  |  | At VDD_DDR = minimum, IOL = 3.4 mA |  |  | 0.32 | V |
|  |  | At VDD_DDR = minimum, IOH = –6.0 mA | 1.38 |  |  | V |
|  |  | At VDD_DDR = minimum, IOL = 6.0 mA |  |  | 0.32 | V |
|  |  | At VDD_EXT = maximum, |  |  | 10 | μA |
|  |  | VIN = VDD_EXT maximum |  |  |  |  |
|  |  | At VDD_EXT = maximum, VIN = 0 V |  |  | 10 | μA |
|  |  | At VDD_EXT = maximum, VIN = 0 V |  |  | 200 | μA |
|  |  | At VDD_EXT = maximum, VIN = VDD_EXT |  |  | 200 | μA |
|  |  | maximum |  |  |  |  |
|  |  | At VDD_EXT/VDD_DDR = maximum, |  |  | 10 | μA |
|  |  | VIN = VDD_EXT/VDD_DDR maximum |  |  |  |  |
|  |  | at VDD_EXT/VDD_DDR = maximum, |  |  | 10 | μA |
|  |  | VIN = 0 V |  |  |  |  |
|  |  | TCASE = 25°C |  |  | 5 | pF |
|  |  | fCCLK = 450 MHz |  | 495 |  | mA |
|  |  | ASFSHARC1 = 0.31 |  |  |  |  |
|  |  | ASFSHARC2 = 0.31 |  |  |  |  |
|  |  | ASFA5 = 0.29 |  |  |  |  |
|  |  | fSYSCLK = 225 MHz |  |  |  |  |
|  |  | fSCLK0/1 = 112.5 MHz |  |  |  |  |
|  |  | (Other clocks are disabled) |  |  |  |  |
|  |  | No peripheral or DMA activity |  |  |  |  |
|  |  | TJ = 25°C |  |  |  |  |
|  |  | VDD_INT = 1.1 V |  |  |  |  |

| Parameter |  | Conditions | Min Typ Max | Unit |
| --- | --- | --- | --- | --- |
| IDD_IDLE | VDD_INT Current in Idle | fCCLK = 500 MHz ASFSHARC1 = 0.31 ASFSHARC2 = 0.31 ASFA5 = 0.29 fSYSCLK = 250 MHz fSCLK0/1 = 125 MHz (Other clocks are disabled) No peripheral or DMA activity TJ = 25°C VDD_INT = 1.15 V | 575 | mA |
| IDD_TYP | VDD_INT Current | fCCLK = 450 MHz ASFSHARC1 = 1.0 ASFSHARC2 = 1.0 ASFA5 = 0.73 fSYSCLK = 225 MHz fSCLK0/1 = 112.5 MHz (Other clocks are disabled) FFT accelerator operating at fSYSCLK/4 DMA data rate = 600 Mbps TJ = 25°C VDD_INT = 1.1 V | 1112 | mA |
| IDD_TYP | VDD_INT Current | fCCLK = 500 MHz ASFSHARC1 = 1.0 ASFSHARC2 = 1.0 ASFA5 = 0.73 fSYSCLK = 250 MHz fSCLK0/1 = 125 MHz (Other clocks are disabled) FFT accelerator operating at fSYSCLK/4 DMA data rate = 600 Mbps TJ = 25°C VDD_INT = 1.15 V | 1185 | mA |
| IDD_IN 11 T | VDD_INT Current | fCCLK  0 MHz fSCLK0/1  0 MHz | See IDD_INT_TOT equation in the Total Internal | mA |
|  |  |  | Power Dissi- |  |
|  |  |  | pation section |  |

1 Applies to all output and bidirectional pins except TWI, DMC, USB, PCIe, and MLB.

2 See the Output Drive Currents section for typical drive current capabilities.

3 Applies to all DMC output and bidirectional signals in DDR2 mode.

4 Applies to all DMC output and bidirectional signals in DDR3 mode.

5 Applies to all DMC output and bidirectional signals in LPDDR mode.

6 Applies to input pins SYS_BMODE0-2, SYS_CLKIN0, SYS_CLKIN1, SYS_HWRST, JTG_TDI, JTG_TMS, and USB0_CLKIN.

7 Applies to input pins with internal pull-ups including JTG_TDI, JTG_TMS, and JTG_TCK.

8 Applies to signals JTAG_TRST, USB0_VBUS, USB1_VBUS.

9 Applies to signals PA0-15, PB0-15, PC0-15, PD0-15, PE0-15, PF0-15, PG0-5, DAI0_PINx, DAI1_PINx, DMC0_DQx, DMC0_LDQS, DMC0_UDQS, DMC0_LDQS, DMC0_UDQS, SYS_FAULT, SYS_FAULT, JTG_TDO, USB0_ID, USBx_DM, USBx_DP, and USBx_VBC.

10Applies to all signal pins.

11 See Estimating Power for ADSP-SC58x/2158x SHARC+ Processors (EE-392) for further information.

#### Total Internal Power Dissipation

Total power dissipation has two components:

  - Static, including leakage current

  - Dynamic, due to transistor switching characteristics for each clock domain

Many operating conditions can affect power dissipation, includ- ing temperature, voltage, operating frequency, and processor activity. The following equation describes the internal current consumption.

IDD_INT_TOT = IDD_INT_STATIC + IDD_INT_CCLK_SHARC1_DYN + IDD_INT_CCLK_SHARC2_DYN + IDD_INT_CCLK_A5_DYN + IDD_INT_DCLK_DYN + IDD_INT_SYSCLK_DYN + IDD_INT_SCLK0_DYN + IDD_INT_SCLK1_DYN + IDD_INT_OCLK_DYN + IDD_INT_ACCL_DYN + IDD_INT_USB_DYN + IDD_INT_MLB_DYN + IDD_INT_EMAC_DYN + IDD_INT_DMA_DR_DYN + IDD_INT_PCIE_DYN

IDD_INT_STATIC is the sole contributor to the static power dissi- pation component and is specified as a function of voltage (VDD_INT) and junction temperature (TJ) in Table 31.

Table 31. Static Current—IDD_INT_STATIC (mA)

| TJ (°C) | Voltage (VDD_INT) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.05 | 1.10 | 1.15 | 1.20 |
| –40 | 7 | 8 | 10 | 12 |
| –20 | 12 | 14 | 17 | 21 |
| –10 | 16 | 19 | 23 | 27 |
| +0 | 21 | 25 | 30 | 35 |
| +10 | 28 | 33 | 39 | 46 |
| +25 | 42 | 49 | 58 | 67 |
| +40 | 63 | 73 | 84 | 98 |
| +55 | 92 | 106 | 122 | 141 |
| +70 | 133 | 152 | 175 | 200 |
| +85 | 190 | 216 | 247 | 282 |
| +100 | 269 | 305 | 346 | 393 |
| +105 | 302 | 342 | 387 | 439 |
| +115 | 376 | 425 | 480 | 544 |
| +125 | 466 | 525 | 592 | 669 |
| +133 | 552 | 621 | 700 | 789 |

The other 14 addends in the IDD_INT_TOT equation comprise the dynamic power dissipation component and fall into four broad categories: application-dependent currents, clock currents, cur- rents from high speed peripheral operation, and data transmission currents.

##### Application Dependent Current

The application dependent currents include the dynamic cur- rent in the core clock domain of the two SHARC+ cores and the Arm Cortex-A5 core, as well as the dynamic current in the accelerator block.

Dynamic current consumed by the core is subject to an activity scaling factor (ASF) that represents application code running on the processor cores (see Table 32 and Table 33). The ASF is combined with the CCLK frequency and VDD_INT dependent dynamic current data in Table 34 and Table 35, respectively, to calculate this portion of the total dynamic power dissipation component.

IDD_INT_CCLK_SHARC1_DYN = Table 34 × ASFSHARC1 IDD_INT_CCLK_SHARC2_DYN = Table 34 × ASFSHARC2 IDD_INT_CCLK_A5_DYN = Table 35 × ASFA5

##### Table 32. Activity Scaling Factors for the SHARC+ Core1 and Core2 (ASFSHARC1 and ASFSHARC2)

| IDD_INT Power Vector | ASF |
| --- | --- |
| IDD-IDLE | 0.31 |
| IDD-NOP | 0.53 |
| IDD-TYP_3070 | 0.74 |
| IDD-TYP_5050 | 0.87 |
| IDD-TYP_7030 | 1.00 |
| IDD-PEAK_100 | 1.14 |

Table 33. Activity Scaling Factors for the Arm Cortex-A5 Core (ASFA5)

| IDD_INT Power Vector | ASF |
| --- | --- |
| IDD-IDLE | 0.29 |
| IDD-DHRYSTONE | 0.73 |
| IDD-TYP_2575 | 0.57 |
| IDD-TYP_5050 | 0.80 |
| IDD-TYP_7525 | 1.00 |
| IDD-PEAK_100 | 1.21 |

Table 34. Dynamic Current for Each SHARC+ Core (mA, with ASF = 1.00)1

| fCCLK (MHz) | Voltage (VDD_INT) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.05 | 1.10 | 1.15 | 1.20 |
| 500 | N/A | 374 | 391 | 408 |
| 450 | 321 | 337 | 352 | 367 |
| 400 | 286 | 299 | 313 | 326 |
| 350 | 250 | 262 | 274 | 286 |
| 300 | 214 | 224 | 235 | 245 |
| 250 | 179 | 187 | 196 | 204 |
| 200 | 143 | 150 | 156 | 163 |
| 150 | 107 | 112 | 117 | 122 |
| 100 | 71 | 75 | 78 | 82 |

1 N/A means not applicable.

##### Table 35. Dynamic Current for the Arm Cortex-A5 Core (mA, with ASF = 1.00)1

| fCCLK (MHz) | Voltage (VDD_INT) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.05 | 1.10 | 1.15 | 1.20 |
| 500 | N/A | 83 | 86 | 90 |
| 450 | 71 | 74 | 78 | 81 |
| 400 | 63 | 66 | 69 | 72 |
| 350 | 55 | 58 | 60 | 63 |
| 300 | 47 | 50 | 52 | 54 |
| 250 | 39 | 41 | 43 | 45 |
| 200 | 32 | 33 | 35 | 36 |
| 150 | 24 | 25 | 26 | 27 |
| 100 | 16 | 17 | 18 | 19 |

1 N/A means not applicable.

The following equation is used to compute the power dissipa- tion when the FFT accelerator is used:

IDD_INT_ACCL_DYN (mA) = ASFACCL × fSYSCLK (MHz) ×

VDD_INT (V)

##### Table 36. Activity Scaling Factors for the FFT Accelerator (ASFACCL)

Clock Current

The dynamic clock currents provide the total power dissipated by all transistors switching in the clock paths. The power dissi- pated by each clock domain is dependent on voltage (VDD_INT), operating frequency, and a unique scaling factor.

IDD_INT_SYSCLK_DYN (mA) = 0.78 × fSYSCLK (MHz) ×

VDD_INT (V)

IDD_INT_SCLK0_DYN (mA) = 0.44 × fSCLK0 (MHz) × VDD_INT (V) IDD_INT_SCLK1_DYN (mA) = 0.06 × fSCLK1 (MHz) × VDD_INT (V) IDD_INT_DCLK_DYN (mA) = 0.14 × fDCLK (MHz) × VDD_INT (V) IDD_INT_OCLK_DYN (mA) = 0.02 × fOCLK (MHz) × VDD_INT (V)

##### Current from High Speed Peripheral Operation

The following modules contribute significantly to power dissi- pation, and a single term is added when the modules are used.

IDD_INT_USB_DYN = 20 mA (if both USBs are enabled in HS mode)

IDD_INT_MLB_DYN = 10 mA (if MLB 6-pin interface is enabled) IDD_INT_EMAC_DYN = 10 mA (if EMAC is enabled) IDD_INT_PCIE_DYN = 240 mA (if PCIe is enabled in 5 Gbps mode)

##### Data Transmission Current

The data transmission current represents the power dissipated when moving data throughout the system via direct memory access (DMA). This current is proportional to the data rate.

Refer to the power calculator available with Estimating Power for ADSP-SC58x/2158x SHARC+ Processors (EE-392) to esti- mate IDD_INT_DMA_DR_DYN based on the bandwidth of the data transfer.

| IDD_INT Power Vector | ASFACCL |
| --- | --- |
| Unused | 0.0 |
| IDD-TYP | 0.32 |

## HADC

#### HADC Electrical Characteristics

##### Table 37. HADC Electrical Characteristics

| Parameter | Conditions | Typ | Unit |
| --- | --- | --- | --- |
| IDD_HADC_IDLE | Current consumption on VDD_HADC. HADC is powered on, but not converting. | 2.0 | mA |
| IDD_HADC_ACTIVE | Current consumption on VDD_HADC during a conversion. | 2.5 | mA |
| IDD_HADC_POWERDOWN | Current consumption on VDD_HADC. Analog circuitry of the HADC is powered down. | 10 | μA |

HADC DC Accuracy

Table 38. HADC DC Accuracy1

| Parameter | Typ | Unit2 |
| --- | --- | --- |
| Resolution | 12 | Bits |
| No Missing Codes (NMC) | 10 | Bits |
| Integral Nonlinearity (INL) | ±2 | LSB |
| Differential Nonlinearity (DNL) | ±2 | LSB |
| Offset Error | ±8 | LSB |
| Offset Error Matching | ±10 | LSB |
| Gain Error | ±4 | LSB |
| Gain Error Matching | ±4 | LSB |

1 See the Operating Conditions section for the HADC0_VINx specification.

2 LSB = HADC0_VREFP ÷ 4096.

#### HADC Timing Specifications

##### Table 39. HADC Timing Specifications

| Parameter | Typ | Max | Unit |
| --- | --- | --- | --- |
| Conversion Time | 20 × TSAMPLE |  | μs |
| Throughput Range |  | 1 | MSPS |
| TWAKEUP |  | 100 | μs |

## TMU

TMU Characteristics

Table 40. TMU Characteristics

| Parameter | Typ | Unit |
| --- | --- | --- |
| Resolution | 1 | °C |
| Accuracy | ±6 | °C |

Table 41. TMU Gain and Offset

| Junction Temperature Range | TMU_GAIN | TMU_OFFSET |
| --- | --- | --- |
| –40°C to +40°C 40°C to 85°C 85°C to 133°C | Contact Analog Devices, Inc. Contact Analog Devices, Inc. Contact Analog Devices, Inc. |  |

## ABSOLUTE MAXIMUM RATINGS

Stresses at or above those listed in Table 42 may cause perma- nent damage to the product. This is a stress rating only; functional operation of the product at these or any other condi- tions above those indicated in the operational section of this specification is not implied. Operation beyond the maximum operating conditions for extended periods may affect product reliability.

##### Table 42. Absolute Maximum Ratings

| Parameter | Rating |
| --- | --- |
| Internal (Core) Supply Voltage (VDD_INT) | –0.33 V to +1.26 V |
| External (I/O) Supply Voltage (VDD_EXT) | –0.33 V to +3.60 V |
| DDR2/LPDDR Controller Supply Voltage (VDD_DMC) | –0.33 V to +1.90 V |
| DDR3 Controller Supply Voltage (VDD_DMC) | –0.33 V to +1.60 V |
| USB PHY Supply Voltage (VDD_USB) | –0.33 V to +3.60 V |
| Real-Time Clock Supply Voltage (VDD_RTC) | –0.33 V to +3.60 V |
| PCIe Transmit Supply Voltage (VDD_PCIE_TX) | –0.33 V to +1.20 V |
| PCIe Receive Supply Voltage (VDD_PCIE_RX) | –0.33 V to +1.20 V |
| PCIe Supply Voltage (VDD_PCIE) | –0.33 V to +3.60 V |
| HADC Supply Voltage (VDD_HADC) | –0.33 V to +3.60 V |
| HADC Reference Voltage (VHADC_REF) | –0.33 V to +3.60 V |
| DDR2/LPDDR Input Voltage1 | –0.33 V to +1.90 V |
| DDR2 Reference Voltage (VDDR_VREF) | –0.33 V to +1.90 V |
| DDR3 Input Voltage1 | –0.33 V to +1.60 V |
| Digital Input Voltage1, 2 | –0.33 V to +3.60 V |
| TWI Input Voltage1, 3 | –0.33 V to +5.50 V |
| USB0_Dx Input Voltage1, 4 | –0.33 V to +5.25 V |
| USB0_VBUS Input Voltage1, 4 | –0.33 V to +6 V |
| Output Voltage Swing | –0.33 V to VDD_EXT +0.5 V |
| Analog Input Voltage5 | –0.2 V to VDD_HADC +0.2 V |
| IOH/IOL Current per Signal2 | 6 mA (maximum) |
| Storage Temperature Range | –65C to +150C |
| Junction Temperature While Biased | 133C |

1 Applies only when the related power supply (VDD_DMC, VDD_EXT, or VDD_USB) is within specification. When the power supply is below specification, the range is the voltage being applied to that power domain ± 0.2 V.

2 Applies to 100% transient duty cycle.

3 Applies to TWI_SCL and TWI_SDA.

4 If the USB is not used, connect these pins according to Table 27.

5 Applies only when VDD_HADC is within specifications and ≤ 3.4 V. When VDD_HADC is within specifications and > 3.4 V, the maximum rating is 3.6 V. When VDD_HADC is below specifications, the range is VDD_HADC ± 0.2 V.

## ESD CAUTION

ESD (electrostatic discharge) sensitive device. Charged devices and circuit boards can discharge without detection. Although this product features patented or proprietary protection circuitry, damage may occur on devices subjected to high energy ESD. Therefore, proper ESD precautions should be taken to avoid performance degradation or loss of functionality.

## TIMING SPECIFICATIONS

#### Power-Up Reset Timing

Table 43 and Figure 9 show the relationship between power supply startup and processor reset timing, related to the clock generation unit (CGU) and reset control unit (RCU).

In Figure 9, VDD_SUPPLIES are VDD_INT, VDD_EXT, VDD_DMC, VDD_USB, VDD_HADC, VDD_RTC, VDD_PCI_TX, VDD_PCI_RX, and VDD_PCI_CORE.

##### Table 43. Power-Up Reset Timing

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Timing Requirement tRST_IN_PWR SYS_HWRST Deasserted after VDD_SUPPLIES (VDD_INT, VDD_EXT, VDD_DMC, VDD_USB, VDD_HADC, VDD_RTC, VDD_PCI_TX, VDD_PCI_RX, VDD_PCI_CORE) and SYS_CLKINx are Stable and Within Specification | 11 × tCKIN | ns |

tRST_IN_PWR

SYS_HWRST

SYS_CLKIN0/1 VDD_SUPPLIES

NOTE: V	REFER TO V	, V	, V	, V	, V	, V	, V	, V	, AND V	.                                      DD_SUPPLIES		DD_INT    DD_EXT    DD_DMC    DD_USB    DD_HADC    DD_RTC   DD_PCI_TX   DD_PCI_RX	DD_PCI_CORE

Figure 9. Power-Up Reset Timing

#### Clock and Reset Timing

Table 44 and Figure 10 describe clock and reset operations related to the CGU and RCU. Per the CCLK, SYSCLK, SCLK, DCLK, and OCLK timing specifications in Table 29, combinations of SYS_CLKIN and clock multipliers must not select clock rates in excess of the maximum instruction rate of the processor.

##### Table 44. Clock and Reset Timing

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| fCKIN SYS_CLKINx Frequency (Crystal)1, 2, 3 | 20 | 50 | MHz |
| SYS_CLKINx Frequency (External CLKIN)1, 2, 3 | 20 | 50 | MHz |
| tCKINL CLKIN Low Pulse1 | 10 |  | ns |
| tCKINH CLKIN High Pulse1 | 10 |  | ns |
| tWRST RESET Asserted Pulse Width Low4 | 11 × tCKIN |  | ns |

1 Applies to PLL bypass mode and PLL nonbypass mode.

2 The tCKIN period (see Figure 10) equals 1/fCKIN.

3 If the CGU_CTL.DF bit is set, the minimum fCKIN specification is 40 MHz.

4 Applies after power-up sequence is complete. See Table 43 and Figure 9 for power-up reset timing.

fCKIN

tCKINL	tCKINH

SYS_CLKIN0/1

tWRST

SYS_HWRST

Figure 10. Clock and Reset Timing

#### Asynchronous Read

Table 45 and Figure 11 show asynchronous memory read timing, related to the SMC.

##### Table 45. Asynchronous Read

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tSDATARE DATA in Setup Before SMC0_ARE High tHDATARE DATA in Hold After SMC0_ARE High tDARDYARE SMC0_ARDY Valid After SMC0_ARE Low1, 2 | 5.1 0.7 | (RAT – 2.5) × tSCLK0 – 17.5 | ns ns ns |
| Switching Characteristics |  |  |  |
| tAMSARE ADDR/SMC0_AMSx Assertion Before SMC0_ARE Low3 | (PREST + RST + PREAT) × tSCLK0 – 2 |  | ns |
| tAOEARE SMC0_AOE Assertion Before SMC0_ARE Low | (RST + PREAT) × tSCLK0 – 2 |  | ns |
| tHARE Output4 Hold After SMC0_ARE High5 | RHT × tSCLK0 –2 |  | ns |
| tWARE SMC0_ARE Active Low Width6 | RAT × tSCLK0 – 2 |  | ns |
| tDAREARDY SMC0_ARE High Delay After SMC0_ARDY Assertion1 | 2.5 × tSCLK0 | 3.5 × tSCLK0 + 17.5 | ns |

1 SMC0_BxCTL.ARDYEN bit = 1.

2 RAT value set using the SMC_BxTIM.RAT bits.

3 PREST, RST, and PREAT values set using the SMC_BxETIM.PREST bits, SMC_BxTIM.RST bits, and the SMC_BxETIM.PREAT bits.

4 Output signals are SMC0_Ax, SMC0_AMS, SMC0_AOE, SMC0_ABEx.

5 RHT value set using the SMC_BxTIM.RHT bits.

6 SMC0_BxCTL.ARDYEN bit = 0.

tWARE

tAMSARE

tHARE

tAOEARE

tDARDYARE

tDAREARDY

tSDATARE

tHDATARE

SMC0_ARE

SMC0_AMSx

SMC0_Ax

SMC0_AOE

SMC0_ARDY

SMC0_Dx (DATA)

Figure 11. Asynchronous Read

#### SMC Read Cycle Timing With Reference to SYS_CLKOUT

The following SMC specifications (Table 46 and Figure 12) with respect to SYS_CLKOUT are given to accommodate the connection of the SMC to programmable logic devices. These specifications assume that SYS_CLKOUT is outputting a buffered version of SCLK0 by setting CGU_CLKOUTSEL.CLKOUTSEL = 0x3.

##### Table 46. SMC Read Cycle Timing With Reference to SYS_CLKOUT (BxMODE = b#00)

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tSDAT SMC0_Dx Setup Before SYS_CLKOUT | 4.3 |  | ns |
| tHDAT SMC0_Dx Hold After SYS_CLKOUT | 5 |  | ns |
| tSARDY SMC0_ARDY Setup Before SYS_CLKOUT | 14.4 |  | ns |
| tHARDY SMC0_ARDY Hold After SYS_CLKOUT | 0.7 |  | ns |
| Switching Characteristics |  |  |  |
| tDO Output Delay After SYS_CLKOUT1 |  | 7 | ns |
| tHO Output Hold After SYS_CLKOUT 1 | –2.5 |  | ns |

1 Output signals are SMC0_Ax, SMC0_AMSx, SMC0_AOE, and SMC0_ABEx.

SYS_CLKOUT

tDO

tHO

SMC0_AMSx

SMC0_ABEx SMC0_Ax

SMC0_AOE

tDO

tHO

SMC0_ARE

tSARDY

tHARDY

SMC0_ARDY

tSARDY

tHARDY

tSDAT

tHDAT

DATA 15–0

SETUP CYCLES

PROGRAMMED READ ACCESS CYCLES

ACCESS EXTENDED CYCLES

HOLD CYCLE

Figure 12. Asynchronous Memory Read Cycle Timing

#### Asynchronous Flash Read

Table 47 and Figure 13 show asynchronous flash memory read timing, related to the SMC.

##### Table 47. Asynchronous Flash Read

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Switching Characteristics |  |  |
| tAMSADV SMC0_Ax (Address)/SMC0_AMSx Assertion Before SMC0_NORDV Low1 | PREST × tSCLK0 – 2 | ns |
| tWADV SMC0_NORDV Active Low Width2 | RST × tSCLK0 – 2 | ns |
| tDADVARE SMC0_ARE Low Delay From SMC0_NORDV High3 | PREAT × tSCLK0 – 2 | ns |
| tHARE Output4 Hold After SMC0_ARE High5 | RHT × tSCLK0 – 2 | ns |
| tWAR 6 SMC0_ARE Active Low Width7 E | RAT × tSCLK0 – 2 | ns |

1 PREST value set using the SMC_BxETIM.PREST bits.

2 RST value set using the SMC_BxTIM.RST bits.

3 PREAT value set using the SMC_BxETIM.PREAT bits.

4 Output signals are SMC0_Ax, SMC0_AMS, SMC0_AOE.

5 RHT value set using the SMC_BxTIM.RHT bits.

6 SMC0_BxCTL.ARDYEN bit = 0.

7 RAT value set using the SMC_BxTIM.RAT bits.

tAMSADV

tWADV

tDADVARE

tWARE		 	tHARE

SMC0_Ax (NOR_Ax)

SMC0_AMSx (NOR_CE)

SMC0_AOE (NOR_ADV)

SMC0_ARE (NOR_OE)

SMC0_Dx (NOR_Dx)

READ LATCHED DATA

Figure 13. Asynchronous Flash Read

#### Asynchronous Page Mode Read

Table 48 and Figure 14 show asynchronous memory page mode read timing, related to the SMC.

##### Table 48. Asynchronous Page Mode Read

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Switching Characteristics tAV SMC0_Axx (Address) Valid for First Address Minimum Width1 tAV1 SMC0_Axx (Address) Valid for Subsequent SMC0_Ax (Address) Minimum Width tWADV SMC0_NORDV Active Low Width2 tHARE Output3 Hold After SMC0_ARE High4 tWAR 5 SMC0_ARE Active Low Width6, 7 E | (PREST + RST + PREAT + RAT) × tSCLK0 – 2 | ns |
|  | PGWS × tSCLK0 – 2 | ns |
|  | RST × tSCLK0 – 2 | ns |
|  | RHT × tSCLK0 – 2 | ns |
|  | (RAT + (Nw – 1) × PGWS) × tSCLK0 – 2 | ns |

1 PREST, RST, PREAT and RAT values set using the SMC_BxETIM.PREST bits, SMC_BxTIM.RST bits, SMC_BxETIM.PREAT bits, and the SMC_BxTIM.RAT bits.

2 RST value set using the SMC_BxTIM.RST bits.

3 Output signals are SMC0_Ax, SMC0_AMSx, SMC0_AOE.

4 RHT value set using the SMC_BxTIM.RHT bits.

5 SMC_BxCTL.ARDYEN bit = 0.

6 RAT value set using the SMC_BxTIM.RAT bits.

7 Nw = Number of 16-bit data words read.

tAV

tAV1

tAV1

tAV1

A0

A0 + 1

A0 + 2

A0 + 3

tWADV

tWARE

tHARE

D0

D1

D2

D3

READ LATCHED DATA

READ LATCHED DATA

READ LATCHED DATA

READ LATCHED DATA

SMC0_Ax (NOR_Ax)

SMC0_AMSx (NOR_CE)

SMC0_AOE NOR_ADV

SMC0_ARE (NOR_OE)

SMC0_Dx (NOR_Dx)

Figure 14. Asynchronous Page Mode Read

#### Asynchronous Write

Table 49 and Figure 15 show asynchronous memory write timing, related to the SMC.

##### Table 49. Asynchronous Memory Write

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirement tDARDYAW 1 SMC0_ARDY Valid After SMC0_AWE Low 2 E | (WAT – 2.5) × tSCLK0 – 17.5 |  | ns |
| Switching Characteristics |  |  |  |
| tENDAT DATA Enable After SMC0_AMSx Assertion | –3.5 |  | ns |
| tDDAT DATA Disable After SMC0_AMSx Deassertion |  | 2.5 | ns |
| tAMSAWE ADDR/SMC0_AMSx Assertion Before SMC0_AWE Low3 | (PREST + WST + PREAT) × tSCLK0 – 2 |  | ns |
| tHAWE Output4 Hold After SMC0_AWE High5 | WHT × tSCLK0 – 3.5 |  | ns |
| tWAWE6 SMC0_AWE Active Low Width2 | WAT × tSCLK0 – 2 |  | ns |
| tDAWEARDY1 SMC0_AWE High Delay After SMC0_ARDY Assertion | 2.5 × tSCLK0 | 3.5 × tSCLK0 + 17.5 | ns |

1 SMC_BxCTL.ARDYEN bit = 1.

2 WAT value set using the SMC_BxTIM.WAT bits.

3 PREST, WST, PREAT values set using the SMC_BxETIM.PREST bits, SMC_BxTIM.WST bits, SMC_BxETIM.PREAT bits, and the SMC_BxTIM.RAT bits.

4 Output signals are DATA, SMC0_Ax, SMC0_AMSx, SMC0_ABEx.

5 WHT value set using the SMC_BxTIM.WHT bits.

6 SMC_BxCTL.ARDYEN bit = 0.

tAMSAWE

tWAWE

tHAWE

tDARDYAWE

tDAWEARDY

tENDAT

tDDAT

SMC0_AWE

SMC0_ABEx SMC0_Ax (ADDRESS)

SMC0_ARDY

SMC0_AMSx

SMC0_Dx (DATA)

Figure 15. Asynchronous Write

#### SMC Write Cycle Timing With Reference to SYS_CLKOUT

The following SMC specifications (Table 50 and Figure 16) with respect to SYS_CLKOUT are given to accommodate the connection of the SMC to programmable logic devices. These specifications assume that SYS_CLKOUT is outputting a buffered version of SCLK0 by setting CGU_CLKOUTSEL.CLKOUTSEL = 0x3.

##### Table 50. SMC Write Cycle Timing With Reference to SYS_CLKOUT (BxMODE = b#00)

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tSARDY SMC0_ARDY Setup Before SYS_CLKOUT | 14.4 |  | ns |
| tHARDY SMC0_ARDY Hold After SYS_CLKOUT | 0.7 |  | ns |
| Switching Characteristics |  |  |  |
| tDDAT SMC0_Dx Disable After SYS_CLKOUT |  | 7 | ns |
| tENDAT SMC0_Dx Enable After SYS_CLKOUT | –2.5 |  | ns |
| tDO Output Delay After SYS_CLKOUT1 |  | 7 | ns |
| tHO Output Hold After SYS_CLKOUT 1 | –2.5 |  | ns |

1 Output pins/balls include SMC0_AMSx, SMC0_ABEx, SMC0_Ax, SMC0_Dx, SMC0_AOE, and SMC0_AWE.

SETUP CYCLES

PROGRAMMED WRITE ACCESS CYCLES

ACCESS EXTEND CYCLE

HOLD CYCLE

SYS_CLKOUT

tDO

tHO

SMC0_AMSx

SMC0_ABEx SMC0_Ax

tDO

tHO

SMC0_AWE

tSARDY

tHARDY

SMC0_ARDY

tENDAT

tSARDY

tHARDY

tDDAT

SMC0_Dx

Figure 16. SMC Write Cycle Timing With Reference to SYS_CLKOUT Timing

#### Asynchronous Flash Write

Table 51 and Figure 17 show asynchronous flash memory write timing, related to the SMC.

##### Table 51. Asynchronous Flash Write

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Switching Characteristics |  |  |
| tAMSADV SMC0_Ax/SMC0_AMSx Assertion Before ADV Low1 | PREST × tSCLK0 – 2 | ns |
| tDADVAWE SMC0_AWE Low Delay From ADV High2 | PREAT × tSCLK0 – 2 | ns |
| tWADV NR_ADV Active Low Width3 | WST × tSCLK0 – 2 | ns |
| tHAWE Output4 Hold After SMC0_AWE High5 | WHT × tSCLK0 – 3.5 | ns |
| tWAWE6 SMC0_AWE Active Low Width7 | WAT × tSCLK0 – 2 | ns |

1 PREST value set using the SMC_BxETIM.PREST bits.

2 PREAT value set using the SMC_BxETIM.PREAT bits.

3 WST value set using the SMC_BxTIM.WST bits.         	 4 Output signals are DATA, SMC0_Ax, SMC0_AMSx, SMC0_ABEx. 5 WHT value set using the SMC_BxTIM.WHT bits.

6 SMC_BxCTL.ARDYEN bit = 0.

7 WAT value set using the SMC_BxTIM.WAT bits.

tAMSADV 	 	tWADV

tDADVAWE

tWAWE	 	tHAWE

NOR_A 25-1 (SMC0_Ax)

NOR_CE

(SMC0_AMSx)

NOR_ADV (SMC0_AOE)

NOR_WE

(SMC0_AWE)

NOR_DQ 15-0 (SMC0_Dx)

#### All Accesses

Figure 17. Asynchronous Flash Write

Table 52 describes timing that applies to all memory accesses, related to the SMC.

##### Table 52. All Accesses

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Switching Characteristic tTURN SMC0_AMSx Inactive Width | (IT + TT) × tSCLK0 – 2 | ns |

DDR2 SDRAM Clock and Control Cycle Timing

Table 53 and Figure 18 show DDR2 SDRAM clock and control cycle timing, related to the DMC.

##### Table 53. DDR2 SDRAM Clock and Control Cycle Timing, VDD_DMC, Nominal 1.8 V1

| Parameter | 400 MHz2 |  | Unit |
| --- | --- | --- | --- |
|  | Min | Max |  |
| Switching Characteristics |  |  |  |
| tCK Clock Cycle Time (CL = 2 Not Supported) | 2.5 |  | ns |
| tCH (abs)3 Minimum Clock Pulse Width | 0.44 | 0.56 | tCK |
| tCL (abs)3 Maximum Clock Pulse Width | 0.44 | 0.56 | tCK |
| tIS Control/Address Setup Relative to DMCx_CK Rise | 175 |  | ps |
| tIH Control/Address Hold Relative to DMCx_CK Rise | 250 |  | ps |

1 Specifications apply to both DMC0 and DMC1.

2 In order to ensure proper operation of the DDR2, all the DDR2 requirements must be strictly followed. See Interfacing DDR3/DDR2/LPDDR Memory to ADSP-SC5xx/215xx Processors (EE-387).

3 As per JESD79-2E definition.

tCK

tCH

tCL

tIS

tIH

DMCx_CK

DMCx_CK

DMCx_Ax DMCx CONTROL

NOTE: CONTROL = DMCx_CS0, DMCx_CKE, DMCx_RAS, DMCx_CAS, AND DMCx_WE. ADDRESS = DMCx_A0-A15 AND DMCx_BA0-BA2.

Figure 18. DDR2 SDRAM Clock and Control Cycle Timing

#### DDR2 SDRAM Read Cycle Timing

Table 54 and Figure 19 show DDR2 SDRAM read cycle timing, related to the DMC.

##### Table 54. DDR2 SDRAM Read Cycle Timing, VDD_DMC, Nominal 1.8 V1

| Parameter |  | 400 MHz2 |  | Unit |
| --- | --- | --- | --- | --- |
|  |  | Min | Max |  |
| Timing Requirements |  |  |  |  |
| tDQSQ | DMCx_DQS to DMCx_DQ Skew for DMCx_DQS and Associated |  | 0.2 | ns |
|  | DMCx_DQxx Signals |  |  |  |
| tQH | DMCx_DQxx, DMCx_DQS Output Hold Time From DMCx_DQS | 0.8 |  | ns |
| tRPRE | Read Preamble | 0.9 |  | tCK |
| tRPST | Read Postamble | 0.4 |  | tCK |

1 Specifications apply to both DMC0 and DMC1.

2 In order to ensure proper operation of the DDR2, all the DDR2 requirements must be strictly followed. See Interfacing DDR3/DDR2/LPDDR Memory to ADSP-SC5xx/215xx Processors (EE-387).

tCK

tCH		 	tCL

DMCx_CKx

DMCx_CKx

DMCx_Ax DMCx CONTROL

tRPRE

tDQSQ

tDQSQ

tRPST

tQH

tQH

DMCx_LDQS/DMCx_UDQS

DMCx_LDQS/DMCx_UDQS

DMCx_DQxx

NOTE: CONTROL = DMCx_CS0, DMCx_CKE, DMCx_RAS, DMCx_CAS, AND DMCx_WE. ADDRESS = DMCx_A00-13 AND DMCx_BA0-1.

Figure 19. DDR2 SDRAM Controller Input AC Timing

#### DDR2 SDRAM Write Cycle Timing

Table 55 and Figure 20 show DDR2 SDRAM write cycle timing, related to the DMC.

##### Table 55. DDR2 SDRAM Write Cycle Timing, VDD_DMC, Nominal 1.8 V1

| Parameter | 400 MHz2 |  | Unit |
| --- | --- | --- | --- |
|  | Min | Max |  |
| Switching Characteristics |  |  |  |
| tDQSS DMCx_DQS Latching Rising Transitions to Associated Clock Edges3 | –0.15 | +0.15 | tCK |
| tDS Last Data Valid to DMCx_DQS Delay | 0.1 |  | ns |
| tDH DMCx_DQS to First Data Invalid Delay | 0.15 |  | ns |
| tDSS DMCx_DQS Falling Edge to Clock Setup Time | 0.2 |  | tCK |
| tDSH DMCx_DQS Falling Edge Hold Time From DMCx_CK | 0.2 |  | tCK |
| tDQSH DMCx_DQS Input High Pulse Width | 0.35 |  | tCK |
| tDQSL DMCx_DQS Input Low Pulse Width | 0.35 |  | tCK |
| tWPRE Write Preamble | 0.35 |  | tCK |
| tWPST Write Postamble | 0.4 |  | tCK |
| tIPW Address and Control Output Pulse Width | 0.6 |  | tCK |
| tDIPW DMCx_DQ and DMCx_DM Output Pulse Width | 0.35 |  | tCK |

1 Specifications apply to both DMC0 and DMC1.

2 To ensure proper operation of the DDR2, all the DDR2 requirements must be strictly followed. See Interfacing DDR3/DDR2/LPDDR Memory to ADSP-SC5xx/215xx Processors (EE-387).

3 Write command to first DMCx_DQS delay = WL × tCK + tDQSS.

tIPW

tDSH	 tDSS

tDQSS

tWPRE

tDQSL

tDS tDH

tDQSH

tDIPW

tWPST

DMCx_CK

DMCx_CK

DMCx_Ax DMCx CONTROL

DMCx_LDQS/DMCx_UDQS

DMCx_LDQS/DMCx_UDQS

DMC0_DQSn

DMC0_DQSn

DMCx_LDM DMCx_UDM

DMCx_DQx

NOTE: CONTROL = DMCx_CS0, DMCx_CKE, DMCx_RAS, DMCx_CAS, AND DMCx_WE. ADDRESS = DMCx_A00-13 AND DMCx_BA0-1.

Figure 20. DDR2 SDRAM Controller Output AC Timing

#### Mobile DDR (LPDDR) SDRAM Clock and Control Cycle Timing

Table 56 and Figure 21 show mobile DDR SDRAM clock and control cycle timing, related to the DMC.

##### Table 56. Mobile DDR SDRAM Clock and Control Cycle Timing, VDD_DMC, Nominal 1.8 V1

| Parameter | 200 MHz2 |  | Unit |
| --- | --- | --- | --- |
|  | Min | Max |  |
| Switching Characteristics |  |  |  |
| tCK Clock Cycle Time (CL = 2 Not Supported) | 5 |  | ns |
| tCH Minimum Clock Pulse Width | 0.45 | 0.55 | tCK |
| tCL Maximum Clock Pulse Width | 0.45 | 0.55 | tCK |
| tIS Control/Address Setup Relative to DMCx_CK Rise | 1 |  | ns |
| tIH Control/Address Hold Relative to DMCx_CK Rise | 1 |  | ns |

1 Specifications apply to both DMC0 and DMC1.

2 To ensure proper operation of LPDDR, all the LPDDR requirements must be strictly followed. See Interfacing DDR3/DDR2/LPDDR Memory to ADSP-SC5xx/215xx Processors (EE-387).

tCK

tCH

tCL

tIS

tIH

DMCx_CK

DMCx_CK

DMCx_Ax DMCx CONTROL

NOTE: CONTROL = DMCx_CS0, DMCx_CKE, DMCx_RAS, DMCx_CAS, AND DMCx_WE. ADDRESS = DMCx_A0-A15 AND DMCx_BA0-BA2.

Figure 21. Mobile DDR SDRAM Clock and Control Cycle Timing

#### Mobile DDR SDRAM Read Cycle Timing

Table 57 and Figure 22 show mobile DDR SDRAM read cycle timing, related to the DMC.

##### Table 57. Mobile DDR SDRAM Read Cycle Timing, VDD_DMC, Nominal 1.8 V1

| Parameter |  | 200 MHz2 |  | Unit |
| --- | --- | --- | --- | --- |
|  |  | Min | Max |  |
| Timing Requirements |  |  |  |  |
| tQH | DMCx_DQ, DMCx_DQS Output Hold Time From DMCx_DQS | 1.75 |  | ns |
| tDQSQ | DMCx_DQS to DMCx_DQ Skew for DMCx_DQS and Associated |  | 0.4 | ns |
|  | DMCx_DQ Signals |  |  |  |
| tRPRE | Read Preamble | 0.9 | 1.1 | tCK |
| tRPST | Read Postamble | 0.4 | 0.6 | tCK |

1 Specifications apply to both DMC0 and DMC1.

2 To ensure proper operation of LPDDR, all the LPDDR requirements must be strictly followed. See Interfacing DDR3/DDR2/LPDDR Memory to ADSP-SC5xx/215xx Processors (EE-387).

DMCx_CK

tRPRE

tRPST

tQH

Dn

Dn+1

Dn+2

Dn+3

tDQSQ

DMCx_LDQS/DMCx_HDQS

DMCx_DQx

(DATA)

Figure 22. Mobile DDR SDRAM Controller Input AC Timing

#### Mobile DDR SDRAM Write Cycle Timing

Table 58 and Figure 23 show mobile DDR SDRAM write cycle timing, related to the DMC.

##### Table 58. Mobile DDR SDRAM Write Cycle Timing, VDD_DMC, Nominal 1.8 V1

| Parameter | 200 MHz2 |  | Unit |
| --- | --- | --- | --- |
|  | Min | Max |  |
| Switching Characteristics |  |  |  |
| tDQ 3 DMCx_DQS Latching Rising Transitions to Associated Clock Edges SS | 0.75 | 1.25 | tCK |
| tDS Last Data Valid to DMCx_DQS Delay (Slew > 1 V/ns) | 0.48 |  | ns |
| tDH DMCx_DQS to First Data Invalid Delay (Slew > 1 V/ns) | 0.48 |  | ns |
| tDSS DMCx_DQS Falling Edge to Clock Setup Time | 0.2 |  | tCK |
| tDSH DMCx_DQS Falling Edge Hold Time From DMCx_CK | 0.2 |  | tCK |
| tDQSH DMCx_DQS Input High Pulse Width | 0.4 |  | tCK |
| tDQSL DMCx_DQS Input Low Pulse Width | 0.4 |  | tCK |
| tWPRE Write Preamble | 0.25 |  | tCK |
| tWPST Write Postamble | 0.4 |  | tCK |
| tIPW Address and Control Output Pulse Width | 2.3 |  | ns |
| tDIPW DMCx_DQ and DMCx_DM Output Pulse Width | 1.8 |  | ns |

1 Specifications apply to both DMC0 and DMC1.

2 To ensure proper operation of LPDDR, all the LPDDR requirements must be strictly followed. See Interfacing DDR3/DDR2/LPDDR Memory to ADSP-SC5xx/215xx Processors (EE-387).

3 Write command to first DMCx_DQS delay = WL × tCK + tDQSS.

DMCx_CK

DMCx_LDQS/DMCx_HDQS

tDQSS

tWPRE

tDSH	tDSS

tDS	tDH tDIPW

tDQSL

tDQSH

tWPST

DMCx_DQ0-15/ DMCx_LDQM/DMCx_HDQM

Dn	Dn+1

Dn+2

tDIPW

Dn+3

DMCx CONTROL

Write CMD

tIPW

NOTE: CONTROL = DMCx_CSx, DMCx_CKE, DMCx_RAS, DMCx_CAS, AND DMCx_WE. ADDRESS = DMCx_A00-13, AND DMCx_BA0-1.

Figure 23. Mobile DDR SDRAM Controller Output AC Timing

#### DDR3 SDRAM Clock and Control Cycle Timing

Table 59 and Figure 24 show mobile DDR3 SDRAM clock and control cycle timing, related to the DMC.

##### Table 59. DDR3 SDRAM Clock and Control Cycle Timing, VDD_DMC, Nominal 1.5 V1

| Parameter | 450 MHz2 |  | Unit |
| --- | --- | --- | --- |
|  | Min | Max |  |
| Switching Characteristics |  |  |  |
| tCK Clock Cycle Time (CL = 2 Not Supported) | 2.22 |  | ns |
| tCH(abs)3 Minimum Clock Pulse Width | 0.43 | 0.57 | tCK |
| tCL(abs)3 Maximum Clock Pulse Width | 0.43 | 0.57 | tCK |
| tIS Control/Address Setup Relative to DMCx_CK Rise | 0.2 |  | ns |
| tIH Control/Address Hold Relative to DMCx_CK Rise | 0.275 |  | ns |

1 Specifications apply to both DMC0 and DMC1.

2 To ensure proper operation of the DDR3, all the DDR3 requirements must be strictly followed. See Interfacing DDR3/DDR2/LPDDR Memory to ADSP-SC5xx/215xx Processors (EE-387).

3 As per JESD79-3F definition.

tCK

tCH

tCL

tIS

tIH

DMCx_CK

DMCx_CK

DMCx_Ax DMCx CONTROL

NOTE: CONTROL = DMCx_CS0, DMCx_CKE, DMCx_RAS, DMCx_CAS, AND DMCx_WE. ADDRESS = DMCx_A0-A15 AND DMCx_BA0-BA2.

Figure 24. DDR3 SDRAM Clock and Control Cycle Timing

#### DDR3 SDRAM Read Cycle Timing

Table 60 and Figure 25 show mobile DDR3 SDRAM read cycle timing, related to the DMC.

##### Table 60. DDR3 SDRAM Read Cycle Timing, VDD_DMC, Nominal 1.5 V1

| Parameter | 450 MHz2 |  | Unit |
| --- | --- | --- | --- |
|  | Min | Max |  |
| Timing Requirements |  |  |  |
| tDQSQ DMCx_DQS to DMCx_DQ Skew for DMCx_DQS and Associated DMCx_DQ |  | 0.15 | ns |
| Signals |  |  |  |
| tQH DMCx_DQ, DMCx_DQS Output Hold Time From DMCx_DQS | 0.38 |  | tCK |
| tRPRE Read Preamble | 0.9 |  | tCK |
| tRPST Read Postamble | 0.3 |  | tCK |

1 Specifications apply to both DMC0 and DMC1.

2 To ensure proper operation of the DDR3, all the DDR3 requirements must be strictly followed. See Interfacing DDR3/DDR2/LPDDR Memory to ADSP-SC5xx/215xx Processors (EE-387).

tCK

tCH		 	tCL

DMCx_CKx

DMCx_CKx

DMCx_Ax DMCx CONTROL

tRPRE

tDQSQ

tDQSQ

tRPST

tQH

tQH

DMCx_LDQS/DMCx_UDQS

DMCx_LDQS/DMCx_UDQS

DMCx_DQxx

NOTE: CONTROL = DMCx_CS0, DMCx_CKE, DMCx_RAS, DMCx_CAS, AND DMCx_WE. ADDRESS = DMCx_A00-13 AND DMCx_BA0-1.

Figure 25. DDR3 SDRAM Controller Input AC Timing

#### DDR3 SDRAM Write Cycle Timing

Table 61 and Figure 26 show mobile DDR3 SDRAM output ac timing, related to the DMC.

##### Table 61. DDR3 SDRAM Write Cycle Timing, VDD_DMC, Nominal 1.5 V1

| Parameter | 450 MHz2 |  | Unit |
| --- | --- | --- | --- |
|  | Min | Max |  |
| Switching Characteristics |  |  |  |
| tDQSS DMCx_DQS Latching Rising Transitions to Associated Clock Edges3 | –0.25 | 0.25 | tCK |
| tDS Last Data Valid to DMCx_DQS Delay (Slew > 1 V/ns) | 0.125 |  | ns |
| tDH DMCx_DQS to First Data Invalid Delay (Slew > 1 V/ns) | 0.150 |  | ns |
| tDSS DMCx_DQS Falling Edge to Clock Setup Time | 0.2 |  | tCK |
| tDSH DMCx_DQS Falling Edge Hold Time From DMCx_CK | 0.2 |  | tCK |
| tDQSH DMCx_DQS Input High Pulse Width | 0.45 | 0.55 | tCK |
| tDQSL DMCx_DQS Input Low Pulse Width | 0.45 | 0.55 | tCK |
| tWPRE Write Preamble | 0.9 |  | tCK |
| tWPST Write Postamble | 0.3 |  | tCK |
| tIPW Address and Control Output Pulse Width | 0.840 |  | ns |
| tDIPW DMCx_DQ and DMCx_DM Output Pulse Width | 0.550 |  | ns |

1 Specifications apply to both DMC0 and DMC1.

2 To ensure proper operation of the DDR3, all the DDR3 requirements must be strictly followed. See Interfacing DDR3/DDR2/LPDDR Memory to ADSP-SC5xx/215xx Processors (EE-387).

3 Write command to first DMCx_DQS delay = WL × tCK + tDQSS.

tIPW

tDSH

tDSS

tDQSS

tWPRE

tDQSL

tDQSH

tWPST

tDS	tDH

tDIPW

DMCx_CK

DMCx_CK

DMCx_Ax DMCx CONTROL

DMCx_LDQS/DMCx_UDQS

DMCx_LDQS/DMCx_UDQS

DMC0_DQSn

DMC0_DQSn

DMCx_LDM DMCx_UDM

DMCx_DQx

NOTE: CONTROL = DMCx_CS0, DMCx_CKE, DMCx_RAS, DMCx_CAS, AND DMCx_WE. ADDRESS = DMCx_A00-13, AND DMCx_BA0-1.

Figure 26. DDR3 SDRAM Controller Output AC Timing

#### Enhanced Parallel Peripheral Interface (EPPI) Timing

Table 62 and Table 63 and Figure 27 through Figure 35 describe enhanced parallel peripheral interface (EPPI) timing operations. In Figure 27 through Figure 35, POLC[1:0] represents the setting of the EPPI_CTL register, which sets the sampling/driving edges of the EPPI clock.

When internally generated, the programmed PPI clock (fPCLKPROG) frequency in MHz is set by the following equation where VALUE is a field in the EPPI_CLKDIV register that can be set from 0 to 65,535:

fPCLKPROG =

f

SCLK0

--V---A--L--U---E---+ 	1--

tPCLKPROG  =  --------1--------

fPCLKPROG

When externally generated, the EPPI_CLK is called fPCLKEXT:

tPCLKEXT  =  ------1-------

fPCLKEXT

##### Table 62. Enhanced Parallel Peripheral Interface (EPPI)—Internal Clock

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tSFSPI External FS Setup Before EPPI_CLK | 6.5 |  | ns |
| tHFSPI External FS Hold After EPPI_CLK | 0 |  | ns |
| tSDRPI Receive Data Setup Before EPPI_CLK | 6.5 |  | ns |
| tHDRPI Receive Data Hold After EPPI_CLK | 0 |  | ns |
| tSFS3GI External FS3 Input Setup Before EPPI_CLK Fall Edge in Clock | 14 |  | ns |
| Gating Mode |  |  |  |
| tHFS3GI External FS3 Input Hold Before EPPI_CLK Fall Edge in Clock | 0 |  | ns |
| Gating Mode |  |  |  |
| Switching Characteristics |  |  |  |
| tPCLKW EPPI_CLK Width1 | 0.5 × tPCLKPROG – 1.5 |  | ns |
| tPCLK EPPI_CLK Period1 | tPCLKPROG – 1.5 |  | ns |
| tDFSPI Internal FS Delay After EPPI_CLK |  | 3.5 | ns |
| tHOFSPI Internal FS Hold After EPPI_CLK | –0.5 |  | ns |
| tDDTPI Transmit Data Delay After EPPI_CLK |  | 3.5 | ns |
| tHDTPI Transmit Data Hold After EPPI_CLK | –0.5 |  | ns |

1 See Table 29 for details on the minimum period that can be programmed for tPCLKPROG.

POLC[1:0] = 10

EPPI_CLK

POLC[1:0] = 01

FRAME SYNC DRIVEN

DATA SAMPLED

tDFSPI

tHOFSPI

tPCLKW

tPCLK

tSDRPI

tHDRPI

EPPI_FS1/2

EPPI_D00-23

Figure 27. EPPI Internal Clock GP Receive Mode with Internal Frame Sync Timing

tPCLK

tDFSPI

tPCLKW

tHOFSPI

tDDTPI

tHDTPI

FRAME SYNC DRIVEN

DATA DRIVEN

DATA DRIVEN

POLC[1:0] = 11

EPPI_CLK

POLC[1:0] = 00

EPPI_FS1/2

EPPI_D00-23

Figure 28. EPPI Internal Clock GP Transmit Mode with Internal Frame Sync Timing

POLC[1:0] = 11

EPPI_CLK

POLC[1:0] = 00

DATA SAMPLED / FRAME SYNC SAMPLED

DATA SAMPLED / FRAME SYNC SAMPLED

tSFSPI

tHFSPI

tPCLKW

tPCLK

tSDRPI

tHDRPI

EPPI_FS1/2

EPPI_D00-23

Figure 29. EPPI Internal Clock GP Receive Mode with External Frame Sync Timing

POLC[1:0] = 11

EPPI_CLK

POLC[1:0] = 00

DATA DRIVEN / FRAME SYNC SAMPLED

tSFSPI	tHFSPI

tPCLKW

tPCLK

tDDTPI tHDTPI

EPPI_FS1/2

EPPI_D00-23

Figure 30. EPPI Internal Clock GP Transmit Mode with External Frame Sync Timing

tHFS3GI

tSFS3GI

EPPI_CLK

EPPI_FS3

Figure 31. Clock Gating Mode with Internal Clock and External Frame Sync Timing

##### Table 63. Enhanced Parallel Peripheral Interface (EPPI)—External Clock

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tPCLKW EPPI_CLK Width1 | 0.5 × tPCLKEXT – 0.5 |  | ns |
| tPCLK EPPI_CLK Period1 | tPCLKEXT – 1 |  | ns |
| tSFSPE External FS Setup Before EPPI_CLK | 2 |  | ns |
| tHFSPE External FS Hold After EPPI_CLK | 3.7 |  | ns |
| tSDRPE Receive Data Setup Before EPPI_CLK | 2 |  | ns |
| tHDRPE Receive Data Hold After EPPI_CLK | 3.7 |  | ns |
| Switching Characteristics |  |  |  |
| tDFSPE Internal FS Delay After EPPI_CLK |  | 15.3 | ns |
| tHOFSPE Internal FS Hold After EPPI_CLK | 2.4 |  | ns |
| tDDTPE Transmit Data Delay After EPPI_CLK |  | 15.3 | ns |
| tHDTPE Transmit Data Hold After EPPI_CLK | 2.4 |  | ns |

1 This specification indicates the minimum instantaneous width or period that can be tolerated due to duty cycle variation or jitter on the external EPPI_CLK. For the external EPPI_CLK ideal maximum frequency see the fPCLKEXT specification in Table 29.

POLC[1:0] = 10

EPPI_CLK

POLC[1:0] = 01

FRAME SYNC DRIVEN

DATA SAMPLED

tDFSPE

tHOFSPE

tPCLKW

tPCLK

tSDRPE

tHDRPE

EPPI_FS1/2

EPPI_D00-23

Figure 32. EPPI External Clock GP Receive Mode with Internal Frame Sync Timing

tPCLK

tDFSPE

tPCLKW

tHOFSPE

tDDTPE

tHDTPE

FRAME SYNC DRIVEN

DATA DRIVEN

DATA DRIVEN

POLC[1:0] = 11

EPPI_CLK

POLC[1:0] = 00

EPPI_FS1/2

EPPI_D00-23

Figure 33. EPPI External Clock GP Transmit Mode with Internal Frame Sync Timing

POLC[1:0] = 11

EPPI_CLK

POLC[1:0] = 00

DATA SAMPLED / FRAME SYNC SAMPLED

DATA SAMPLED / FRAME SYNC SAMPLED

tSFSPE

tHFSPE

tPCLKW

tPCLK

tSDRPE

tHDRPE

EPPI_FS1/2

EPPI_D00-23

Figure 34. EPPI External Clock GP Receive Mode with External Frame Sync Timing

POLC[1:0] = 11

EPPI_CLK

POLC[1:0] = 00

DATA DRIVEN / FRAME SYNC SAMPLED

tSFSPE	tHFSPE

tPCLKW

tPCLK

tDDTPE tHDTPE

EPPI_FS1/2

EPPI_D00-23

Figure 35. EPPI External Clock GP Transmit Mode with External Frame Sync Timing

#### Link Ports (LP)

In LP receive mode, the link port clock is supplied externally and is called fLCLKREXT, therefore the period can be represented by:

tLCLKREXT  =  -------1--------

fLCLKREXT

In link port transmit mode, the programmed link port clock (fLCLKTPROG) frequency in MHz is set by the following equation where VALUE is a field in the LP_DIV register that can be set from 1 to 255:

fLCLKTPROG =

f

CLKO8

--V---A--L--U---E--- 	2--

In the case where VALUE = 0, fLCLKTPROG = fCLKO8. For all settings of VALUE, the following equation is true:

tLCLKTPROG  =  --------1----------

fLCLKTPROG

Calculation of the link receiver data setup and hold relative to the link clock is required to determine the maximum allowable skew that can be introduced in the transmission path length difference between LPx_Dx and LPx_CLK. Setup skew is the maximum delay that can be introduced in LPx_Dx relative to LPx_CLK (setup skew = tLCLKTWH min – tDLDCH – tSLDCL). Hold skew is the maximum delay that can be introduced in LPx_CLK relative to LPx_Dx (hold skew = tLCLKTWL min – tHLDCH – tHLDCL).

##### Table 64. Link Ports—Receive1

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| fLCLKREXT LPx_CLK Frequency |  | 150 | MHz |
| tSLDCL Data Setup Before LPx_CLK Low | 0.9 |  | ns |
| tHLDCL Data Hold After LPx_CLK Low | 1.4 |  | ns |
| tLCLKEW LPx_CLK Period2 | tLCLKREXT – 0.42 |  | ns |
| tLCLKRWL LPx_CLK Width Low2 | 0.5 × tLCLKREXT |  | ns |
| tLCLKRWH LPx_CLK Width High2 | 0.5 × tLCLKREXT |  | ns |
| Switching Characteristic tDLALC LPx_ACK Low Delay After LPx_CLK Low3 | 1.5 × tCLKO8 + 4 | 2.5 × tCLKO8 + 12 | ns |

1 Specifications apply to LP0 and LP1.

2 This specification indicates the minimum instantaneous width or period that can be tolerated due to duty cycle variation or jitter on the external LPx_CLK. For the external LPx_CLK ideal maximum frequency see the fLCLKTEXT specification in Table 29.

3 LPx_ACK goes low with tDLALC relative to rise of LPx_CLK after first byte, but does not go low if the link buffer of the receiver is not about to fill.

tLCLKEW

tLCLKRWH	 	tLCLKRWL

tHLDCL

tSLDCL

IN

tDLALC

LPx_CLK

LPx_D7–0

LPx_ACK (OUT)

Figure 36. Link Ports—Receive

##### Table 65. Link Ports—Transmit1

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements tSLACH LPx_ACK Setup Before LPx_CLK Low tHLACH LPx_ACK Hold After LPx_CLK Low | 2 × tCLKO8 + 13.5 –5.5 |  | ns ns |
| Switching Characteristics |  |  |  |
| tDLDCH Data Delay After LPx_CLK High |  | 1.6 | ns |
| tHLDCH Data Hold After LPx_CLK High | –0.8 |  | ns |
| tLCLKTWL2 LPx_CLK Width Low | 0.33 × tLCLKTPROG | 0.6 × tLCLKTPROG | ns |
| tLCLKTW 2 LPx_CLK Width High H | 0.45 × tLCLKTPROG | 0.66 × tLCLKTPROG | ns |
| tLCLKTW2 LPx_CLK Period | N × tLCLKTPROG – 0.5 |  | ns |
| tDLACLK LPx_CLK Low Delay After LPx_ACK High | tCLKO8 + 4 | 2 × tCLKO8 + 1 × tLPCLK + 10 | ns |

1 Specifications apply to LP0 and LP1.

2 See Table 29 for details on the minimum period that can be programmed for tLCLKTPROG.

tLCLKTWH tLCLKTWL

LAST BYTE TRANSMITTED

TRANSMITTED

FIRST BYTE 1

tDLDCH tHLDCH

OUT

tSLACH

tHLACH	tDLACLK

LPx_CLK

LPx_Dx (DATA)

LPx_ACK (IN)

NOTES

The tSLACH and tHLACH specifications apply only to the LPx_CLK falling edge. If these specifications are met, LPx_CLK would extend and the dotted LPx_CLK falling edge would not occur as shown. The position of the dotted falling edge can be calculated using the tLCLKTWH specification. tLCLKTWH Min should be used for tSLACH

and tLCLKTWH Max for tHLACH.

Figure 37. Link Ports—Transmit

#### Serial Ports (SPORT)

To determine whether a device is compatible with the SPORT at clock speed n, the following specifications must be confirmed: frame sync delay and frame sync setup and hold; data delay and data setup and hold; and serial clock (SPTx_CLK) width. In Figure 38, either the ris- ing edge or the falling edge of SPTx_CLK (external or internal) can be used as the active sampling edge.

When externally generated, the SPORT clock is called fSPTCLKEXT:

tSPTCLKEXT  =  -----------1------------

fSPTCLKEXT

When internally generated, the programmed SPORT clock (fSPTCLKPROG) frequency in MHz is set by the following equation where CLKDIV is a field in the SPORT_DIV register that can be set from 0 to 65,535:

fSPTCLKPROG =

f

SCLK0

--C--L--K--D----I-V---+ 	1--

tSPTCLKPROG  = -------------1--------------

fSPTCLKPROG

##### Table 66. Serial Ports—External Clock1

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tSFSE Frame Sync Setup Before SPTx_CLK | 2 |  | ns |
| (Externally Generated Frame Sync in either Transmit or Receive |  |  |  |
| Mode)2 |  |  |  |
| tHFSE Frame Sync Hold After SPTx_CLK | 2.7 |  | ns |
| (Externally Generated Frame Sync in either Transmit or Receive |  |  |  |
| Mode)2 |  |  |  |
| tSDRE Receive Data Setup Before Receive SPTx_CLK2 | 2 |  | ns |
| tHDRE Receive Data Hold After SPTx_CLK2 | 2.7 |  | ns |
| tSPTCLKW SPTx_CLK Width3 | 0.5 × tSPTCLKEXT – 1.5 |  | ns |
| tSPTCLK SPTx_CLK Period3 | tSPTCLKEXT – 1.5 |  | ns |
| Switching Characteristics |  |  |  |
| tDFSE Frame Sync Delay After SPTx_CLK |  | 14.5 | ns |
| (Internally Generated Frame Sync in either Transmit or Receive |  |  |  |
| Mode)4 |  |  |  |
| tHOFSE Frame Sync Hold After SPTx_CLK | 2 |  | ns |
| (Internally Generated Frame Sync in either Transmit or Receive |  |  |  |
| Mode)4 |  |  |  |
| tDDTE Transmit Data Delay After Transmit SPTx_CLK4 |  | 14 | ns |
| tHDTE Transmit Data Hold After Transmit SPTx_CLK4 | 2 |  | ns |

1 Specifications apply to all eight SPORTs.

2 Referenced to sample edge.

3 This specification indicates the minimum instantaneous width or period that can be tolerated due to duty cycle variation or jitter on the external SPTx_CLK. For the external SPTx_CLK ideal maximum frequency see the fSPTCLKEXT specification in Table 29.

4 Referenced to drive edge.

##### Table 67. Serial Ports—Internal Clock1

| Parameter |  | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Timing Requirements |  |  |  |  |
| tSFSI | Frame Sync Setup Before SPTx_CLK | 12 |  |  |
|  | (Externally Generated Frame Sync in either Transmit or |  |  | ns |
|  | Receive Mode)2 |  |  |  |
| tHFSI | Frame Sync Hold After SPTx_CLK | –0.5 |  |  |
|  | (Externally Generated Frame Sync in either Transmit or |  |  | ns |
|  | Receive Mode)2 |  |  |  |
| tSDRI | Receive Data Setup Before SPTx_CLK2 | 3.4 |  | ns |
| tHDRI | Receive Data Hold After SPTx_CLK2 | 1.5 |  | ns |
| Switching Characteristics |  |  |  |  |
| tDFSI Frame Sync Delay After SPTx_CLK (Internally Generated |  |  | 3.5 | ns |
| Frame Sync in Transmit or Receive Mode)3 |  |  |  |  |
| tHOFSI Frame Sync Hold After SPTx_CLK (Internally Generated |  | –2.5 |  | ns |
| Frame Sync in Transmit or Receive Mode)3 |  |  |  |  |
| tDDTI Transmit Data Delay After SPTx_CLK3 |  |  | 3.5 | ns |
| tHDTI Transmit Data Hold After SPTx_CLK3 |  | –2.5 |  | ns |
| tSCLKIW SPTx_CLK Width4 |  | 0.5 × tSPTCLKPROG – 1.5 |  | ns |
| tSPTCLK SPTx_CLK Period4 |  | tSPTCLKPROG – 1.5 |  | ns |

1 Specifications apply to all eight SPORTs.

2 Referenced to the sample edge.

3 Referenced to drive edge.

4 See Table 29 for details on the minimum period that can be programmed for tSPTCLKPROG.

DATA RECEIVE—INTERNAL CLOCK

tSCLKIW

tDFSI

tHOFSI

tSFSI

tHFSI

tSDRI

tHDRI

DRIVE EDGE	SAMPLE EDGE

DATA RECEIVE—EXTERNAL CLOCK

tSCLKW

tDFSE

tHOFSE

tSFSE

tHFSE

tSDRE

tHDRE

DRIVE EDGE	SAMPLE EDGE

SPTx_A/BCLK (SPORT CLOCK)

SPTx_A/BCLK (SPORT CLOCK)

SPTx_A/BFS (FRAME SYNC)

SPTx_A/BFS (FRAME SYNC)

SPTx_A/BDx (DATA CHANNEL A/B)

SPTx_A/BDx (DATA CHANNEL A/B)

tSCLKIW

tDFSI

tHOFSI

tSFSI

tHFSI

tDDTI

tHDTI

DATA TRANSMIT—INTERNAL CLOCK DRIVE EDGE	SAMPLE EDGE

DATA TRANSMIT—EXTERNAL CLOCK DRIVE EDGE	SAMPLE EDGE

SPTx_A/BCLK (SPORT CLOCK)

SPTx_A/BCLK (SPORT CLOCK)

SPTx_A/BFS (FRAME SYNC)

SPTx_A/BFS (FRAME SYNC)

SPTx_A/BDx (DATA CHANNEL A/B)

SPTx_A/BDx (DATA CHANNEL A/B)

tSCLKW

tDFSE

tHOFSE

tSFSE

tHFSE

tDDTE

tHDTE

Figure 38. Serial Ports

##### Table 68. Serial Ports—Enable and Three-State1

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Switching Characteristics |  |  |  |
| tDDTEN Data Enable from External Transmit SPTx_CLK2 | 1 |  | ns |
| tDDTTE Data Disable from External Transmit SPTx_CLK2 |  | 14 | ns |
| tDDTIN Data Enable from Internal Transmit SPTx_CLK2 | –2.5 |  | ns |
| tDDTTI Data Disable from Internal Transmit SPTx_CLK2 |  | 2.8 | ns |

1 Specifications apply to all eight SPORTs.

2 Referenced to drive edge.

tDDTEN

tDDTTE

DRIVE EDGE	DRIVE EDGE

SPTx_CLK (SPORT CLOCK EXTERNAL)

SPTx_A/BDx (DATA CHANNEL A/B)

tDDTIN

tDDTTI

DRIVE EDGE	DRIVE EDGE

SPTx_CLK (SPORT CLOCK INTERNAL)

SPTx_A/BDx (DATA CHANNEL A/B)

Figure 39. Serial Ports—Enable and Three-State

The SPTx_TDV output signal becomes active in SPORT multichannel mode. During transmit slots (enabled with active channel selection registers) the SPTx_TDV is asserted for communication with external devices.

##### Table 69. Serial Ports—TDV (Transmit Data Valid)1

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Switching Characteristics |  |  |  |
| tDRDVEN Data Valid Enable Delay from Drive Edge of External Clock2 | 2 |  | ns |
| tDFDVEN Data Valid Disable Delay from Drive Edge of External Clock2 |  | 14 | ns |
| tDRDVIN Data Valid Enable Delay from Drive Edge of Internal Clock2 | –2.5 |  | ns |
| tDFDVIN Data Valid Disable Delay from Drive Edge of Internal Clock2 |  | 3.5 | ns |

1 Specifications apply to all eight SPORTs.

2 Referenced to drive edge.

LK L)

tDRDVEN

tDFDVEN

DV

DRIVE EDGE	DRIVE EDGE

SPTx_C (SPORT CLOCK EXTERNA

SPTx_A/BT

tDRDVIN

tDFDVIN

DRIVE EDGE	DRIVE EDGE

SPTx_CLK (SPORT CLOCK INTERNAL)

SPTx_A/BTDV

Figure 40. Serial Ports—Transmit Data Valid Internal and External Clock

##### Table 70. Serial Ports—External Late Frame Sync1

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Switching Characteristics |  |  |  |
| tDDTLFSE Data Delay from Late External Transmit Frame Sync or External Receive Frame |  | 14 | ns |
| Sync with MCE = 1, MFD = 02 |  |  |  |
| tDDTENFS Data Enable for MCE = 1, MFD = 02 | 0.5 |  | ns |

1 Specifications apply to all eight SPORTs.

2 The tDDTLFSE and tDDTENFS parameters apply to left justified as well as standard serial mode and MCE = 1, MFD = 0.

SPTx_A/BCLK (SPORT CLOCK)

DRIVE	SAMPLE

DRIVE

SPTx_A/BFS (FRAME SYNC)

SPTx_A/BDx (DATA CHANNEL A/B)

tHFSE/I

tSFSE/I

tDDTE/I

tDDTENFS

tHDTE/I

1ST BIT

2ND BIT

tDDTLFSE

Figure 41. External Late Frame Sync

#### Sample Rate Converter—Serial Input Port

The ASRC input signals are routed from the DAIx_PINx pins using the SRU. Therefore, the timing specifications provided in Table 71 are valid at the DAIx_PINx pins.

##### Table 71. ASRC, Serial Input Port

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Timing Requirements |  |  |
| tSRCSFS1 Frame Sync Setup Before Serial Clock Rising Edge | 4 | ns |
| tSRCH 1 Frame Sync Hold After Serial Clock Rising Edge FS | 5.5 | ns |
| tSRCSD1 Data Setup Before Serial Clock Rising Edge | 4 | ns |
| tSRCHD1 Data Hold After Serial Clock Rising Edge | 5.5 | ns |
| tSRCCLKW Clock Width | tSCLK0 – 1 | ns |
| tSRCCLK Clock Period | 2 × tSCLK0 | ns |

1 The serial clock, data, and frame sync signals can come from any of the DAI pins. The serial clock and frame sync signals can also come via PCG or SPORTs. The input of the PCG can be either CLKIN or any of the DAI pins.

SAMPLE EDGE

tSRCCLK

tSRCCLKW

tSRCSFS

tSRCHFS

tSRCSD

tSRCHD

DAIx_PIN20–1 (SCLK)

DAIx_PIN20–1 (FS)

DAIX_PIN20–1 (SDATA)

Figure 42. ASRC Serial Input Port Timing

#### Sample Rate Converter—Serial Output Port

For the serial output port, the frame sync is an input and it must meet setup and hold times with regard to SCLK on the output port. The serial data output has a hold time and delay specification with regard to serial clock. The serial clock rising edge is the sampling edge, and the falling edge is the drive edge.

##### Table 72. ASRC, Serial Output Port

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tSRCSFS1 Frame Sync Setup Before Serial Clock Rising Edge | 4 |  | ns |
| tSRCH 1 Frame Sync Hold After Serial Clock Rising Edge FS | 5.5 |  | ns |
| tSRCCLKW Clock Width | tSCLK0 – 1 |  | ns |
| tSRCCLK Clock Period | 2 × tSCLK0 |  | ns |
| Switching Characteristics |  |  |  |
| tSRCT 1 Transmit Data Delay After Serial Clock Falling Edge DD |  | 13 | ns |
| tSRCT 1 Transmit Data Hold After Serial Clock Falling Edge DH | 1 |  | ns |

1 The serial clock, data, and frame sync signals can come from any of the DAI pins. The serial clock and frame sync signals can also come via PCG or SPORTs. The input of the PCG can be either CLKIN, SCLK0, or any of the DAI pins.

SAMPLE EDGE

tSRCCLK

tSRCCLKW

tSRCSFS	 tSRCHFS

tSRCTDD

tSRCTDH

DAIx_PIN20–1 (SCLK)

DAIx_PIN20–1 (FS)

DAIx_PIN20–1 (SDATA)

Figure 43. ASRC Serial Output Port Timing

#### SPI Port—Master Timing

Table 73 and Figure 44 describe SPI port master operations.

When internally generated, the programmed SPI clock (fSPICLKPROG) frequency in MHz is set by the following equation where BAUD is a field in the SPIx_CLK register that can be set from 0 to 65,535:

fSPICLKPROG =

f

SCLK1

--B--A---U--D-----+---1---

tSPICLKPROG  = -------------1-------------

fSPICLKPROG

Note that

  - In dual-mode data transmit, the SPIx_MISO signal is an output.

  - In quad-mode data transmit, the SPIx_MISO, SPIx_D2, and SPIx_D3 signals are outputs.

  - In dual-mode data receive, the SPIx_MOSI signal is an input.

  - In quad-mode data receive, the SPIx_MOSI, SPIx_D2, and SPIx_D3 signals are inputs.

  - Quad-mode is supported by SPI2 only.

  - CPHA is a configuration bit in the SPI_CTL register.

##### Table 73. SPI Port—Master Timing1

| Parameter | Min Max |  | Unit |
| --- | --- | --- | --- |
| Timing Requirements tSSPIDM Data Input Valid to SPIx_CLK Edge (Data Input Setup) tHSPIDM SPIx_CLK Sampling Edge to Data Input Invalid | 3.2 1.2 |  | ns ns |
| Switching Characteristics |  |  |  |
| tSDSCIM SPIx_SEL Low to First SPI_CLK Edge for CPHA = 1 | tSCLK1 – 2 |  | ns |
| SPIx_SEL Low to First SPI_CLK Edge for CPHA = 0 | 1.5 × tSCLK1 – 2 |  | ns |
| tSPICHM SPIx_CLK High Period2 | 0.5 × tSPICLKPROG – 1 |  | ns |
| tSPICLM SPIx_CLK Low Period2 | 0.5 × tSPICLKPROG – 1 |  | ns |
| tSPICLK SPIx_CLK Period2 | tSPICLKPROG – 1 |  | ns |
| tHDSM Last SPIx_CLK Edge to SPIx_SEL High for CPHA = 1 | 1.5 × tSCLK1 – 2 |  | ns |
| Last SPIx_CLK Edge to SPIx_SEL High for CPHA = 0 | tSCLK1 – 2 |  | ns |
| tSPITDM Sequential Transfer Delay3 | tSCLK1 – 1 |  | ns |
| tDDSPIDM SPIx_CLK Edge to Data Out Valid (Data Out Delay) |  | 2.6 | ns |
| tHDSPIDM SPIx_CLK Edge to Data Out Invalid (Data Out Hold) | –1.5 |  | ns |

1 All specifications apply to all three SPIs.

2 See Table 29 for details on the minimum period that can be programmed for tSPICLKPROG.

3 Applies to sequential mode with STOP ≥ 1.

tSDSCIM	 tSPICLM	 tSPICHM

tSPICLK

tHDSM

tSPITDM

tHDSPIDM

tDDSPIDM

tSSPIDM

tHSPIDM

tHDSPIDM

tDDSPIDM

tSSPIDM

tHSPIDM

SPIx_SEL

(OUTPUT)

SPIx_CLK (OUTPUT)

DATA OUTPUTS (SPIx_MOSI)

CPHA = 1

DATA INPUTS (SPIx_MISO)

DATA OUTPUTS (SPIx_MOSI)

CPHA = 0

DATA INPUTS (SPIx_MISO)

Figure 44. SPI Port—Master Timing

#### SPI Port—Slave Timing

Table 74 and Figure 45 describe SPI port slave operations. Note that

  - In dual-mode data transmit, the SPIx_MOSI signal is an output.

  - In quad-mode data transmit, the SPIx_MOSI, SPIx_D2, and SPIx_D3 signals are outputs.

  - In dual-mode data receive, the SPIx_MISO signal is an input.

  - In quad-mode data receive, the SPIx_MISO, SPIx_D2, and SPIx_D3 signals are inputs.

  - In SPI slave mode, the SPI clock is supplied externally and is called fSPICLKEXT, as follows:

tSPICLKEXT  = -----------1-----------

fSPICLKEXT

  - Quad mode is supported by SPI2 only.

  - CPHA is a configuration bit in the SPI_CTL register.

##### Table 74. SPI Port—Slave Timing1

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tSPICHS SPIx_CLK High Period2 | 0.5 × tSPICLKEXT – 1 |  | ns |
| tSPICLS SPIx_CLK Low Period2 | 0.5 × tSPICLKEXT – 1 |  | ns |
| tSPICLK SPIx_CLK Period2 | tSPICLKEXT – 1 |  | ns |
| tHDS Last SPIx_CLK Edge to SPIx_SS Not Asserted | 5 |  | ns |
| tSPITDS Sequential Transfer Delay | tSPICLK – 1 |  | ns |
| tSDSCI SPIx_SS Assertion to First SPIx_CLK Edge | 10.5 |  | ns |
| tSSPID Data Input Valid to SPIx_CLK Edge (Data Input Setup) | 2 |  | ns |
| tHSPID SPIx_CLK Sampling Edge to Data Input Invalid | 1.6 |  | ns |
| Switching Characteristics |  |  |  |
| tDSOE SPIx_SS Assertion to Data Out Active | 0 | 14 | ns |
| tDSDHI SPIx_SS Deassertion to Data High Impedance | 0 | 12.5 | ns |
| tDDSPID SPIx_CLK Edge to Data Out Valid (Data Out Delay) |  | 14 | ns |
| tHDSPID SPIx_CLK Edge to Data Out Invalid (Data Out Hold) | 0 |  | ns |

1 All specifications apply to all three SPIs.

2 This specification indicates the minimum instantaneous width or period that can be tolerated due to duty cycle variation or jitter on the external SPIx_CLK. For the external SPIx_CLK ideal maximum frequency, see the fSPICLKTEXT specification in Table 29.

tSDSCI	 tSPICLS	 tSPICHS

tSPICLK

tHDS

tSPITDS

tDSOE

tDDSPID

tHDSPID

tDDSPID

tDSDHI

tSSPID

tHSPID

tDSOE

tHDSPID

tDDSPID

tDSDHI

tHSPID

tSSPID

SPIx_SS

(INPUT)

SPIx_CLK (INPUT)

DATA OUTPUTS (SPIx_MISO)

CPHA = 1

DATA INPUTS (SPIx_MOSI)

DATA OUTPUTS (SPIx_MISO)

CPHA = 0

DATA INPUTS (SPIx_MOSI)

Figure 45. SPI Port—Slave Timing

#### SPI Port—SPI Ready (SPIx_RDY) Slave Timing

SPIx_RDY is used to provide flow control. CPOL, CPHA, and FCCH are configuration bits in the SPIx_CTL register.

##### Table 75. SPI Port—SPIx_RDY Slave Timing1

| Parameter | Conditions | Min Max | Unit |
| --- | --- | --- | --- |
| Switching Characteristic tDSPISCKRDYS SPIx_RDY Deassertion from Last Valid Input SPIx_CLK Edge | FCCH = 0 FCCH = 1 | - × tSCLK1 4 × tSCLK1 + 10 - × tSCLK1 5 × tSCLK1 + 10 | ns ns |

1 All specifications apply to all three SPIs.

tDSPISCKRDYS

SPIx_CLK (CPOL = 0)

CPHA = 0

SPIx_CLK (CPOL = 1)

SPIx_CLK (CPOL = 0)

CPHA = 1

SPIx_CLK (CPOL = 1)

SPIx_RDY (O)

Figure 46. SPIx_RDY Deassertion from Valid Input SPIx_CLK Edge in Slave Mode

#### SPI Port—Open Drain Mode (ODM) Timing

In Figure 47 and Figure 48 and Table 77 and Table 78, the outputs can be SPIx_MOSI, SPIx_MISO, SPIx_D2, and/or SPIx_D3, depending on the mode of operation. CPOL and CPHA are configuration bits in the SPI_CTL register.

##### Table 76. SPI Port—ODM Master Mode 1

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Switching Characteristics |  |  |  |
| tHDSPIODMM SPIx_CLK Edge to High Impedance from Data Out Valid | –1 |  | ns |
| tDDSPIODMM SPIx_CLK Edge to Data Out Valid from High Impedance | –1 | +6 | ns |

1 All specifications apply to all three SPIs.

SPIx_CLK (CPOL = 0)

tHDSPIODMM

tHDSPIODMM

SPIx_CLK (CPOL = 1)

OUTPUT (CPHA = 1)

OUTPUT (CPHA = 0)

tDDSPIODMM

tDDSPIODMM

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tHDSPIODMS SPIx_CLK Edge to High Impedance from Data Out Valid | 0 |  | ns |
| tDDSPIODMS SPIx_CLK Edge to Data Out Valid from High Impedance |  | 11 | ns |

Table 77. SPI Port—ODM Slave Mode1

1 All specifications apply to all three SPIs.

Figure 47. ODM Master Mode

tHDSPIODMS	tHDSPIODMS

SPIx_CLK (CPOL = 0)

SPIx_CLK (CPOL = 1)

OUTPUT (CPHA = 1)

OUTPUT (CPHA = 0)

tDDSPIODMS

tDDSPIODMS

Figure 48. ODM Slave Mode

#### SPI Port—SPIx_RDY Master Timing

SPIx_RDY provides flow control. CPOL and CPHA are configuration bits in the SPIx_CTL register, whereas LEADX, LAGX, and STOP are configuration bits in the SPIx_DLY register.

##### Table 78. SPI Port—SPIx_RDY Master Timing1

| Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Timing Requirement tSRDYSCKM Setup Time for SPIx_RDY Deassertion Before Last Valid Data SPIx_CLK Edge |  | (2 + 2 × BAUD2) × tSCLK1 + 10 |  | ns |
| Switching Characteristic tDRDYSC 3 Assertion of SPIx_RDY to First SPIx_CLK KM Edge of Next Transfer | Baud = 0, CPHA = 0 | 4.5 × tSCLK1 | 5.5 × tSCLK1 + 10 | ns |
|  | Baud = 0, CPHA = 1 | 4 × tSCLK1 | 5 × tSCLK1 + 10 | ns |
|  | Baud > 0, CPHA = 0 | (1 + 1.5 × BAUD2) × tSCLK1 | (2 + 2.5 × BAUD2) × tSCLK1 + 10 | ns |
|  | Baud > 0, CPHA = 1 | (1 + 1 × BAUD2) × tSCLK1 | (2 + 2 × BAUD2) × tSCLK1 + 10 | ns |

1 All specifications apply to all three SPIs.

2 BAUD value is set using the SPIx_CLK.BAUD bits. BAUD value = SPIx_CLK.BAUD bits + 1.

3 Specification assumes the LEADX, LAGX, and STOP bits in the SPI_DLY register are zero.

tSRDYSCKM

SPIx_RDY

SPIx_CLK (CPOL = 0)

SPIx_CLK (CPOL = 1)

Figure 49. SPIx_RDY Setup Before SPIx_CLK

tDRDYSCKM

SPIx_RDY

SPIx_CLK (CPOL = 0)

CPHA = 0

SPIx_CLK (CPOL = 1)

SPIx_CLK (CPOL = 0)

CPHA = 1

SPIx_CLK (CPOL = 1)

Figure 50. SPIx_CLK Switching Diagram After SPIx_RDY Assertion

#### Precision Clock Generator (PCG) (Direct Pin Routing)

This timing is only valid when the SRU is configured such that the precision clock generator (PCG) takes inputs directly from the DAI pins (via pin buffers) and sends outputs directly to the DAI pins. For the other cases, where the PCG inputs and outputs are not directly routed to/from DAI pins (via pin buffers), there is no timing data available. All timing parameters and switching characteristics apply to external DAI pins (DAIx_PINx).

##### Table 79. Precision Clock Generator (Direct Pin Routing)

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Timing Requirements tPCGIP Input Clock Period tSTRIG PCG Trigger Setup Before Falling Edge of PCG Input Clock tHTRIG PCG Trigger Hold After Falling Edge of PCG Input Clock | tSCLK × 2 4.5 3 | ns ns ns |
| Switching Characteristics |  |  |
| tDPCGIO PCG Output Clock and Frame Sync Active Edge Delay After PCG Input Clock | 2.5 13.5 | ns |
| tDTRIGCLK PCG Output Clock Delay After PCG Trigger | 2.5 + (2.5 × tPCGIP) 13.5 + (2.5 × tPCGIP) | ns |
| tDTRIGFS1 PCG Frame Sync Delay After PCG Trigger | 2.5 + ((2.5 + D – PH) × tPCGIP) 13.5 + ((2.5 + D – PH) × tPCGIP) | ns |
| tPCGOW2 Output Clock Period | 2 × tPCGIP – 1 | ns |

1 D = FSxDIV, PH = FSxPHASE. For more information, see the ADSP-SC58x/ADSP-2158x SHARC+ Processor Hardware Reference.

2 Normal mode of operation.

tSTRIG

tHTRIG

tDPCGIO

tPCGIP

tDTRIGCLK

tDPCGIO

tPCGOW

tDTRIGFS

DAIx_PIN20–1 PCG_TRIGx_I

DAIx_PIN20–1 PCG_EXTx_I (CLKIN)

DAIx_PIN20–1 PCG_CLKx_O

DAIx_PIN20–1

PCG_FSx_O

Figure 51. PCG (Direct Pin Routing)

#### General-Purpose I/O Port Timing

Table 80 and Figure 52 describe I/O timing, related to the general-purpose I/O port.

##### Table 80. General-Purpose Port Timing

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Timing Requirement tWFI General-Purpose Port Pin Input Pulse Width | 2 × tSCLK0 – 1.5 | ns |

tWFI

GPIO INPUT

#### General-Purpose I/O Timer Cycle Timing

Figure 52. General-Purpose Port Pin Timing

Table 81, Table 82, and Figure 53 describe timer expired operations related to the general-purpose timer. The input signal is asynchronous in Width Capture Mode and External Clock Mode and has an absolute maximum input frequency of fSCLK/4 MHz. The Width Value value is the timer period assigned in the TMx_TMRn_WIDTH register and can range from 1 to 232 – 1. When externally generated, the

TMx_CLK clock is called fTMRCLKEXT, as follows:

tTMRCLKEXT  = ------------1------------

fTMRCLKEXT

##### Table 81. Timer Cycle Timing (Internal Mode)

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Timing Requirements tWL Timer Pulse Width Input Low (Measured In SCLK Cycles)1 tWH Timer Pulse Width Input High (Measured In SCLK Cycles)1 | 2 × tSCLK 2 × tSCLK | ns ns |
| Switching Characteristic tHTO Timer Pulse Width Output (Measured In SCLK Cycles)2 | tSCLK × WIDTH – 1.5 tSCLK × WIDTH + 1.5 | ns |

1 The minimum pulse width applies for TMRx signals in width capture and external clock modes.

2 WIDTH refers to the value in the TMRx_WIDTH register (it can vary from 1 to 232 – 1).

##### Table 82. Timer Cycle Timing (External Mode)

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Timing Requirements tWL Timer Pulse Width Input Low (Measured In EXT_CLK Cycles)1 tWH Timer Pulse Width Input High (Measured In EXT_CLK Cycles)1 tEXT_CLK Timer External Clock Period2 | 2 × tEXT_CLK 2 × tEXT_CLK tTMRCLKEXT | ns ns ns |
| Switching Characteristic tHTO Timer Pulse Width Output (Measured In EXT_CLK Cycles)3 | tEXT_CLK × WIDTH – 1.5 tEXT_CLK × WIDTH + 1.5 | ns |

1 The minimum pulse width applies for TMRx signals in width capture and external clock modes.

2 This specification indicates the minimum instantaneous width or period that can be tolerated due to duty cycle variation or jitter on the external TMR_CLK. For the external TMR_CLK maximum frequency see the fTMRCLKEXT specification in Table 29.

3 WIDTH refers to the value in the TMRx_WIDTH register (it can vary from 1 to 232 – 1).

tHTO

tWH, tWL

TMR OUTPUT

TMR INPUT

Figure 53. Timer Cycle Timing

#### DAIx Pin to DAIx Pin Direct Routing (DAI0 Block and DAI1 Block)

Table 83 and Figure 54 describe I/O timing related to the digital audio interface (DAI) for direct pin connections only (for example, DAIx_PB01_I to DAIx_PB02_O).

##### Table 83. DAI Pin to DAI Pin Routing

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Switching Characteristic tDPIO Delay DAI Pin Input Valid to DAI Output Valid | 1.5 12 | ns |

DAIx_PINn

DAIx_PINm

tDPIO

#### Up/Down Counter/Rotary Encoder Timing

Figure 54. DAI Pin to DAI Pin Direct Routing

Table 84 and Figure 55 describe timing related to the general-purpose counter (CNT).

##### Table 84. Up/Down Counter/Rotary Encoder Timing

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Timing Requirement tWCOUNT Up/Down Counter/Rotary Encoder Input Pulse Width | 2 × tSCLK0 | ns |

CNT0_UD CNT0_DG CNT0_ZM

tWCOUNT

Figure 55. Up/Down Counter/Rotary Encoder Timing

#### Pulse Width Modulator (PWM) Timing

Table 85 and Figure 56 describe timing, related to the PWM.

##### Table 85. PWM Timing1

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Timing Requirement tES External Sync Pulse Width | 2 × tSCLK0 | ns |
| Switching Characteristics tDODIS Output Inactive (off ) After Trip Input2 tDOE Output Delay After External Sync2, 3 | 15 2 × tSCLK0 + 5.5 5 × tSCLK0 + 14 | ns ns |

1 All specifications apply to all three PWMs.

2 PWM outputs are PWMx_AH, PWMx_AL, PWMx_BH, PWMx_BL, PWMx_CH, and PWMx_CL.

3 When the external sync signal is synchronous to the peripheral clock, it takes fewer clock cycles for the output to appear compared to when the external sync signal is asynchronous to the peripheral clock.

tES

tDOE

tDODIS

PWMx_SYNC (AS INPUT)

OUTPUT

PWMx_TRIP

Figure 56. PWM Timing

#### PWM — Medium Precision (MP) Mode Timing

Table 86 and Figure 57 describe medium precision (MP) PWM operations.

##### Table 86. PWM—MP Mode, Output Pulse

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Switching Characteristic tMPWMW MP PWM Output Pulse Width1, 2 | (N + m × 0.25) × tSCLK – 1.0 (N + m × 0.25) × tSCLK + 1.0 | ns |

1 N is the DUTY bit field (coarse duty) from the duty register. m is the ENHDIV (Enhanced Precision Divider bits) value from the HP duty register.

2 Applies to individual PWM channel with 50% duty cycle. Other PWM channels within the same unit are toggling at the same time. No other GPIO pins toggle.

tMPWMW

PWMOUTPUT

Figure 57. PWM MP Mode Timing, Output Pulse

#### PWM — Heightened Precision (HP) Mode Timing

Table 87, Table 88, and Figure 58 through Figure 61 describe heightened precision (HP) PWM operations.

##### Table 87. PWM—HP Mode (HPPWM), Output Pulse Width Accuracy

| Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| HPPWM Pulse Width Accuracy |  |  |  |  |  |
| Resolution1, 2 | Maximum allowed heightened precision divider bits for fractional duty cycles within system clock period |  |  | 4 | Bits |
| Differential Nonlinearity (DNL)1, 3 | Guaranteed monotonic | –0.99 |  | +1.0 | LSB |
| Integral Nonlinearity (INL)1, 4 |  | –1.0 |  | +1.0 | LSB |
| RMS Jitter1 | RMS jitter of any given pulse width code step |  | 200 |  | ps |

1 This specification applies when the system clock SCLK0 is running at 112.5 and 125 MHz.

2 See Figure 58 for an example of 4-bit resolution of fractional duty cycle edge placement.

3 DNL definition. See Figure 59 for an example of DNL calculation. For each heightened precision duty register value (n) is as follows:

DNLn = PWn – PWn – 1 – 1

IdealLSBStepWidth

4 INL definition. See Figure 60 for an example of INL calculation. For each heightened precision duty register value (n) is as follows:

INLn =

PWn – PW0	– n IdealLSBPulseWidth

SYSCLK

0	8	16

PWMOUT

HP DUTY CYCLE CONTROL CODE n

PWMOUT

HP DUTY CYCLE CONTROL CODE n + 1

Figure 58. Fractional Duty Cycle Edge Placement (4-Bit Resolution)

…

…

|  | IDEAL PULSE WIDTH |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DNL = 0.5 |  |  |  |  |
|  |  |  | DNL = 0 |  |  |  |  |  |
|  |  | DNL = 0.5 |  |  |  |  |  |  |
|  | DNL = 0 |  |  |  |  |  |  |  |

15	15

7	7

PWM OUTPUT EDGE PLACEMENT (LSB)

PWM OUTPUT EDGE PLACEMENT (LSB)

6	6	IDEAL PULSE WIDTH

5	5

4	4

3	3

2	2	INL = 0.5

INL = 0

INL = -0.3

1

0

0	1	2	3	4	5	6	7  … 15

HEIGHTENED PRECISION DUTY CYCLE CODE (ONLY THE FIRST 8 CODES ARE SHOWN)

Figure 59. HPPWM Pulse Width Accuracy: DNL Calculation

Note that Figure 59 and Figure 60 show sample data for calcu- lating DNL and INL, respectively. They do not reflect actual measured performance.

##### Table 88. PWM—HP and MP Modes, Output Skew

1

0

0	1	2	3	4	5	6	7  … 15

HEIGHTENED PRECISION DUTY CYCLE CODE (ONLY THE FIRST 8 CODES ARE SHOWN)

Figure 60. HPPWM Pulse Width Accuracy: INL Calculation

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Switching Characteristic tPWMS HP and MP PWM Output Skew 1 | 1.0 | ns |

1 Output edge difference between any two PWM channels (AH, AL, BH, BL, CH, CL, DH and DL) in the same PWM unit (a unit is PWMx where x = 0, 1, 2), with the same HP/MP edge placement.

tPWMS

PWM OUTPUTS

PWM OUTPUTS

Figure 61. PWM HP and MP Modes Timing, Output Skew

#### ADC Controller Module (ACM) Timing

Table 89 and Figure 62 describe ACM operations.

When internally generated, the programmed ACM clock (fACLKPROG) frequency in MHz is set by the following equation where CKDIV is a field in the ACM_TC0 register and ranges from 1 to 255:

fACLKPROG =

f

SCLK1

-C--K--D----I-V---+ 	1-

tACLKPROG  =  --------1---------

fACLKPROG

Setup cycles (SC) in Table 89 is also a field in the ACM0_TC0 register and ranges from 0 to 4095. Hold cycles (HC) is a field in the ACM0_TC1 register that ranges from 0 to 15.

##### Table 89. ACM Timing

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Timing Requirements |  |  |
| tSDR SPORT DRxPRI/DRxSEC Setup Before ACMx_CLK | 3.5 | ns |
| tHDR SPORT DRxPRI/DRxSEC Hold After ACMx_CLK | 1.5 | ns |
| Switching Characteristics |  |  |
| tSCTLCS ACM Controls (ACMx_A[4:0]) Setup Before Assertion of CS | (SC + 1) × tSCLK1 – 3 | ns |
| tHCTLCS ACM Control (ACMx_A[4:0]) Hold After Deassertion of CS | HC × tACLKPROG – 1 | ns |
| tACLKW ACM Clock Pulse Width1 | (0.5 × tACLKPROG) – 1.5 | ns |
| tACLK ACM Clock Period1 | tACLKPROG – 1.5 | ns |
| tHCSACLK CS Hold to ACMx_CLK Edge | –2.5 | ns |
| tSCSACLK CS Setup to ACMx_CLK Edge | tACLKPROG – 3.5 | ns |

1 See Table 29 for details on the minimum period that can be programmed for tACLKPROG.

tSCSACLK

t

tACLK

tHCSACLK

ACLKW

t SCTLCS

tSDR	tHDR

t HCTLCS

DAIx_PIN20–1 (ACM0_FS/CS) CSPOL = 1/0

DAIx_PIN20–1 (ACM0_CLK) CLKPOL = 1/0

DAIx_PIN20–1 (ACM0_A0-4)

DAIx_PIN20–1 (ACM0_T0)

Figure 62. ACM Timing

#### Universal Asynchronous Receiver-Transmitter (UART) Ports—Receive and Transmit Timing

The UART ports receive and transmit operations are described in the ADSP-SC58x/ADSP-2158x SHARC+ Processor Hardware Reference.

#### Controller Area Network (CAN) Interface

The CAN interface timing is described in the ADSP-SC58x/ADSP-2158x SHARC+ Processor Hardware Reference.

#### Universal Serial Bus (USB)

Table 90 describes the universal serial bus (USB) clock timing. Refer to the USB 2.0 Specification for timing and dc specifications for USB pins (including output characteristics for driver types E, F, and G listed in the ADSP-SC58x/ADSP-2158x Designer Quick Reference).

##### Table 90. USB Clock Timing1

| Parameter |  | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Timing Requirements |  |  |  |  |
| fUSBS | USB_CLKIN Frequency | 24 | 24 | MHz |
| fsUSB | USB_CLKIN Clock Frequency Stability | –50 | +50 | ppm |

1 This specification is supported by USB0.

#### PCI Express (PCIe)

The PCIe interface complies with the Gen1 and Gen2 x1 lane data rate specification and supports up to 3.0 PCIe base functionality. For more information about PCIe, see the following standards:

- PCI Express Base 3.0 Specification, Revision 1.0, PCI-SIG

- PCI Express 2.0 Card Electromechanical Specification, Revision 2.0, PCI-SIG

- PHY Interface for the PCI Express Architecture, Revision 2.0, Intel Corporation

- PCI-SIG Engineering Change Request: L1 Substates, February 1, 2012, PCI-SIG

- IEEE Standard 1149.1-2001, IEEE

- IEEE Standard 1149.6-2003, IEEE

#### 10/100 EMAC Timing (ETH0 and ETH1)

Table 91 through Table 93 and Figure 63 through Figure 65 describe the RMII EMAC operations.

##### Table 91. 10/100 EMAC Timing—RMII Receive Signal1

| Parameter2 | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tREFCLKF ETHx_REFCLK Frequency (fSCLK0 = SCLK0 Frequency) |  | 50 + 1% | MHz |
| tREFCLKW ETHx_REFCLK Width (tREFCLKF = ETHx_REFCLK Period) | tREFCLKF × 35% | tREFCLKF × 65% | ns |
| tREFCLKIS Rx Input Valid to RMII ETHx_REFCLK Rising Edge (Data Input Setup) | 1.75 |  | ns |
| tREFCLKIH RMII ETHx_REFCLK Rising Edge to Rx Input Invalid (Data Input Hold) | 1.6 |  | ns |

1 These specifications apply to ETH0 and ETH1.

2 RMII inputs synchronous to RMII ETHx_REFCLK are ETHx_RXD1–0, RMII ETHx_CRS, and ERxER.

tREFCLKF

tREFCLKW

tREFCLKW

ETHx_REFCLK

ETHx_RXD1–0 ETHx_CRS

tREFCLKIS

tREFCLKIH

Figure 63. 10/100 EMAC Controller Timing—RMII Receive Signal

##### Table 92. 10/100 EMAC Timing—RMII Transmit Signal1

| Parameter2 | Min | Max | Unit |
| --- | --- | --- | --- |
| Switching Characteristics |  |  |  |
| tREFCLKOV RMII ETHx_REFCLK Rising Edge to Transmit Output Valid (Data Out Valid) |  | 11.9 | ns |
| tREFCLKOH RMII ETHx_REFCLK Rising Edge to Transmit Output Invalid (Data Out Hold) | 2 |  | ns |

1 These specifications apply to ETH0 and ETH1.

2 RMII outputs synchronous to RMII ETHx_REFCLK are ETHx_TXD1 and TXD0.

tREFCLKF

tREFCLKOH

tREFCLKOV

ETHx_REFCLK

ETHx_TXD1–0 ETHx_TXEN

Figure 64. 10/100 EMAC Controller Timing—RMII Transmit Signal

##### Table 93. EMAC Timing— Station Management1

| Parameter2 | Min Max | Unit |
| --- | --- | --- |
| Timing Requirements tMDIOS ETHx_MDIO Input Valid to ETHx_MDC Rising Edge (Setup) tMDCIH ETHx_MDC Rising Edge to ETHx_MDIO Input Invalid (Hold) | 10.8 0 | ns ns |
| Switching Characteristics tMDCOV ETHx_MDC Falling Edge to ETHx_MDIO Output Valid tMDCOH ETHx_MDC Falling Edge to ETHx_MDIO Output Invalid (Hold) | tSCLK0 + 2 tSCLK0 –2.9 | ns ns |

1 These specifications apply to ETH0 and ETH1.

2 ETHx_MDC/ETHx_MDIO is a 2-wire serial bidirectional port for controlling one or more external PHYs. ETHx_MDC is an output clock with a minimum period that is programmable as a multiple of the system clock SCLK0. ETHx_MDIO is a bidirectional data line.

tMDCOH

tMDCOV

tMDIOS

tMDCIH

ETHx_MDC (OUTPUT)

ETHx_MDIO (OUTPUT)

ETHx_MDIO

(INPUT)

Figure 65. Ethernet MAC Controller Timing— Station Management

#### 10/100/1000 EMAC Timing (ETH0 Only)

Table 94 and Figure 66 describe the RGMII EMAC timing.

##### Table 94. 10/100/1000 EMAC Timing—RGMII Receive and Transmit Signals 1

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tSETUPR Data to Clock Input Setup at Receiver | 1 |  | ns |
| tHOLDR Data to Clock Input Hold at Receiver | 1 |  | ns |
| tGREFCLKF RGMII Receive Clock Period | 8 |  | ns |
| tGREFCLKW RGMII Receive Clock Pulse Width | 4 |  | ns |
| Switching Characteristics |  |  |  |
| tSKEWT Data to Clock Output Skew at Transmitter tCYC Clock Cycle Duration tDUTY_G Duty Cycle for RGMII Minimum | –0.5 7.2 tGREFCLKF × 45% | 0.5 8.8 tGREFCLKF × 55% | ns ns ns |

1 This specification is supported by ETH0 only (10/100/1000 EMAC controller).

tSKEWT

t

DUTY_G

t

DUTY_G

t

CYC

ETH_TXCLK (AT TRANSMITTER)

ETH_TXD3–0

ETH_TXCTL_TXEN

tSETUPR

tGREFCLKW

tHOLDR

tGREFCLKW

tGREFCLKF

ETH_RXCLK_REFCLK

(AT RECEIVER)

ETH_RXD3–0

ETH_RXCTL_CRS

Figure 66. Gigabit EMAC Controller Timing—RGMII

#### Sinus Cardinalis (SINC) Filter Timing

The programmed SINC filter clock (fSINCLKPROG) frequency in MHz is set by the following equation where MDIV is a field in the CLK control register that can be set from 4 to 63:

fSINCLKPROG =

f

SCLK

-M---D---I-V-

tSINCLKPROG  = -------------1-------------

fSINCLKPROG

##### Table 95. SINC Timing

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Timing Requirements tSSINC SINC0_Dx Setup Before SINC0_CLKx Rise tHSINC SINC0_Dx Hold After SINC0_CLKx Rise | 13.5 0 | ns ns |
| Switching Characteristics tSINCLK SINC0_CLKx Period1 tSINCLKW SINC0_CLKx Width1 | tSINCLKPROG – 2.5 0.5 × tSINCLKPROG – 2.5 | ns ns |

1 See Table 29 for details on the minimum period that may be programmed for tSINCLKPROG.

tSINCLK

tSINCLKW

t

SINCLKW

SINC0_CLKx

tSSINC

tHSINC

SINC_Dx

Figure 67. SINC Timing

#### Sony/Philips Digital Interface (S/PDIF) Transmitter

Serial data input to the S/PDIF transmitter can be formatted as left justified, I2S, or right justified with word widths of 16, 18, 20, or 24 bits. The following sections provide timing for the transmitter.

##### S/PDIF Transmitter Serial Input Waveforms

Figure 68 and Table 96 show the right justified mode. Frame sync is high for the left channel and low for the right channel. Data is valid on the rising edge of serial clock. The MSB is delayed the minimum in 24-bit output mode or the maximum in 16-bit output mode from a frame sync transition, so that when there are 64 serial clock periods per frame sync period, the LSB of the data is right justified to the next frame sync transition.

##### Table 96. S/PDIF Transmitter Right Justified Mode

| Parameter | Conditions | Nominal | Unit |
| --- | --- | --- | --- |
| Timing Requirement tRJD Frame Sync to MSB Delay in Right Justified Mode | 16-bit word mode 18-bit word mode 20-bit word mode 24-bit word mode | 16 14 12 8 | SCLK SCLK SCLK SCLK |

LEFT/RIGHT CHANNEL

tRJD

LSB

MSB MSB–1 MSB–2

LSB+2 LSB+1 LSB

DAI_P20–1 FS

DAI_P20–1 SCLK

DAI_P20–1 SDATA

Figure 68. Right Justified Mode

Figure 69 and Table 97 show the default I2S justified mode. The frame sync is low for the left channel and high for the right channel. Data is valid on the rising edge of serial clock. The MSB is left justified to the frame sync transition but with a delay.

##### Table 97. S/PDIF Transmitter I2S Mode

| Parameter | Nominal | Unit |
| --- | --- | --- |
| Timing Requirement tI2SD Frame Sync to MSB Delay in I2S Mode | 1 | SCLK |

LEFT/RIGHT CHANNEL

tI2SD

MSB MSB–1 MSB–2

LSB+2     LSB+1	LSB

DAI_P20–1 FS

DAI_P20–1 SCLK

DAI_P20–1 SDATA

Figure 69. I2S Justified Mode

Figure 70 and Table 98 show the left justified mode. The frame sync is high for the left channel and low for the right channel. Data is valid on the rising edge of serial clock. The MSB is left justified to the frame sync transition with no delay.

##### Table 98. S/PDIF Transmitter Left Justified Mode

| Parameter | Nominal | Unit |
| --- | --- | --- |
| Timing Requirement tLJD Frame Sync to MSB Delay in Left Justified Mode | 0 | SCLK |

LEFT/RIGHT CHANNEL

tLJD

MSB MSB–1 MSB–2

LSB+2    LSB+1	LSB

DAI_P20–1 FS

DAI_P20–1 SCLK

DAI_P20–1 SDATA

Figure 70. Left Justified Mode

##### S/PDIF Transmitter Input Data Timing

The timing requirements for the S/PDIF transmitter are given in Table 99. Input signals are routed to the DAIx_PINx pins using the SRU. Therefore, the timing specifications provided below are valid at the DAIx_PINx pins.

##### Table 99. S/PDIF Transmitter Input Data Timing

| Parameter | Min Max | Unit |
| --- | --- | --- |
| Timing Requirements |  |  |
| tSISFS1 Frame Sync Setup Before Serial Clock Rising Edge | 3 | ns |
| tSIHF 1 Frame Sync Hold After Serial Clock Rising Edge S | 3 | ns |
| tSISD1 Data Setup Before Serial Clock Rising Edge | 3 | ns |
| tSIHD1 Data Hold After Serial Clock Rising Edge | 3 | ns |
| tSITXCLKW Transmit Clock Width | 9 | ns |
| tSITXCLK Transmit Clock Period | 20 | ns |
| tSISCLKW Clock Width | 36 | ns |
| tSISCLK Clock Period | 80 | ns |

1 The serial clock, data, and frame sync signals can come from any of the DAI pins. The serial clock and frame sync signals can also come via PCG or SPORTs. The input of the PCG can be either CLKIN or any of the DAI pins.

tSITXCLKW

tSITXCLK

tSISCLK

tSISCLKW

tSISFS

tSIHFS

tSISD

tSIHD

SAMPLE EDGE

DAIx_PIN20–1 (TxCLK)

DAIx_PIN20–1 (SCLK)

DAIx_PIN20–1 (FS)

DAIx_PIN20–1 (SDATA)

Figure 71. S/PDIF Transmitter Input Timing

##### Oversampling Clock (TxCLK) Switching Characteristics

The S/PDIF transmitter requires an oversampling clock input. This high frequency clock (TxCLK) input is divided down to generate the internal biphase clock.

##### Table 100. Oversampling Clock (TxCLK) Switching Characteristics

| Parameter | Max | Unit |
| --- | --- | --- |
| Switching Characteristics |  |  |
| fTXCLK_384 Frequency for TxCLK = 384 × Frame Sync | Oversampling ratio × frame sync ≤ 1/tSITXCLK | MHz |
| fTXCLK_256 Frequency for TxCLK = 256 × Frame Sync | 49.2 | MHz |
| fFS Frame Rate (FS) | 192.0 | kHz |

S/PDIF Receiver

The following section describes timing as it relates to the S/PDIF receiver.

##### Internal Digital PLL Mode

In the internal digital PLL mode, the internal digital PLL generates the 512 × FS clock.

##### Table 101. S/PDIF Receiver Internal Digital PLL Mode Timing

| Parameter |  | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Switching Characteristics |  |  |  |  |
| tDFSI | Frame Sync Delay After Serial Clock |  | 5 | ns |
| tHOFSI | Frame Sync Hold After Serial Clock | –2 |  | ns |
| tDDTI | Transmit Data Delay After Serial Clock |  | 5 | ns |
| tHDTI | Transmit Data Hold After Serial Clock | –2 |  | ns |

DRIVE EDGE	SAMPLE EDGE

tDFSI tHOFSI

tDDTI tHDTI

DAIx_PIN20–1 (SCLK)

DAIx_PIN20–1 (FS)

DAIx_PIN20–1 (DATA CHANNEL A/B)

Figure 72. S/PDIF Receiver Internal Digital PLL Mode Timing

#### Media LB (MLB)

All the numbers shown in Table 102 are applicable for all MLB speed modes (1024 FS, 512 FS, and 256 FS) for the 3-pin protocol, unless otherwise specified. Refer to the Media Local Bus Specification version 4.2 for more details.

##### Table 102. 3-Pin MLB Interface Specifications

| Parameter |  | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| tMLBCLK | MLB Clock Period |  |  |  |  |
|  | 1024 FS |  | 20.3 |  | ns |
|  | 512 FS |  | 40 |  | ns |
|  | 256 FS |  | 81 |  | ns |
| tMCKL | MLBCLK Low Time |  |  |  |  |
|  | 1024 FS | 6.1 |  |  | ns |
|  | 512 FS | 14 |  |  | ns |
|  | 256 FS | 30 |  |  | ns |
| tMCKH | MLBCLK High Time |  |  |  |  |
|  | 1024 FS | 9.3 |  |  | ns |
|  | 512 FS | 14 |  |  | ns |
|  | 256 FS | 30 |  |  | ns |
| tMCKR | MLBCLK Rise Time (VIL to VIH) |  |  |  |  |
|  | 1024 FS |  |  | 1 | ns |
|  | 512 FS/256 FS |  |  | 3 | ns |
| tMCKF | MLBCLK Fall Time (VIH to VIL) |  |  |  |  |
|  | 1024 FS |  |  | 1 | ns |
|  | 512 FS/256 FS |  |  | 3 | ns |
| tMPWV1 | MLBCLK Pulse Width Variation |  |  |  |  |
|  | 1024 FS |  |  | 0.7 | nspp |
|  | 512 FS/256 |  |  | 2.0 | nspp |
| tDSMCF | DAT/SIG Input Setup Time | 1 |  |  | ns |
| tDHMCF | DAT/SIG Input Hold Time | 2 |  |  | ns |
| tMCFDZ | DAT/SIG Output Time to Three-State | 0 |  | 15 | ns |
| tMCDRV | DAT/SIG Output Data Delay From MLBCLK Rising Edge |  |  | 8 | ns |
| tMDZH2 | Bus Hold Time |  |  |  |  |
|  | 1024 FS | 2 |  |  | ns |
|  | 512 FS/256 | 4 |  |  | ns |
| CMLB | DAT/SIG Pin Load |  |  |  |  |
|  | 1024 FS |  |  | 40 | pf |
|  | 512 FS/256 |  |  | 60 | pf |

1 Pulse width variation is measured at 1.25 V by triggering on one edge of MLBCLK and measuring the spread on the other edge, measured in ns peak-to-peak.

2 Board designs must ensure the high impedance bus does not leave the logic state of the final driven bit for this time period. Therefore, coupling must be minimized while meeting the maximum capacitive load listed.

VALID

tDHMCF

tDSMCF

tMCKH

tMCKL

tMCKR

tMCDRV

tMCKF

tMLBCLK

tMCFDZ

tMDZH

VALID

MLB_SIG/ MLB_DAT

(Rx, Input)

MLB_CLK

MLB_SIG/ MLB_DAT

(Tx, Output)

Figure 73. MLB Timing (3-Pin Interface)

The ac timing specifications of the 6-pin MLB interface is detailed in Table 103. Refer to the Media Local Bus Specification version 4.2 for more details.

##### Table 103. 6-Pin MLB Interface Specifications

| Parameter | Conditions | Min Typ Max | Unit |
| --- | --- | --- | --- |
| tMT Differential Transition Time at the Input Pin (See Figure 74) | 20% to 80% VIN+/VIN– | 1 | ns |
|  | 80% to 20% VIN+/VIN– |  |  |
| fMCKE MLBCP/N External Clock Operating Frequency (See Figure 75)1 | 2048 × FS at 44.0 kHz | 90.112 | MHz |
|  | 2048 × FS at 50.0 kHz | 102.4 | MHz |
| fMCKR Recovered Clock Operating Frequency (Internal, not Observable | 2048 × FS at 44.0 kHz | 90.112 | MHz |
| at Pins, Only for Timing References) (See Figure 75) | 2048 × FS at 50.0 kHz | 102.4 | MHz |
| tDELAY Transmitter MLBSP/N (MLBDP/N) Output Valid From Transition of MLBCP/N (Low to High) (See Figure 76) | fMCKR = 2048 × FS | 0.6 5 | ns |
| tPHZ Disable Turnaround Time From Transition of MLBCP/N (Low to High) (See Figure 77) | fMCKR = 2048 × FS | 0.6 7 | ns |
| tPLZ Enable Turnaround Time From Transition of MLBCP/N (Low to High) (See Figure 77) | fMCKR = 2048 × FS | 0.6 11.2 | ns |
| tSU MLBSP/N (MLBDP/N) Valid to Transition of MLBCP/N (Low to High) (See Figure 76) | fMCKR = 2048 × FS | 1 | ns |
| tHD MLBSP/N (MLBDP/N) Hold From Transition of MLBCP/N (Low to High) |  | 0.6 | ns |
| (See Figure 76)2 |  |  |  |

1 fMCKE (maximum) and fMCKR (maximum) include maximum cycle to cycle system jitter (tJITTER) of 600 ps for a bit error rate of 10E-9.

2 Receivers must latch MLBSP/N (MLBDP/N) data within tHD (min) of the rising edge of MLBCP/N.

tMT

tMT

80%

20%

MLBCP/N MLBDP/N MLBSP/N

Figure 74. MLB 6-Pin Transition Time

1/fMCKE

T1:1

MLBCP/N

RECOVERED CLOCK (1:1)

NOTE:  T1:1 = 1/fMCKE

Figure 75. MLB 6-Pin Clock Definitions

1/fMCKE

1/fMCKR

tDELAY

tDELAY

tSU

VALID

VALID

tHD	tHD

MLBCP/N

RECOVERED CLOCK

MLBSP/N MLBDP/N (TRANSMIT)

MLBSP/N MLBDP/N (RECEIVE)

Figure 76. MLB 6-Pin Delay, Setup, and Hold Times

tPHZ

tPLZ

MLBCP/N RECOVERED CLOCK (1:1)

MLBDP/N MLNSP/N

Figure 77. MLB 6-Pin Disable and Enable Turnaround Times

#### Mobile Storage Interface (MSI) Controller Timing

Table 104 and Figure 78 show I/O timing related to the MSI.

##### Table 104. MSI Controller Timing

| Parameter | Min Max |  | Unit |
| --- | --- | --- | --- |
| Timing Requirements tISU Input Setup Time tIH Input Hold Time | 4.8 –0.5 |  | ns ns |
| Switching Characteristics |  |  |  |
| fPP Clock Frequency Data Transfer Mode1 |  | 50 | MHz |
| tWL Clock Low Time | 8 |  | ns |
| tWH Clock High Time | 8 |  | ns |
| tTLH Clock Rise Time |  | 3 | ns |
| tTHL Clock Fall Time |  | 3 | ns |
| tODLY Output Delay Time During Data Transfer Mode |  | 2 | ns |
| tOH Output Hold Time | –1.8 |  | ns |

1 tPP = 1/fPP.

tPP

tTHL

tTLH

tISU tIH

tWL	tWH

tODLY

tOH

VOH (MIN)

MSI_CLK

VOL (MAX)

INPUT

OUTPUT

NOTES:

- INPUT INCLUDES MSI_Dx AND MSI_CMD SIGNALS.

- OUTPUT INCLUDES MSI_Dx AND MSI_CMD SIGNALS.

Figure 78. MSI Controller Timing

#### Program Trace Macrocell (PTM) Timing

Table 105 and Figure 79 provide I/O timing related to the PTM.

##### Table 105. Trace Timing

| Parameter | Min | Max | Unit |
| --- | --- | --- | --- |
| Switching Characteristics |  |  |  |
| tDTRD Trace Data Delay From Trace Clock Maximum tHTRD Trace Data Hold From Trace Clock Minimum tPTRCK Trace Clock Period Minimum | 0.5 × tSCLK0 – 1.2 2 × tSCLK0 – 1 | 0.5 × tSCLK0 + 2 | ns ns ns |

tPTRCK

tHTRD

tHTRD

D0

D1

tDTRD

tDTRD

TRACE0_CLK

TRACE0_DX

Figure 79. Trace Timing

#### Debug Interface (JTAG Emulation Port) Timing

Table 106 and Figure 80 provide I/O timing related to the debug interface (JTAG Emulator Port).

##### Table 106. JTAG Emulation Port Timing

| Parameter |  | Min Max | Unit |
| --- | --- | --- | --- |
| Timing Requirements |  |  |  |
| tTCK | JTG_TCK Period | 20 | ns |
| tSTAP | JTG_TDI, JTG_TMS Setup Before JTG_TCK High | 4 | ns |
| tHTAP | JTG_TDI, JTG_TMS Hold After JTG_TCK High | 4 | ns |
| tSSYS | System Inputs Setup Before JTG_TCK High1 | 12 | ns |
| tHSYS | System Inputs Hold After JTG_TCK High1 | 5 | ns |
| tTRSTW | JTG_TRST Pulse Width (measured in JTG_TCK cycles)2 | 4 | TCK |
| Switching Characteristics |  |  |  |
| tDTDO JTG_TDO Delay From JTG_TCK Low |  | 13.5 | ns |
| tDSYS System Outputs Delay After JTG_TCK Low3 |  | 17 | ns |

1 System Inputs = MLB0_CLKP, MLB0_DATP, MLB0_SIGP, DAI0_PIN20-01, DAI1_PIN20-01, DMC0_A15-0, DMC1_A15-0, DMC0_DQ15-0, DMC1_DQ15-0, DMC0_RESET, DMC1_RESET, PA_15-0, PB_15-0, PC_15-0, PD_15-0, PE_15-0, PF_15-0, PG_5-0, SYS_BMODE2-0, SYS_FAULT, SYS_FAULT, SYS_RESOUT, TWI2-0_SCL, TWI2-0_SDA2.

2 50 MHz maximum.

3 System Outputs = DMC0_A15-0, DMC0_BA2-0, DMC0_CAS , DMC0_CK, DMC0_CKE, DMC0_CS0, DMC0_DQ15-0, DMC0_LDM, DMC0_LDQS, DMC0_ODT, DMC0_RAS, DMC0_RESET, DMC0_UDM, DMC0_UDQS, DMC0_WE, DMC1_A15-0, DMC1_BA2-0, DMC1_CAS, DMC1_CK, DMC1_CKE, DMC1_CS0, DMC1_DQ15-0, DMC1_LDM, DMC1_LDQS, DMC1_ODT, DMC1_RAS, DMC1_RESET, DMC1_UDM, DMC1_UDQS, DMC1_WE, MLB0_DATP, MLB0_SIGP, PA_15-0, PB_15-0, PC_15-0, PCIE_TXP, PD_15-0, PE_15-0, PF_15-0, PG_5-0, SYS_BMODE2-0, SYS_CLKOUT, SYS_FAULT, SYS_FAULT, SYS_RESOUT.

tTCK

tSTAP

tHTAP

tDTDO

tSSYS

tHSYS

tDSYS

JTG_TCK

JTG_TMS JTG_TDI

JTG_TDO

SYSTEM INPUTS

SYSTEM OUTPUTS

Figure 80. JTAG Port Timing

## OUTPUT DRIVE CURRENTS

Figure 81 through Figure 93 show typical current-voltage char- acteristics for the output drivers of the ADSP-SC58x and ADSP- 2158x processors. The curves represent the current drive capa- bility of the output drivers as a function of output voltage.

Output drive currents for PCIe pins are compliant with PCIe Gen1 and Gen2 x1 lane data rate specifications. Output drive currents for MLB pins are compliant with MOST150 LVDS specifications. Output drive currents for USB pins are compli- ant with the USB 2.0 specifications.

VOH

VDD_EXT = 3.47V AT –40°C

VDD_EXT = 3.30V AT +25°C

VDD_EXT = 3.13V AT +133°C

VOL

VDD_EXT = 3.13V AT +133°C

VDD_EXT = 3.30V AT +25°C VDD_EXT = 3.47V AT –40°C

50

40

SOURCE CURRENT (mA)

30

20

10

0

–10

–20

50

40

SOURCE CURRENT (mA)

30

20

10

0

–10

–20

–30

–40

–50

VOH

V

VDD_EXT = 3.30V AT +25°C VDD_EXT = 3.13V AT +133°C

DD_EXT = 3.47V AT –40°C

VOL

VDD_EXT = 3.13V AT +133°C

VDD_EXT = 3.30V AT +25°C

VDD_EXT = 3.47V AT –40°C

0	0.5	1.0	1.5	2.0	2.5	3.0	3.5	4.0

SOURCE VOLTAGE (V)

Figure 83. Driver Type H Current (3.3 V VDD_EXT)

VDD_DMC = 1.425V AT +133°C

VDD_DMC = 1.500V AT +25°C

VDD_DMC = 1.575V AT –40°C

0

–30

–40

–50

0	0.5	1.0

1.5

2.0	2.5	3.0	3.5	4.0

–5

SOURCE CURRENT (mA)

–10

0

–5

SOURCE CURRENT (mA)

–10

SOURCE VOLTAGE (V)

Figure 81. Driver Type A Current (3.3 V VDD_EXT)

–15

–20

–25

0	0.2	0.4	0.6	0.8	1.0	1.2	1.4	1.6

SOURCE VOLTAGE (V)

|  |  |  | VDD_EXT = 3.13V AT +133°C VDD_EXT = 3.30V AT +25°C |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VOL |  | VDD_EXT = 3.47V AT –40°C |  |  |  |  |

–15

–20

–25

–30

–35

–40

–45

0	0.5	1.0	1.5	2.0

SOURCE VOLTAGE (V)

2.5	3.0

Figure 84. Driver Type B and Driver Type C (DDR3 Drive Strength 40 Ω)

VDD_DMC = 1.425V AT +133°C

VDD_DMC = 1.500V AT +25°C

VDD_DMC = 1.575V AT –40°C

0

–2

SOURCE CURRENT (mA)

–4

–6

–8

–10

Figure 82. Driver Type D Current (3.3 V VDD_EXT)

–12

–14

–16

0	0.2	0.4	0.6	0.8	1.0	1.2

SOURCE VOLTAGE (V)

Figure 85. Driver Type B and Driver Type C (DDR3 Drive Strength 60 Ω)

VDD_DMC = 1.575V AT –40°C

VDD_DMC = 1.500V AT +25°C

VDD_DMC = 1.425V AT +133°C

25

SOURCE CURRENT (mA)

20

15

10

5

0

0	0.2	0.4	0.6	0.8	1.0	1.2	1.4	1.6	1.8

SOURCE VOLTAGE (V)

0

–2

SOURCE CURRENT (mA)

–4

–6

v8

–10

–12

–14

–16

v18

–20

VDD_DMC = 1.7V AT +133°C

VDD_DMC = 1.8V AT +25°C VDD_DMC = 1.9V AT –40°C

0	0.2	0.4

0.6	0.8	1.0	1.2	1.4

SOURCE VOLTAGE (V)

Figure 86. Driver Type B and Driver Type C (DDR3 Drive Strength 40 Ω)

Figure 89. Driver Type B and Driver Type C (DDR2 Drive Strength 60Ω)

VDD_DMC = 1.575V AT –40°C

VDD_DMC = 1.500V AT +25°C VDD_DMC = 1.425V AT +133°C

16

14

SOURCE CURRENT (mA)

12

10

8

6

4

2

0

0	0.2	0.4	0.6	0.8	1.0	1.2	1.4	1.6	1.8

SOURCE VOLTAGE (V)

30

VDD_DMC = 1.9V AT –40°C

VDD_DMC = 1.7V AT +133°C

VDD_DMC = 1.8V AT +25°C

SOURCE CURRENT (mA)

25

20

15

10

5

0

0	0.2 0.4 0.6 0.8 1.0 1.2 1.4 1.6 1.8 2.0

SOURCE VOLTAGE (V)

Figure 87. Driver Type B and Driver Type C (DDR3 Drive Strength 60 Ω)

Figure 90. Driver Type B and Driver Type C (DDR2 Drive Strength 40Ω)

0

–5

SOURCE CURRENT (mA)

–10

–15

–20

–25

–30

–35

VDD_DMC = 1.7V AT +133°C

VDD_DMC = 1.8V AT +25°C VDD_DMC = 1.9V AT –40°C

0	0.2	0.4	0.6	0.8	1.0	1.2	1.4	1.6	1.8

SOURCE VOLTAGE (V)

20

VDD_DMC = 1.9V AT –40°C VDD_DMC = 1.8V AT +25°C

VDD_DMC = 1.7V AT +133°C

18

SOURCE CURRENT (mA)

16

14

12

10

8

6

4

2

0

0	0.5	1.0	1.5	2.0	2.5

SOURCE VOLTAGE(V)

Figure 88. Driver Type B and Driver Type C (DDR2 Drive Strength 40 Ω)	Figure 91. Driver Type B and Driver Type C (DDR2 Drive Strength 60 Ω)

0

–5

SOURCE CURRENT (mA)

–10

–15

–20

–25

–30

–35

–40

–45

VDD_DMC = 1.7V AT +133°C

VDD_DMC = 1.8V AT +25°C VDD_DMC = 1.9V AT –40°C

0	0.2	0.4	0.6	0.8	1.0	1.2	1.4	1.6

#### Output Enable Time Measurement

Output pins are considered enabled when they make a transi- tion from a high impedance state to the point when they start driving.

The output enable time, tENA, is the interval from the point when a reference signal reaches a high or low voltage level to the point when the output starts driving, as shown on the right side of Figure 95. If multiple pins are enabled, the measurement value is that of the first pin to start driving.

REFERENCE SIGNAL

SOURCE VOLTAGE (V)

Figure 92. Driver Type B and Device Driver C (LPDDR)

tDIS

tENA

VDD_DMC = 1.9V AT –40°C

VDD_DMC = 1.7V AT +133°C

VDD_DMC = 1.8V AT +25°C

45

40

SOURCE CURRENT (mA)

35

OUTPUT STOPS DRIVING

30

25

OUTPUT STARTS DRIVING HIGH IMPEDANCE STATE

20

15

10

5

0

0	0.5	1.0	1.5	2.0	2.5

SOURCE VOLTAGE (V)

Figure 93. Driver Type B and Device Driver C (LPDDR)

## TEST CONDITIONS

All timing parameters appearing in this data sheet were mea- sured under the conditions described in this section. Figure 94 shows the measurement point for ac measurements (except out- put enable/disable). The measurement point, VMEAS, is VDD_EXT/2 for VDD_EXT (nominal) = 3.3 V.

Figure 95. Output Enable/Disable

#### Output Disable Time Measurement

Output pins are considered disabled when they stop driving, enter a high impedance state, and start to decay from the output high or low voltage. The output disable time, tDIS, is the interval from when a reference signal reaches a high or low voltage level to the point when the output stops driving, as shown on the left side of Figure 95.

#### Capacitive Loading

Output delays and holds are based on standard capacitive loads of an average of 6 pF on all pins (see Figure 96). VLOAD is equal to VDD_EXT/2. Figure 97 through Figure 101 show how output rise time varies with capacitance. The delay and hold specifica- tions given must be derated by a factor derived from these figures. The graphs in these figures may not be linear outside the ranges shown.

INPUT OR OUTPUT

VMEAS

VMEAS

Figure 94. Voltage Reference Levels for AC Measurements (Except Output Enable/Disable)

TESTER PIN ELECTRONICS

50:

VLOAD

45:

DUT OUTPUT

70:

50:

ZO = 50: (impedance) TD = 4.04  1.18 ns

4pF

0.5pF

2pF

400:

NOTES:

THE WORST CASE TRANSMISSION LINE DELAY IS SHOWN AND CAN BE USED FOR THE OUTPUT TIMING ANALYSIS TO REFLECT THE TRANSMISSION LINE EFFECT AND MUST BE CONSIDERED. THE TRANSMISSION LINE (TD) IS FOR LOAD ONLY AND DOES NOT AFFECT THE DATA SHEET TIMING SPECIFICATIONS.

ANALOG DEVICES RECOMMENDS USING THE IBIS MODEL TIMING FOR A GIVEN SYSTEM REQUIREMENT. IF NECESSARY, A SYSTEM MAY INCORPORATE EXTERNAL DRIVERS TO COMPENSATE FOR ANY TIMING DIFFERENCES.

T1

|  |  | tRISE = 3.3V AT 25°C |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | tFALL = 3.3V AT 25°C |  |  |  |

3.0

RISE AND FALL TIMES (ns)

2.5

REFERENCE SIGNAL

tDIS_MEASURED	tENA_MEASURED

tDIS	tENA	VOH

VOH	(MEASURED)

(MEASURED)	VOH (MEASURED) - ΔV	VTRIP (HIGH)

VOL	VOL  (MEASURED) + ΔV	VTRIP (LOW) (MEASURED)			VOL

tDECAY	t	(MEASURED)

TRIP

OUTPUT STOPS DRIVING		OUTPUT STARTS DRIVING HIGH IMPEDANCE STATE

2.0

1.5

1.0

0.5

0 0	5	10	15	20	25	30	35	40

LOAD CAPACITANCE (pF)

Figure 98. Driver Type H Typical Rise and Fall Times (10% to 90%) vs. Load Capacitance (VDD_EXT = 3.3 V)

Figure 96. Equivalent Device Loading for AC Measurements (Includes All Fixtures)

|  |  | tRISE = 3.3V AT 25°C |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | tFALL = 3.3V AT 25°C |  |  |  |

5.0

4.5

RISE AND FALL TIMES (ns)

4.0

3.5

3.0

2.5

2.0

1.5

1.0

0.5

0.9

0.8

RISE AND FALL TIMES (ns)

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

tRISE = 1.8V AT 25°C

tFALL = 1.8V AT 25°C

0	2	4	6	8	10	12

LOAD CAPACITANCE (pF)

0

0	5	10	15	20	25	30	35	40

LOAD CAPACITANCE (pF)

Figure 99. Driver Type B and Driver Type C Typical Rise and Fall Times (10% to 90%) vs. Load Capacitance (VDD_DMC = 1.8 V) for LPDDR

Figure 97. Driver Type A Typical Rise and Fall Times (10% to 90%) vs. Load Capacitance (VDD_EXT = 3.3 V)

0.9

0.8

RISE AND FALL TIMES (ns)

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

tRISE = 1.8V AT 25°C

tFALL = 1.8V AT 25°C

0	2	4	6	8	10	12

LOAD CAPACITANCE (pF)

Figure 100. Driver Type B and Driver Type C Typical Rise and Fall Times (10% to 90%) vs. Load Capacitance (VDD_DMC = 1.8 V) for DDR2

tRISE = 1.5V AT 25°C

tFALL = 1.5V AT 25°C

0.9

0.8

RISE AND FALL TIMES (ns)

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

0	2	4	6	8	10	12

LOAD CAPACITANCE (pF)

Figure 101. Driver Type B and Driver Type C Typical Rise and Fall Times (10% to 90%) vs. Load Capacitance (VDD_DMC = 1.5 V) for DDR3

## ENVIRONMENTAL CONDITIONS

To determine the junction temperature on the application PCB, use the following equation:

TJ = TCASE + JT  PD

where:

TJ = junction temperature (°C).

TCASE = case temperature (°C) measured at top center of package.

JT = from Table 107 and Table 108.

PD = power dissipation (see the Total Internal Power Dissipa- tion section for the method to calculate PD).

Values of JA are provided for package comparison and PCB design considerations. JA can be used for a first order approxi- mation of TJ by the following equation:

##### Table 107. Thermal Characteristics for 349 CSP_BGA

| Parameter | Conditions | Typ | Unit |
| --- | --- | --- | --- |
| JA | 0 linear m/s air flow | 13.3 | °C/W |
| JA | 1 linear m/s air flow | 12.1 | °C/W |
| JA | 2 linear m/s air flow | 11.6 | °C/W |
| JC |  | 3.65 | °C/W |
| JT | 0 linear m/s air flow | 0.08 | °C/W |
| JT | 1 linear m/s air flow | 0.12 | °C/W |
| JT | 2 linear m/s air flow | 0.14 | °C/W |

Table 108. Thermal Characteristics for 529 CSP_BGA

| Parameter | Conditions | Typ | Unit |
| --- | --- | --- | --- |
| JA | 0 linear m/s air flow | 13.4 | °C/W |
| JA | 1 linear m/s air flow | 12.1 | °C/W |
| JA | 2 linear m/s air flow | 11.6 | °C/W |
| JC |  | 3.63 | °C/W |
| JT | 0 linear m/s air flow | 0.08 | °C/W |
| JT | 1 linear m/s air flow | 0.11 | °C/W |
| JT | 2 linear m/s air flow | 0.13 | °C/W |

TJ = TA + JA  PD

where TA = ambient temperature (°C).

Values of JC are provided for package comparison and PCB design considerations when an external heat sink is required.

In Table 107 and Table 108, airflow measurements comply with JEDEC standards JESD51-2 and JESD51-6. The junction to case measurement complies with MIL-STD-883 (Method 1012.1). All measurements use a 6-layer PCB with

101.6 mm × 152.4 mm dimensions.

# ADSP-SC58x/ADSP-2158x 349-BALL BGA BALL ASSIGNMENTS

The ADSP-SC58x/ADSP-2158x 349-Ball BGA Ball Assignments (Numerical by Ball Number) table lists the 349-ball BGA pack- age by ball number.

The ADSP-SC58x/ADSP-2158x 349-Ball BGA Ball Assignments (Alphabetical by Pin Name) table lists the 349-ball BGA package by pin name.

## ADSP-SC58x/ADSP-2158x 349-BALL BGA BALL ASSIGNMENTS (NUMERICAL BY BALL NUMBER)

Ball No.	Pin Name

Ball No.	Pin Name

Ball No.	Pin Name

Ball No.	Pin Name

| A01 | GND | B20 | PD_05 | F02 | PC_11 | H17 | VDD_DMC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A02 | DMC0_A06 | B21 | GND | F03 | DMC0_BA1 | H20 | VDD_INT |
| A03 | DMC0_A04 | B22 | PD_08 | F06 | VDD_DMC | H21 | PE_03 |
| A04 | DMC0_RAS | C01 | DMC0_A10 | F07 | VDD_INT | H22 | PE_04 |
| A05 | DMC0_CKE | C02 | DMC0_A09 | F08 | VDD_INT | J01 | PC_05 |
| A06 | DMC0_DQ15 | C03 | GND | F09 | VDD_INT | J02 | PC_06 |
| A07 | DMC0_DQ13 | C04 | DMC0_A08 | F10 | VDD_INT | J03 | JTG_TDI |
| A08 | DMC0_UDQS | C05 | DMC0_A03 | F11 | VDD_DMC | J06 | VDD_DMC |
| A09 | DMC0_UDQS | C06 | DMC0_CAS | F12 | VDD_INT | J09 | GND |
| A10 | DMC0_DQ09 | C07 | DMC0_BA0 | F13 | VDD_INT | J10 | GND |
| A11 | DMC0_VREF | C08 | DMC0_A01 | F14 | VDD_INT | J11 | GND |
| A12 | DMC0_CK | C09 | DMC0_RZQ | F15 | VDD_INT | J12 | GND |
| A13 | DMC0_CK | C10 | DMC0_WE | F16 | VDD_INT | J13 | GND |
| A14 | DMC0_DQ06 | C11 | DMC0_CS0 | F17 | VDD_INT | J14 | GND |
| A15 | DMC0_LDQS | C12 | GND | F20 | VDD_INT | J17 | VDD_EXT |
| A16 | DMC0_LDQS | C13 | DMC0_LDM | F21 | PD_15 | J20 | VDD_INT |
| A17 | DMC0_DQ01 | C14 | DMC0_UDM | F22 | PE_00 | J21 | PE_05 |
| A18 | GND | C15 | PD_01 | G01 | PC_12 | J22 | PE_06 |
| A19 | PD_00 | C16 | PC_14 | G02 | PC_10 | K01 | PC_03 |
| A20 | PD_03 | C17 | SYS_CLKOUT | G03 | PC_04 | K02 | PC_02 |
| A21 | PD_06 | C18 | PC_15 | G06 | VDD_DMC | K03 | SYS_FAULT |
| A22 | GND | C19 | PD_04 | G07 | VDD_DMC | K06 | VDD_INT |
| B01 | DMC0_A07 | C20 | GND | G08 | VDD_DMC | K08 | GND |
| B02 | GND | C21 | PD_07 | G09 | VDD_DMC | K09 | GND |
| B03 | DMC0_A02 | C22 | PD_11 | G10 | VDD_DMC | K10 | GND |
| B04 | DMC0_A00 | D01 | DMC0_A11 | G11 | VDD_DMC | K11 | GND |
| B05 | DMC0_ODT | D02 | DMC0_A12 | G12 | VDD_DMC | K12 | GND |
| B06 | DMC0_DQ14 | D03 | DMC0_BA2 | G13 | VDD_DMC | K13 | GND |
| B07 | DMC0_DQ12 | D11 | VDD_INT | G14 | VDD_DMC | K14 | GND |
| B08 | GND | D12 | VDD_INT | G15 | VDD_DMC | K15 | GND |
| B09 | DMC0_DQ11 | D20 | PD_10 | G16 | VDD_DMC | K17 | VDD_EXT |
| B10 | DMC0_DQ10 | D21 | PD_09 | G17 | VDD_DMC | K20 | VDD_INT |
| B11 | DMC0_DQ08 | D22 | PD_12 | G20 | VDD_INT | K21 | PE_08 |
| B12 | DMC0_DQ07 | E01 | DMC0_A14 | G21 | PE_01 | K22 | PE_07 |
| B13 | DMC0_DQ05 | E02 | DMC0_A15 | G22 | PE_02 | L01 | PC_01 |
| B14 | DMC0_DQ04 | E03 | DMC0_A13 | H01 | PC_08 | L02 | SYS_HWRST |
| B15 | DMC0_DQ03 | E05 | DMC0_A05 | H02 | PC_07 | L03 | PC_09 |
| B16 | DMC0_DQ02 | E20 | VDD_INT | H03 | SYS_FAULT | L04 | VDD_INT |
| B17 | DMC0_DQ00 | E21 | PD_13 | H06 | VDD_DMC | L06 | VDD_INT |
| B18 | PC_13 | E22 | PD_14 | H07 | VDD_DMC | L08 | GND |
| B19 | PD_02 | F01 | DMC0_RESET | H16 | GND | L09 | GND |

Ball No.	Pin Name

Ball No.	Pin Name

Ball No.	Pin Name

Ball No.	Pin Name

| L10 | GND | P03 | JTG_TDO | U10 | VDD_INT | AA01 | DAI0_PIN11 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| L11 | GND | P06 | VDD_EXT | U11 | VDD_INT | AA02 | GND |
| L12 | GND | P09 | GND | U12 | VDD_INT | AA03 | DAI0_PIN10 |
| L13 | GND | P10 | GND | U13 | VDD_INT | AA04 | DAI0_PIN04 |
| L14 | GND | P11 | GND | U14 | VDD_EXT | AA05 | DAI0_PIN05 |
| L15 | GND | P12 | GND | U15 | VDD_EXT | AA06 | USB0_ID |
| L17 | VDD_EXT | P13 | GND | U16 | VDD_EXT | AA07 | USB0_VBUS |
| L19 | VDD_INT | P14 | GND | U17 | VDD_EXT | AA08 | TWI2_SCL |
| L20 | PE_11 | P17 | VDD_EXT | U20 | DAI1_PIN20 | AA09 | TWI2_SDA |
| L21 | PE_10 | P20 | DAI1_PIN01 | U21 | DAI1_PIN11 | AA10 | TWI0_SDA |
| L22 | PE_09 | P21 | DAI1_PIN05 | U22 | DAI1_PIN19 | AA11 | HADC0_VIN2 |
| M01 | JTG_TRST | P22 | DAI1_PIN03 | V01 | PB_13 | AA12 | HADC0_VIN5 |
| M02 | JTG_TMS | R01 | GND | V02 | PB_12 | AA13 | HADC0_VIN4 |
| M03 | JTG_TCK | R02 | PB_15 | V03 | DAI0_PIN20 | AA14 | HADC0_VIN7 |
| M04 | VDD_INT | R03 | PB_14 | V20 | PA_00 | AA15 | PB_05 |
| M06 | VDD_INT | R06 | VDD_EXT | V21 | PA_01 | AA16 | PB_02 |
| M08 | GND | R07 | GND | V22 | PA_02 | AA17 | PA_14 |
| M09 | GND | R16 | GND | W01 | PB_10 | AA18 | PB_03 |
| M10 | GND | R17 | VDD_EXT | W02 | PB_11 | AA19 | PA_12 |
| M11 | GND | R20 | DAI1_PIN08 | W03 | DAI0_PIN19 | AA20 | PA_11 |
| M12 | GND | R21 | DAI1_PIN07 | W11 | VDD_INT | AA21 | GND |
| M13 | GND | R22 | DAI1_PIN06 | W12 | VDD_INT | AA22 | PA_09 |
| M14 | GND | T01 | SYS_XTAL0 | W20 | PA_05 | AB01 | GND |
| M15 | GND | T02 | SYS_BMODE2 | W21 | PA_03 | AB02 | DAI0_PIN09 |
| M17 | VDD_EXT | T03 | DAI0_PIN07 | W22 | PA_04 | AB03 | DAI0_PIN08 |
| M19 | VDD_INT | T06 | VDD_EXT | Y01 | PB_09 | AB04 | USB_CLKIN |
| M20 | PE_13 | T07 | GND | Y02 | PB_08 | AB05 | USB_XTAL |
| M21 | PE_15 | T08 | GND | Y03 | DAI0_PIN12 | AB06 | USB0_DP |
| M22 | PE_12 | T09 | GND | Y04 | DAI0_PIN06 | AB07 | USB0_DM |
| N01 | SYS_XTAL1 | T10 | GND | Y05 | DAI0_PIN02 | AB08 | TWI1_SCL |
| N02 | SYS_BMODE0 | T11 | GND | Y06 | DAI0_PIN03 | AB09 | HADC0_VREFP |
| N03 | PC_00 | T12 | GND | Y07 | DAI0_PIN01 | AB10 | HADC0_VREFN |
| N06 | VDD_EXT | T13 | GND | Y08 | USB0_VBC | AB11 | HADC0_VIN0 |
| N08 | GND | T14 | GND | Y09 | TWI0_SCL | AB12 | HADC0_VIN1 |
| N09 | GND | T15 | GND | Y10 | TWI1_SDA | AB13 | HADC0_VIN3 |
| N10 | GND | T16 | GND | Y11 | VDD_HADC | AB14 | MLB0_SIGP |
| N11 | GND | T17 | VDD_EXT | Y12 | GND | AB15 | MLB0_SIGN |
| N12 | GND | T20 | DAI1_PIN12 | Y13 | HADC0_VIN6 | AB16 | MLB0_DATP |
| N13 | GND | T21 | DAI1_PIN10 | Y14 | PB_06 | AB17 | MLB0_DATN |
| N14 | GND | T22 | DAI1_PIN09 | Y15 | PB_00 | AB18 | MLB0_CLKP |
| N15 | GND | U01 | SYS_CLKIN0 | Y16 | PB_04 | AB19 | MLB0_CLKN |
| N17 | VDD_EXT | U02 | SYS_RESOUT | Y17 | PB_01 | AB20 | PA_13 |
| N20 | DAI1_PIN04 | U03 | PB_07 | Y18 | PA_10 | AB21 | PA_07 |
| N21 | DAI1_PIN02 | U06 | VDD_EXT | Y19 | PA_15 | AB22 | GND |
| N22 | PE_14 | U07 | VDD_EXT | Y20 | GND |  |  |
| P01 | SYS_CLKIN1 | U08 | VDD_USB | Y21 | PA_06 |  |  |
| P02 | SYS_BMODE1 | U09 | VDD_INT | Y22 | PA_08 |  |  |

## ADSP-SC58x/ADSP-2158x 349-BALL BGA BALL ASSIGNMENTS (ALPHABETICAL BY PIN NAME)

Pin Name	Ball No.

Pin Name	Ball No.

Pin Name	Ball No.

Pin Name	Ball No.

| DAI0_PIN01 | Y07 | DMC0_CAS | C06 | GND | J09 | GND | T07 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DAI0_PIN02 | Y05 | DMC0_CK | A13 | GND | J10 | GND | T08 |
| DAI0_PIN03 | Y06 | DMC0_CKE | A05 | GND | J11 | GND | T09 |
| DAI0_PIN04 | AA04 | DMC0_CK | A12 | GND | J12 | GND | T10 |
| DAI0_PIN05 | AA05 | DMC0_CS0 | C11 | GND | J13 | GND | T11 |
| DAI0_PIN06 | Y04 | DMC0_DQ00 | B17 | GND | J14 | GND | T12 |
| DAI0_PIN07 | T03 | DMC0_DQ01 | A17 | GND | K08 | GND | T13 |
| DAI0_PIN08 | AB03 | DMC0_DQ02 | B16 | GND | K09 | GND | T14 |
| DAI0_PIN09 | AB02 | DMC0_DQ03 | B15 | GND | K10 | GND | T15 |
| DAI0_PIN10 | AA03 | DMC0_DQ04 | B14 | GND | K11 | GND | T16 |
| DAI0_PIN11 | AA01 | DMC0_DQ05 | B13 | GND | K12 | GND | Y12 |
| DAI0_PIN12 | Y03 | DMC0_DQ06 | A14 | GND | K13 | GND | Y20 |
| DAI0_PIN19 | W03 | DMC0_DQ07 | B12 | GND | K14 | HADC0_VIN0 | AB11 |
| DAI0_PIN20 | V03 | DMC0_DQ08 | B11 | GND | K15 | HADC0_VIN1 | AB12 |
| DAI1_PIN01 | P20 | DMC0_DQ09 | A10 | GND | L08 | HADC0_VIN2 | AA11 |
| DAI1_PIN02 | N21 | DMC0_DQ10 | B10 | GND | L09 | HADC0_VIN3 | AB13 |
| DAI1_PIN03 | P22 | DMC0_DQ11 | B09 | GND | L10 | HADC0_VIN4 | AA13 |
| DAI1_PIN04 | N20 | DMC0_DQ12 | B07 | GND | L11 | HADC0_VIN5 | AA12 |
| DAI1_PIN05 | P21 | DMC0_DQ13 | A07 | GND | L12 | HADC0_VIN6 | Y13 |
| DAI1_PIN06 | R22 | DMC0_DQ14 | B06 | GND | L13 | HADC0_VIN7 | AA14 |
| DAI1_PIN07 | R21 | DMC0_DQ15 | A06 | GND | L14 | HADC0_VREFN | AB10 |
| DAI1_PIN08 | R20 | DMC0_LDM | C13 | GND | L15 | HADC0_VREFP | AB09 |
| DAI1_PIN09 | T22 | DMC0_LDQS | A16 | GND | M08 | JTG_TCK | M03 |
| DAI1_PIN10 | T21 | DMC0_LDQS | A15 | GND | M09 | JTG_TDI | J03 |
| DAI1_PIN11 | U21 | DMC0_ODT | B05 | GND | M10 | JTG_TDO | P03 |
| DAI1_PIN12 | T20 | DMC0_RAS | A04 | GND | M11 | JTG_TMS | M02 |
| DAI1_PIN19 | U22 | DMC0_RESET | F01 | GND | M12 | JTG_TRST | M01 |
| DAI1_PIN20 | U20 | DMC0_RZQ | C09 | GND | M13 | MLB0_CLKN | AB19 |
| DMC0_A00 | B04 | DMC0_UDM | C14 | GND | M14 | MLB0_CLKP | AB18 |
| DMC0_A01 | C08 | DMC0_UDQS | A09 | GND | M15 | MLB0_DATN | AB17 |
| DMC0_A02 | B03 | DMC0_UDQS | A08 | GND | N08 | MLB0_DATP | AB16 |
| DMC0_A03 | C05 | DMC0_VREF | A11 | GND | N09 | MLB0_SIGN | AB15 |
| DMC0_A04 | A03 | DMC0_WE | C10 | GND | N10 | MLB0_SIGP | AB14 |
| DMC0_A05 | E05 | GND | A01 | GND | N11 | PA_00 | V20 |
| DMC0_A06 | A02 | GND | A18 | GND | N12 | PA_01 | V21 |
| DMC0_A07 | B01 | GND | A22 | GND | N13 | PA_02 | V22 |
| DMC0_A08 | C04 | GND | AA02 | GND | N14 | PA_03 | W21 |
| DMC0_A09 | C02 | GND | AA21 | GND | N15 | PA_04 | W22 |
| DMC0_A10 | C01 | GND | AB01 | GND | P09 | PA_05 | W20 |
| DMC0_A11 | D01 | GND | AB22 | GND | P10 | PA_06 | Y21 |
| DMC0_A12 | D02 | GND | B02 | GND | P11 | PA_07 | AB21 |
| DMC0_A13 | E03 | GND | B08 | GND | P12 | PA_08 | Y22 |
| DMC0_A14 | E01 | GND | B21 | GND | P13 | PA_09 | AA22 |
| DMC0_A15 | E02 | GND | C03 | GND | P14 | PA_10 | Y18 |
| DMC0_BA0 | C07 | GND | C12 | GND | R01 | PA_11 | AA20 |
| DMC0_BA1 | F03 | GND | C20 | GND | R07 | PA_12 | AA19 |
| DMC0_BA2 | D03 | GND | H16 | GND | R16 | PA_13 | AB20 |

Pin Name	Ball No.

Pin Name	Ball No.

Pin Name	Ball No.

Pin Name	Ball No.

| PA_14 | AA17 | PD_14 | E22 | VDD_DMC | G09 | VDD_INT | J20 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PA_15 | Y19 | PD_15 | F21 | VDD_DMC | G10 | VDD_INT | K06 |
| PB_00 | Y15 | PE_00 | F22 | VDD_DMC | G11 | VDD_INT | K20 |
| PB_01 | Y17 | PE_01 | G21 | VDD_DMC | G12 | VDD_INT | L04 |
| PB_02 | AA16 | PE_02 | G22 | VDD_DMC | G13 | VDD_INT | L06 |
| PB_03 | AA18 | PE_03 | H21 | VDD_DMC | G14 | VDD_INT | L19 |
| PB_04 | Y16 | PE_04 | H22 | VDD_DMC | G15 | VDD_INT | M04 |
| PB_05 | AA15 | PE_05 | J21 | VDD_DMC | G16 | VDD_INT | M06 |
| PB_06 | Y14 | PE_06 | J22 | VDD_DMC | G17 | VDD_INT | M19 |
| PB_07 | U03 | PE_07 | K22 | VDD_DMC | H06 | VDD_INT | U09 |
| PB_08 | Y02 | PE_08 | K21 | VDD_DMC | H07 | VDD_INT | U10 |
| PB_09 | Y01 | PE_09 | L22 | VDD_DMC | H17 | VDD_INT | U11 |
| PB_10 | W01 | PE_10 | L21 | VDD_DMC | J06 | VDD_INT | U12 |
| PB_11 | W02 | PE_11 | L20 | VDD_EXT | J17 | VDD_INT | U13 |
| PB_12 | V02 | PE_12 | M22 | VDD_EXT | K17 | VDD_INT | W11 |
| PB_13 | V01 | PE_13 | M20 | VDD_EXT | L17 | VDD_INT | W12 |
| PB_14 | R03 | PE_14 | N22 | VDD_EXT | M17 | VDD_USB | U08 |
| PB_15 | R02 | PE_15 | M21 | VDD_EXT | N06 |  |  |
| PC_00 | N03 | SYS_BMODE0 | N02 | VDD_EXT | N17 |  |  |
| PC_01 | L01 | SYS_BMODE1 | P02 | VDD_EXT | P06 |  |  |
| PC_02 | K02 | SYS_BMODE2 | T02 | VDD_EXT | P17 |  |  |
| PC_03 | K01 | SYS_CLKIN0 | U01 | VDD_EXT | R06 |  |  |
| PC_04 | G03 | SYS_CLKIN1 | P01 | VDD_EXT | R17 |  |  |
| PC_05 | J01 | SYS_CLKOUT | C17 | VDD_EXT | T06 |  |  |
| PC_06 | J02 | SYS_FAULT | H03 | VDD_EXT | T17 |  |  |
| PC_07 | H02 | SYS_FAULT | K03 | VDD_EXT | U06 |  |  |
| PC_08 | H01 | SYS_HWRST | L02 | VDD_EXT | U07 |  |  |
| PC_09 | L03 | SYS_RESOUT | U02 | VDD_EXT | U14 |  |  |
| PC_10 | G02 | SYS_XTAL0 | T01 | VDD_EXT | U15 |  |  |
| PC_11 | F02 | SYS_XTAL1 | N01 | VDD_EXT | U16 |  |  |
| PC_12 | G01 | TWI0_SCL | Y09 | VDD_EXT | U17 |  |  |
| PC_13 | B18 | TWI0_SDA | AA10 | VDD_HADC | Y11 |  |  |
| PC_14 | C16 | TWI1_SCL | AB08 | VDD_INT | D11 |  |  |
| PC_15 | C18 | TWI1_SDA | Y10 | VDD_INT | D12 |  |  |
| PD_00 | A19 | TWI2_SCL | AA08 | VDD_INT | E20 |  |  |
| PD_01 | C15 | TWI2_SDA | AA09 | VDD_INT | F07 |  |  |
| PD_02 | B19 | USB0_DM | AB07 | VDD_INT | F08 |  |  |
| PD_03 | A20 | USB0_DP | AB06 | VDD_INT | F09 |  |  |
| PD_04 | C19 | USB0_ID | AA06 | VDD_INT | F10 |  |  |
| PD_05 | B20 | USB0_VBC | Y08 | VDD_INT | F12 |  |  |
| PD_06 | A21 | USB0_VBUS | AA07 | VDD_INT | F13 |  |  |
| PD_07 | C21 | USB_CLKIN | AB04 | VDD_INT | F14 |  |  |
| PD_08 | B22 | USB_XTAL | AB05 | VDD_INT | F15 |  |  |
| PD_09 | D21 | VDD_DMC | F06 | VDD_INT | F16 |  |  |
| PD_10 | D20 | VDD_DMC | F11 | VDD_INT | F17 |  |  |
| PD_11 | C22 | VDD_DMC | G06 | VDD_INT | F20 |  |  |
| PD_12 | D22 | VDD_DMC | G07 | VDD_INT | G20 |  |  |
| PD_13 | E21 | VDD_DMC | G08 | VDD_INT | H20 |  |  |

## CONFIGURATION OF THE 349-BALL CSP_BGA

Figure 102 shows an overview of signal placement on the 349-ball CSP_BGA.

A1 BALL CORNER

TOP VIEW

1 2 3  4 5 6  7 8  9 10 11 12 13 14 15 16 17 18 19 20 21 22

A B C D E F G

H J

K L M N P

R T U

V W Y

AA

AB

GND

I/O SIGNALS VDD_EXT

VDD_INT VDD_DDR

U      VDD_USB

H VDD_HADC

22 21 20 19

18 17

16 15 14 13 12 11 10 9 8 7

6 5 4 3 2 1

A1 BALL CORNER

A B C D E F G

H J

K L M N P

R T U

V W Y

AA AB

U

H

BOTTOM VIEW

Figure 102. 349-Ball CSP_BGA Configuration

# ADSP-SC58x/ADSP-2158x 529-BALL BGA BALL ASSIGNMENTS

The ADSP-SC58x/ADSP-2158x 529-Ball BGA Ball Assignments (Numerical by Ball Number) table lists the 529-ball BGA pack- age by ball number.

The ADSP-SC58x/ADSP-2158x 529-Ball BGA Ball Assignments (Alphabetical by Pin Name) table lists the 529-ball BGA package by pin name.

## ADSP-SC58x/ADSP-2158x 529-BALL BGA BALL ASSIGNMENTS (NUMERICAL BY BALL NUMBER)

Ball No.	Pin Name

Ball No.	Pin Name

Ball No.	Pin Name

Ball No.	Pin Name

| A01 | GND | B19 | DMC1_DQ11 | D14 | DMC1_BA2 | F09 | GND |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A02 | DMC0_UDQS | B20 | DMC1_DQ12 | D15 | DMC1_CAS | F10 | VDD_INT |
| A03 | DMC0_CK | B21 | DMC1_DQ14 | D16 | DMC1_RAS | F11 | VDD_INT |
| A04 | DMC0_CK | B22 | PD_00 | D17 | DMC1_A09 | F12 | VDD_INT |
| A05 | DMC0_DQ09 | B23 | PD_04 | D18 | DMC1_A15 | F13 | VDD_INT |
| A06 | DMC0_LDQS | C01 | DMC0_DQ14 | D19 | DMC1_A10 | F14 | VDD_INT |
| A07 | DMC0_LDQS | C02 | DMC0_DQ13 | D20 | DMC1_A11 | F15 | VDD_INT |
| A08 | DMC0_DQ05 | C03 | DMC0_CS0 | D21 | PC_14 | F16 | GND |
| A09 | DMC0_DQ03 | C04 | DMC0_CKE | D22 | PD_10 | F17 | VDD_INT |
| A10 | DMC0_DQ01 | C05 | DMC0_LDM | D23 | PD_09 | F18 | VDD_INT |
| A11 | DMC1_DQ03 | C06 | DMC1_RESET | E01 | DMC0_A04 | F19 | VDD_INT |
| A12 | DMC1_DQ00 | C07 | DMC1_A03 | E02 | DMC0_RAS | F20 | PE_06 |
| A13 | DMC1_LDQS | C08 | DMC1_A00 | E03 | DMC0_BA1 | F21 | PD_02 |
| A14 | DMC1_LDQS | C09 | DMC1_A01 | E04 | DMC0_WE | F22 | PD_13 |
| A15 | DMC1_VREF | C10 | DMC1_A04 | E05 | DMC0_RZQ | F23 | PD_12 |
| A16 | DMC1_CK | C11 | DMC1_A06 | E06 | GND | G01 | DMC0_A13 |
| A17 | DMC1_CK | C12 | DMC1_BA1 | E07 | GND | G02 | DMC0_A09 |
| A18 | DMC1_DQ09 | C13 | DMC1_ODT | E08 | GND | G03 | DMC0_A03 |
| A19 | DMC1_UDQS | C14 | DMC1_CS0 | E09 | GND | G04 | DMC0_A11 |
| A20 | DMC1_UDQS | C15 | DMC1_LDM | E10 | VDD_INT | G05 | VDD_INT |
| A21 | DMC1_DQ13 | C16 | DMC1_UDM | E11 | VDD_INT | G06 | VDD_DMC |
| A22 | DMC1_DQ15 | C17 | DMC1_A14 | E12 | VDD_INT | G07 | VDD_DMC |
| A23 | GND | C18 | DMC1_A12 | E13 | VDD_INT | G08 | VDD_DMC |
| B01 | DMC0_UDQS | C19 | DMC1_A13 | E14 | VDD_INT | G09 | VDD_DMC |
| B02 | DMC0_DQ12 | C20 | PC_13 | E15 | VDD_INT | G10 | VDD_DMC |
| B03 | DMC0_DQ11 | C21 | PD_01 | E16 | VDD_INT | G11 | VDD_DMC |
| B04 | DMC0_DQ10 | C22 | PD_06 | E17 | VDD_INT | G12 | VDD_DMC |
| B05 | DMC0_DQ08 | C23 | PD_05 | E18 | VDD_INT | G13 | VDD_DMC |
| B06 | DMC0_DQ06 | D01 | DMC0_VREF | E19 | DMC1_RZQ | G14 | VDD_DMC |
| B07 | DMC0_DQ07 | D02 | DMC0_DQ15 | E20 | PC_15 | G15 | VDD_DMC |
| B08 | DMC0_DQ04 | D03 | DMC0_BA0 | E21 | PD_08 | G16 | VDD_DMC |
| B09 | DMC0_DQ02 | D04 | DMC0_BA2 | E22 | PD_14 | G17 | VDD_DMC |
| B10 | DMC0_DQ00 | D05 | DMC0_ODT | E23 | PD_11 | G18 | VDD_DMC |
| B11 | DMC1_DQ01 | D06 | DMC0_UDM | F01 | DMC0_A01 | G19 | VDD_INT |
| B12 | DMC1_DQ02 | D07 | DMC1_A05 | F02 | DMC0_A06 | G20 | PE_04 |
| B13 | DMC1_DQ04 | D08 | DMC1_WE | F03 | DMC0_CAS | G21 | PE_13 |
| B14 | DMC1_DQ05 | D09 | DMC1_A07 | F04 | DMC0_A02 | G22 | PE_01 |
| B15 | DMC1_DQ06 | D10 | DMC1_A02 | F05 | DMC0_A07 | G23 | PE_00 |
| B16 | DMC1_DQ07 | D11 | DMC1_BA0 | F06 | GND | H01 | DMC0_A14 |
| B17 | DMC1_DQ08 | D12 | DMC1_A08 | F07 | VDD_INT | H02 | DMC0_A12 |
| B18 | DMC1_DQ10 | D13 | DMC1_CKE | F08 | VDD_INT | H03 | DMC0_A05 |

Ball No.	Pin Name

Ball No.	Pin Name

Ball No.	Pin Name

Ball No.	Pin Name

| H04 | DMC0_A00 | K05 | VDD_INT | M06 | VDD_DMC | P07 | GND |
| --- | --- | --- | --- | --- | --- | --- | --- |
| H05 | VDD_INT | K06 | VDD_DMC | M07 | GND | P08 | GND |
| H06 | VDD_DMC | K07 | GND | M08 | GND | P09 | GND |
| H07 | VDD_DMC | K08 | GND | M09 | GND | P10 | GND |
| H08 | VDD_DMC | K09 | GND | M10 | GND | P11 | GND |
| H09 | VDD_DMC | K10 | GND | M11 | GND | P12 | GND |
| H10 | VDD_DMC | K11 | GND | M12 | GND | P13 | GND |
| H11 | VDD_DMC | K12 | GND | M13 | GND | P14 | GND |
| H12 | VDD_DMC | K13 | GND | M14 | GND | P15 | GND |
| H13 | VDD_DMC | K14 | GND | M15 | GND | P16 | GND |
| H14 | VDD_DMC | K15 | GND | M16 | GND | P17 | GND |
| H15 | VDD_DMC | K16 | GND | M17 | GND | P18 | VDD_EXT |
| H16 | VDD_DMC | K17 | GND | M18 | VDD_EXT | P19 | PF_10 |
| H17 | VDD_DMC | K18 | VDD_EXT | M19 | PE_08 | P20 | PF_08 |
| H18 | VDD_DMC | K19 | VDD_INT | M20 | PE_11 | P21 | PF_15 |
| H19 | VDD_INT | K20 | PD_15 | M21 | PF_03 | P22 | PF_12 |
| H20 | SYS_CLKOUT | K21 | PF_11 | M22 | PF_00 | P23 | PG_00 |
| H21 | PE_12 | K22 | PF_06 | M23 | PF_02 | R01 | SYS_XTAL1 |
| H22 | PE_05 | K23 | PE_10 | N01 | JTG_TMS | R02 | SYS_BMODE1 |
| H23 | PE_02 | L01 | PC_04 | N02 | JTG_TRST | R03 | SYS_BMODE2 |
| J01 | DMC0_A15 | L02 | PC_12 | N03 | SYS_HWRST | R04 | SYS_BMODE0 |
| J02 | DMC0_A10 | L03 | PC_07 | N04 | PC_03 | R05 | VDD_INT |
| J03 | DMC0_A08 | L04 | PC_10 | N05 | VDD_INT | R06 | VDD_EXT |
| J04 | PC_08 | L05 | VDD_INT | N06 | VDD_EXT | R07 | GND |
| J05 | VDD_INT | L06 | VDD_DMC | N07 | GND | R08 | GND |
| J06 | VDD_DMC | L07 | GND | N08 | GND | R09 | GND |
| J07 | GND | L08 | GND | N09 | GND | R10 | GND |
| J08 | GND | L09 | GND | N10 | GND | R11 | GND |
| J09 | GND | L10 | GND | N11 | GND | R12 | GND |
| J10 | GND | L11 | GND | N12 | GND | R13 | GND |
| J11 | GND | L12 | GND | N13 | GND | R14 | GND |
| J12 | GND | L13 | GND | N14 | GND | R15 | GND |
| J13 | GND | L14 | GND | N15 | GND | R16 | GND |
| J14 | GND | L15 | GND | N16 | GND | R17 | GND |
| J15 | GND | L16 | GND | N17 | GND | R18 | VDD_EXT |
| J16 | GND | L17 | GND | N18 | VDD_EXT | R19 | VDD_INT |
| J17 | GND | L18 | VDD_EXT | N19 | VDD_INT | R20 | PG_01 |
| J18 | VDD_EXT | L19 | VDD_INT | N20 | PE_15 | R21 | PG_05 |
| J19 | PD_03 | L20 | PE_03 | N21 | PF_04 | R22 | PG_04 |
| J20 | PD_07 | L21 | PF_09 | N22 | PF_05 | R23 | PF_13 |
| J21 | PF_14 | L22 | PE_09 | N23 | PF_07 | T01 | SYS_CLKIN1 |
| J22 | PF_01 | L23 | PE_14 | P01 | JTG_TDO | T02 | PB_15 |
| J23 | PE_07 | M01 | PC_01 | P02 | JTG_TDI | T03 | GND |
| K01 | DMC0_RESET | M02 | PC_05 | P03 | SYS_FAULT | T04 | PB_14 |
| K02 | PC_11 | M03 | PC_02 | P04 | JTG_TCK | T05 | VDD_INT |
| K03 | PC_06 | M04 | SYS_FAULT | P05 | VDD_INT | T06 | VDD_EXT |
| K04 | PC_09 | M05 | VDD_INT | P06 | VDD_EXT | T07 | GND |

Ball No.	Pin Name

Ball No.	Pin Name

Ball No.	Pin Name

Ball No.	Pin Name

| T08 | GND | V10 | VDD_EXT | Y12 | HADC0_VIN0 | AB14 | HADC0_VIN3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T09 | GND | V11 | VDD_EXT | Y13 | HADC0_VIN7 | AB15 | RTC0_XTAL |
| T10 | GND | V12 | HADC0_VIN4 | Y14 | GND | AB16 | MLB0_SIGN |
| T11 | GND | V13 | VDD_EXT | Y15 | PB_05 | AB17 | MLB0_DATN |
| T12 | GND | V14 | VDD_EXT | Y16 | PA_14 | AB18 | MLB0_CLKN |
| T13 | GND | V15 | VDD_EXT | Y17 | PA_13 | AB19 | PA_15 |
| T14 | GND | V16 | VDD_EXT | Y18 | PA_12 | AB20 | PA_11 |
| T15 | GND | V17 | VDD_EXT | Y19 | PA_10 | AB21 | PA_06 |
| T16 | GND | V18 | VDD_EXT | Y20 | PA_00 | AB22 | PA_04 |
| T17 | GND | V19 | VDD_INT | Y21 | DAI1_PIN14 | AB23 | PA_02 |
| T18 | VDD_EXT | V20 | DAI1_PIN16 | Y22 | DAI1_PIN17 | AC01 | GND |
| T19 | VDD_INT | V21 | DAI1_PIN06 | Y23 | DAI1_PIN15 | AC02 | PCIE0_RXP |
| T20 | DAI1_PIN03 | V22 | DAI1_PIN12 | AA01 | PB_08 | AC03 | PCIE0_RXM |
| T21 | PG_03 | V23 | DAI1_PIN09 | AA02 | PB_07 | AC04 | PCIE0_CLKM |
| T22 | PG_02 | W01 | PB_12 | AA03 | DAI0_PIN16 | AC05 | PCIE0_CLKP |
| T23 | DAI1_PIN01 | W02 | PB_09 | AA04 | DAI0_PIN07 | AC06 | PCIE0_TXP |
| U01 | SYS_XTAL0 | W03 | DAI0_PIN18 | AA05 | DAI0_PIN06 | AC07 | PCIE0_TXM |
| U02 | SYS_RESOUT | W04 | DAI0_PIN11 | AA06 | DAI0_PIN01 | AC08 | USB1_DM |
| U03 | PC_00 | W05 | VDD_INT | AA07 | PCIE0_REF | AC09 | USB1_DP |
| U04 | DAI0_PIN20 | W06 | VDD_INT | AA08 | USB1_VBUS | AC10 | USB0_DP |
| U05 | VDD_INT | W07 | VDD_PCIE | AA09 | USB0_VBUS | AC11 | USB0_DM |
| U06 | VDD_EXT | W08 | VDD_INT | AA10 | TWI1_SCL | AC12 | HADC0_VREFP |
| U07 | GND | W09 | VDD_INT | AA11 | TWI1_SDA | AC13 | VDD_HADC |
| U08 | GND | W10 | VDD_INT | AA12 | HADC0_VIN1 | AC14 | GND |
| U09 | GND | W11 | VDD_INT | AA13 | HADC0_VIN5 | AC15 | RTC0_CLKIN |
| U10 | GND | W12 | HADC0_VIN6 | AA14 | PB_06 | AC16 | MLB0_SIGP |
| U11 | GND | W13 | VDD_INT | AA15 | PB_02 | AC17 | MLB0_DATP |
| U12 | GND | W14 | VDD_RTC | AA16 | PB_04 | AC18 | MLB0_CLKP |
| U13 | GND | W15 | VDD_INT | AA17 | PB_03 | AC19 | PB_01 |
| U14 | GND | W16 | VDD_INT | AA18 | PB_00 | AC20 | PA_07 |
| U15 | GND | W17 | VDD_INT | AA19 | PA_09 | AC21 | PA_08 |
| U16 | GND | W18 | VDD_INT | AA20 | PA_05 | AC22 | PA_03 |
| U17 | GND | W19 | VDD_INT | AA21 | PA_01 | AC23 | GND |
| U18 | VDD_EXT | W20 | DAI1_PIN20 | AA22 | DAI1_PIN19 |  |  |
| U19 | DAI1_PIN08 | W21 | DAI1_PIN11 | AA23 | DAI1_PIN18 |  |  |
| U20 | DAI1_PIN07 | W22 | DAI1_PIN10 | AB01 | DAI0_PIN15 |  |  |
| U21 | DAI1_PIN04 | W23 | DAI1_PIN13 | AB02 | DAI0_PIN14 |  |  |
| U22 | DAI1_PIN05 | Y01 | PB_11 | AB03 | DAI0_PIN09 |  |  |
| U23 | DAI1_PIN02 | Y02 | PB_10 | AB04 | DAI0_PIN13 |  |  |
| V01 | SYS_CLKIN0 | Y03 | DAI0_PIN17 | AB05 | DAI0_PIN04 |  |  |
| V02 | PB_13 | Y04 | DAI0_PIN08 | AB06 | DAI0_PIN02 |  |  |
| V03 | DAI0_PIN19 | Y05 | DAI0_PIN05 | AB07 | DAI0_PIN03 |  |  |
| V04 | DAI0_PIN12 | Y06 | DAI0_PIN10 | AB08 | USB_XTAL |  |  |
| V05 | VDD_INT | Y07 | USB0_ID | AB09 | USB_CLKIN |  |  |
| V06 | VDD_EXT | Y08 | VDD_USB | AB10 | TWI2_SCL |  |  |
| V07 | VDD_PCIE_RX | Y09 | USB0_VBC | AB11 | TWI0_SDA |  |  |
| V08 | VDD_PCIE_TX | Y10 | TWI0_SCL | AB12 | HADC0_VREFN |  |  |
| V09 | VDD_EXT | Y11 | TWI2_SDA | AB13 | HADC0_VIN2 |  |  |

## ADSP-SC58x/ADSP-2158x 529-BALL BGA BALL ASSIGNMENTS (ALPHABETICAL BY PIN NAME)

Pin Name	Ball No.

Pin Name	Ball No.

Pin Name	Ball No.

Pin Name	Ball No.

| DAI0_PIN01 | AA06 | DMC0_A06 | F02 | DMC1_A00 | C08 | DMC1_RZQ | E19 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DAI0_PIN02 | AB06 | DMC0_A07 | F05 | DMC1_A01 | C09 | DMC1_UDM | C16 |
| DAI0_PIN03 | AB07 | DMC0_A08 | J03 | DMC1_A02 | D10 | DMC1_UDQS | A20 |
| DAI0_PIN04 | AB05 | DMC0_A09 | G02 | DMC1_A03 | C07 | DMC1_UDQS | A19 |
| DAI0_PIN05 | Y05 | DMC0_A10 | J02 | DMC1_A04 | C10 | DMC1_VREF | A15 |
| DAI0_PIN06 | AA05 | DMC0_A11 | G04 | DMC1_A05 | D07 | DMC1_WE | D08 |
| DAI0_PIN07 | AA04 | DMC0_A12 | H02 | DMC1_A06 | C11 | GND | A01 |
| DAI0_PIN08 | Y04 | DMC0_A13 | G01 | DMC1_A07 | D09 | GND | A23 |
| DAI0_PIN09 | AB03 | DMC0_A14 | H01 | DMC1_A08 | D12 | GND | E06 |
| DAI0_PIN10 | Y06 | DMC0_A15 | J01 | DMC1_A09 | D17 | GND | E07 |
| DAI0_PIN11 | W04 | DMC0_BA0 | D03 | DMC1_A10 | D19 | GND | E08 |
| DAI0_PIN12 | V04 | DMC0_BA1 | E03 | DMC1_A11 | D20 | GND | E09 |
| DAI0_PIN13 | AB04 | DMC0_BA2 | D04 | DMC1_A12 | C18 | GND | F06 |
| DAI0_PIN14 | AB02 | DMC0_CAS | F03 | DMC1_A13 | C19 | GND | F09 |
| DAI0_PIN15 | AB01 | DMC0_CK | A04 | DMC1_A14 | C17 | GND | F16 |
| DAI0_PIN16 | AA03 | DMC0_CKE | C04 | DMC1_A15 | D18 | GND | J07 |
| DAI0_PIN17 | Y03 | DMC0_CK | A03 | DMC1_BA0 | D11 | GND | J08 |
| DAI0_PIN18 | W03 | DMC0_CS0 | C03 | DMC1_BA1 | C12 | GND | J09 |
| DAI0_PIN19 | V03 | DMC0_DQ00 | B10 | DMC1_BA2 | D14 | GND | J10 |
| DAI0_PIN20 | U04 | DMC0_DQ01 | A10 | DMC1_CAS | D15 | GND | J11 |
| DAI1_PIN01 | T23 | DMC0_DQ02 | B09 | DMC1_CK | A16 | GND | J12 |
| DAI1_PIN02 | U23 | DMC0_DQ03 | A09 | DMC1_CKE | D13 | GND | J13 |
| DAI1_PIN03 | T20 | DMC0_DQ04 | B08 | DMC1_CK | A17 | GND | J14 |
| DAI1_PIN04 | U21 | DMC0_DQ05 | A08 | DMC1_CS0 | C14 | GND | J15 |
| DAI1_PIN05 | U22 | DMC0_DQ06 | B06 | DMC1_DQ00 | A12 | GND | J16 |
| DAI1_PIN06 | V21 | DMC0_DQ07 | B07 | DMC1_DQ01 | B11 | GND | J17 |
| DAI1_PIN07 | U20 | DMC0_DQ08 | B05 | DMC1_DQ02 | B12 | GND | K07 |
| DAI1_PIN08 | U19 | DMC0_DQ09 | A05 | DMC1_DQ03 | A11 | GND | K08 |
| DAI1_PIN09 | V23 | DMC0_DQ10 | B04 | DMC1_DQ04 | B13 | GND | K09 |
| DAI1_PIN10 | W22 | DMC0_DQ11 | B03 | DMC1_DQ05 | B14 | GND | K10 |
| DAI1_PIN11 | W21 | DMC0_DQ12 | B02 | DMC1_DQ06 | B15 | GND | K11 |
| DAI1_PIN12 | V22 | DMC0_DQ13 | C02 | DMC1_DQ07 | B16 | GND | K12 |
| DAI1_PIN13 | W23 | DMC0_DQ14 | C01 | DMC1_DQ08 | B17 | GND | K13 |
| DAI1_PIN14 | Y21 | DMC0_DQ15 | D02 | DMC1_DQ09 | A18 | GND | K14 |
| DAI1_PIN15 | Y23 | DMC0_LDM | C05 | DMC1_DQ10 | B18 | GND | K15 |
| DAI1_PIN16 | V20 | DMC0_LDQS | A07 | DMC1_DQ11 | B19 | GND | K16 |
| DAI1_PIN17 | Y22 | DMC0_LDQS | A06 | DMC1_DQ12 | B20 | GND | K17 |
| DAI1_PIN18 | AA23 | DMC0_ODT | D05 | DMC1_DQ13 | A21 | GND | L07 |
| DAI1_PIN19 | AA22 | DMC0_RAS | E02 | DMC1_DQ14 | B21 | GND | L08 |
| DAI1_PIN20 | W20 | DMC0_RESET | K01 | DMC1_DQ15 | A22 | GND | L09 |
| DMC0_A00 | H04 | DMC0_RZQ | E05 | DMC1_LDM | C15 | GND | L10 |
| DMC0_A01 | F01 | DMC0_UDM | D06 | DMC1_LDQS | A13 | GND | L11 |
| DMC0_A02 | F04 | DMC0_UDQS | B01 | DMC1_LDQS | A14 | GND | L12 |
| DMC0_A03 | G03 | DMC0_UDQS | A02 | DMC1_ODT | C13 | GND | L13 |
| DMC0_A04 | E01 | DMC0_VREF | D01 | DMC1_RAS | D16 | GND | L14 |
| DMC0_A05 | H03 | DMC0_WE | E04 | DMC1_RESET | C06 | GND | L15 |

Pin Name	Ball No.

Pin Name	Ball No.

Pin Name	Ball No.

Pin Name	Ball No.

| GND | L16 | GND | T08 | PA_02 | AB23 | PC_11 | K02 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GND | L17 | GND | T09 | PA_03 | AC22 | PC_12 | L02 |
| GND | M07 | GND | T10 | PA_04 | AB22 | PC_13 | C20 |
| GND | M08 | GND | T11 | PA_05 | AA20 | PC_14 | D21 |
| GND | M09 | GND | T12 | PA_06 | AB21 | PC_15 | E20 |
| GND | M10 | GND | T13 | PA_07 | AC20 | PD_00 | B22 |
| GND | M11 | GND | T14 | PA_08 | AC21 | PD_01 | C21 |
| GND | M12 | GND | T15 | PA_09 | AA19 | PD_02 | F21 |
| GND | M13 | GND | T16 | PA_10 | Y19 | PD_03 | J19 |
| GND | M14 | GND | T17 | PA_11 | AB20 | PD_04 | B23 |
| GND | M15 | GND | U07 | PA_12 | Y18 | PD_05 | C23 |
| GND | M16 | GND | U08 | PA_13 | Y17 | PD_06 | C22 |
| GND | M17 | GND | U09 | PA_14 | Y16 | PD_07 | J20 |
| GND | N07 | GND | U10 | PA_15 | AB19 | PD_08 | E21 |
| GND | N08 | GND | U11 | PB_00 | AA18 | PD_09 | D23 |
| GND | N09 | GND | U12 | PB_01 | AC19 | PD_10 | D22 |
| GND | N10 | GND | U13 | PB_02 | AA15 | PD_11 | E23 |
| GND | N11 | GND | U14 | PB_03 | AA17 | PD_12 | F23 |
| GND | N12 | GND | U15 | PB_04 | AA16 | PD_13 | F22 |
| GND | N13 | GND | U16 | PB_05 | Y15 | PD_14 | E22 |
| GND | N14 | GND | U17 | PB_06 | AA14 | PD_15 | K20 |
| GND | N15 | GND | Y14 | PB_07 | AA02 | PE_00 | G23 |
| GND | N16 | GND | AC01 | PB_08 | AA01 | PE_01 | G22 |
| GND | N17 | GND | AC14 | PB_09 | W02 | PE_02 | H23 |
| GND | P07 | GND | AC23 | PB_10 | Y02 | PE_03 | L20 |
| GND | P08 | HADC0_VIN0 | Y12 | PB_11 | Y01 | PE_04 | G20 |
| GND | P09 | HADC0_VIN1 | AA12 | PB_12 | W01 | PE_05 | H22 |
| GND | P10 | HADC0_VIN2 | AB13 | PB_13 | V02 | PE_06 | F20 |
| GND | P11 | HADC0_VIN3 | AB14 | PB_14 | T04 | PE_07 | J23 |
| GND | P12 | HADC0_VIN4 | V12 | PB_15 | T02 | PE_08 | M19 |
| GND | P13 | HADC0_VIN5 | AA13 | PCIE0_CLKM | AC04 | PE_09 | L22 |
| GND | P14 | HADC0_VIN6 | W12 | PCIE0_CLKP | AC05 | PE_10 | K23 |
| GND | P15 | HADC0_VIN7 | Y13 | PCIE0_REF | AA07 | PE_11 | M20 |
| GND | P16 | HADC0_VREFN | AB12 | PCIE0_RXM | AC03 | PE_12 | H21 |
| GND | P17 | HADC0_VREFP | AC12 | PCIE0_RXP | AC02 | PE_13 | G21 |
| GND | R07 | JTG_TCK | P04 | PCIE0_TXM | AC07 | PE_14 | L23 |
| GND | R08 | JTG_TDI | P02 | PCIE0_TXP | AC06 | PE_15 | N20 |
| GND | R09 | JTG_TDO | P01 | PC_00 | U03 | PF_00 | M22 |
| GND | R10 | JTG_TMS | N01 | PC_01 | M01 | PF_01 | J22 |
| GND | R11 | JTG_TRST | N02 | PC_02 | M03 | PF_02 | M23 |
| GND | R12 | MLB0_CLKN | AB18 | PC_03 | N04 | PF_03 | M21 |
| GND | R13 | MLB0_CLKP | AC18 | PC_04 | L01 | PF_04 | N21 |
| GND | R14 | MLB0_DATN | AB17 | PC_05 | M02 | PF_05 | N22 |
| GND | R15 | MLB0_DATP | AC17 | PC_06 | K03 | PF_06 | K22 |
| GND | R16 | MLB0_SIGN | AB16 | PC_07 | L03 | PF_07 | N23 |
| GND | R17 | MLB0_SIGP | AC16 | PC_08 | J04 | PF_08 | P20 |
| GND | T03 | PA_00 | Y20 | PC_09 | K04 | PF_09 | L21 |
| GND | T07 | PA_01 | AA21 | PC_10 | L04 | PF_10 | P19 |

Pin Name	Ball No.

Pin Name	Ball No.

Pin Name	Ball No.

Pin Name	Ball No.

| PF_11 | K21 | VDD_DMC | G13 | VDD_INT | E10 | VDD_INT | W16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PF_12 | P22 | VDD_DMC | G14 | VDD_INT | E11 | VDD_INT | W17 |
| PF_13 | R23 | VDD_DMC | G15 | VDD_INT | E12 | VDD_INT | W18 |
| PF_14 | J21 | VDD_DMC | G16 | VDD_INT | E13 | VDD_INT | W19 |
| PF_15 | P21 | VDD_DMC | G17 | VDD_INT | E14 | VDD_PCIE | W07 |
| PG_00 | P23 | VDD_DMC | G18 | VDD_INT | E15 | VDD_PCIE_RX | V07 |
| PG_01 | R20 | VDD_DMC | H06 | VDD_INT | E16 | VDD_PCIE_TX | V08 |
| PG_02 | T22 | VDD_DMC | H07 | VDD_INT | E17 | VDD_RTC | W14 |
| PG_03 | T21 | VDD_DMC | H08 | VDD_INT | E18 | VDD_USB | Y08 |
| PG_04 | R22 | VDD_DMC | H09 | VDD_INT | F07 |  |  |
| PG_05 | R21 | VDD_DMC | H10 | VDD_INT | F08 |  |  |
| RTC0_CLKIN | AC15 | VDD_DMC | H11 | VDD_INT | F10 |  |  |
| RTC0_XTAL | AB15 | VDD_DMC | H12 | VDD_INT | F11 |  |  |
| SYS_BMODE0 | R04 | VDD_DMC | H13 | VDD_INT | F12 |  |  |
| SYS_BMODE1 | R02 | VDD_DMC | H14 | VDD_INT | F13 |  |  |
| SYS_BMODE2 | R03 | VDD_DMC | H15 | VDD_INT | F14 |  |  |
| SYS_CLKIN0 | V01 | VDD_DMC | H16 | VDD_INT | F15 |  |  |
| SYS_CLKIN1 | T01 | VDD_DMC | H17 | VDD_INT | F17 |  |  |
| SYS_CLKOUT | H20 | VDD_DMC | H18 | VDD_INT | F18 |  |  |
| SYS_FAULT | P03 | VDD_DMC | J06 | VDD_INT | F19 |  |  |
| SYS_FAULT | M04 | VDD_DMC | K06 | VDD_INT | G05 |  |  |
| SYS_HWRST | N03 | VDD_DMC | L06 | VDD_INT | G19 |  |  |
| SYS_RESOUT | U02 | VDD_DMC | M06 | VDD_INT | H05 |  |  |
| SYS_XTAL0 | U01 | VDD_EXT | J18 | VDD_INT | H19 |  |  |
| SYS_XTAL1 | R01 | VDD_EXT | K18 | VDD_INT | J05 |  |  |
| TWI0_SCL | Y10 | VDD_EXT | L18 | VDD_INT | K05 |  |  |
| TWI0_SDA | AB11 | VDD_EXT | M18 | VDD_INT | K19 |  |  |
| TWI1_SCL | AA10 | VDD_EXT | N06 | VDD_INT | L05 |  |  |
| TWI1_SDA | AA11 | VDD_EXT | N18 | VDD_INT | L19 |  |  |
| TWI2_SCL | AB10 | VDD_EXT | P06 | VDD_INT | M05 |  |  |
| TWI2_SDA | Y11 | VDD_EXT | P18 | VDD_INT | N05 |  |  |
| USB0_DM | AC11 | VDD_EXT | R06 | VDD_INT | N19 |  |  |
| USB0_DP | AC10 | VDD_EXT | R18 | VDD_INT | P05 |  |  |
| USB0_ID | Y07 | VDD_EXT | T06 | VDD_INT | R05 |  |  |
| USB0_VBC | Y09 | VDD_EXT | T18 | VDD_INT | R19 |  |  |
| USB0_VBUS | AA09 | VDD_EXT | U06 | VDD_INT | T05 |  |  |
| USB1_DM | AC08 | VDD_EXT | U18 | VDD_INT | T19 |  |  |
| USB1_DP | AC09 | VDD_EXT | V06 | VDD_INT | U05 |  |  |
| USB1_VBUS | AA08 | VDD_EXT | V09 | VDD_INT | V05 |  |  |
| USB_CLKIN | AB09 | VDD_EXT | V10 | VDD_INT | V19 |  |  |
| USB_XTAL | AB08 | VDD_EXT | V11 | VDD_INT | W05 |  |  |
| VDD_DMC | G06 | VDD_EXT | V13 | VDD_INT | W06 |  |  |
| VDD_DMC | G07 | VDD_EXT | V14 | VDD_INT | W08 |  |  |
| VDD_DMC | G08 | VDD_EXT | V15 | VDD_INT | W09 |  |  |
| VDD_DMC | G09 | VDD_EXT | V16 | VDD_INT | W10 |  |  |
| VDD_DMC | G10 | VDD_EXT | V17 | VDD_INT | W11 |  |  |
| VDD_DMC | G11 | VDD_EXT | V18 | VDD_INT | W13 |  |  |
| VDD_DMC | G12 | VDD_HADC | AC13 | VDD_INT | W15 |  |  |

## CONFIGURATION OF THE 529-BALL CSP_BGA

Figure 103 shows an overview of signal placement on the 529-ball CSP_BGA.

A1 BALL CORNER

C

P

R

1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23

A B C D E F G H J K L M N P R T U V W Y AA AB

AC

TOP VIEW

23 22 21 20 19 18 17 16 15 14 13 12 11 10 9 8 7 6 5 4 3 2 1

A1 BALL CORNER

|  |  |  | A B C D E F G H J K L M N P R T |  | GND I/O SIGNALS VDD_EXT VDD_INT VDD_DDR VDD_USB VDD_RTC VDD_PCIE VDD_HADC VDD_CORE_PCIRX |
| --- | --- | --- | --- | --- | --- |
|  | T | C | U V |  | VDD_CORE_PCITX |
| R |  | P | W |  |  |
|  | U |  | Y |  |  |
|  |  |  | AA |  |  |
|  |  |  | AB |  |  |
| H |  |  | AC |  |  |
| BOTTOM VIEW |  |  |  |  |  |

U R P H C T

Figure 103. 529-Ball CSP_BGA Configuration

# OUTLINE DIMENSIONS

Dimensions for the 19 mm × 19 mm 349-ball CSP_BGA package in Figure 104 are shown in millimeters.

| 19.00 SQ 18.90 |  |
| --- | --- |

A1 BALL CORNER

19.10

A1 BALL RNER

TOP VIEW

22 20 18 16 14 12 10 8 6 4 2

21 19 17 15 13 11 9 7 5 3 1

CO

16.80

BSC SQ

0.80

BSC

A  B

C  D

E  F

G  H

J   K

L   M

N  P

R  T

U  V

W Y

AAAB

1.10 REF	BOTTOM VIEW

1.50

1.36

1.21

DETAIL A

DETAIL A

1.11

1.01

0.91

0.35 NOM

0.30 MIN

SEATING PLANE

0.50

0.45

0.40

BALL DIAMETER

COPLANARITY 0.20

COMPLIANT TO JEDEC STANDARDS MO-275-PPAB-2.

Figure 104. 349-Ball Chip Scale Package Ball Grid Array [CSP_BGA] (BC-349-1)

Dimensions shown in millimeters

Dimensions for the 19 mm × 19 mm 529-ball CSP_BGA package in Figure 105 are shown in millimeters.

| 19.00 SQ 18.90 |  |
| --- | --- |

A1 BALL CORNER

19.10

A1 BALL RNER

TOP VIEW

22 20 18 16 14 12 10 8 6 4 2

23 21 19 17 15 13 11 9 7 5 3 1

CO

E F

A  B

C D

17.60

REF SQ

G H

0.80

BSC

J

L   K

N  M

R  P

U  T

W V

AAY ACAB

0.70 REF

BOTTOM VIEW

1.50

1.36

1.21

DETAIL A

DETAIL A

1.11

1.01

0.91

0.39

0.35

0.30

SEATING PLANE

0.50

0.45

0.40

BALL DIAMETER

COPLANARITY 0.2

COMPLIANT TO JEDEC STANDARDS MO-275-RRAB-2.

Figure 105. 529-Ball Chip Scale Package Ball Grid Array [CSP_BGA] (BC-529-1)

Dimensions shown in millimeters

## SURFACE-MOUNT DESIGN

Table 109 is an aid for PCB design. For industry-standard design recommendations, refer to IPC-7351, Generic Requirements for Surface- Mount Design and Land Pattern Standard.

##### Table 109. CSP_BGA Data for Use with Surface-Mount Design

| Package | Package Ball Attach Type | Package Solder Mask Opening | Package Ball Pad Size |
| --- | --- | --- | --- |
| BC-349-1 BC-529-1 | Solder Mask Defined Solder Mask Defined | 0.4 mm Diameter 0.4 mm Diameter | 0.5 mm Diameter 0.5 mm Diameter |

AUTOMOTIVE PRODUCTS

The following models are available with controlled manufactur- ing to support the quality and reliability requirements of automotive applications. Note that these automotive models may have specifications that differ from the nonautomotive models; therefore designers should review the Specifications section of this data sheet carefully. Only the automotive grade

##### Table 110. Automotive Products

products shown in Table 110 are available for use in automotive applications. Contact your local Analog Devices account repre- sentative for specific product ordering information and to obtain the specific Automotive Reliability reports for these models.

| Model 1, 2 | Processor Instruction Rate (Max) | Temperature Range3 | Arm Cores4 | SHARC+ Cores | SHARC+ SRAM | PCIe Lanes4 | Package Description | Package Option |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AD21583WCBCZ4Axx | 450 MHz | –40°C to +105°C | N/A | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| AD21584WCBCZ4Axx | 450 MHz | –40°C to +105°C | N/A | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| AD21584WCBCZ5Axx | 500 MHz | –40°C to +100°C | N/A | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSC582WCBCZ4Axx | 450 MHz | –40°C to +105°C | 1 | 1 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSC583WCBCZ3Axx | 300 MHz | –40°C to +105°C | 1 | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSC583WCBCZ4Axx | 450 MHz | –40°C to +105°C | 1 | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSC584WCBCZ3Axx | 300 MHz | –40°C to +105°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSC584WCBCZ4Axx | 450 MHz | –40°C to +105°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSC584WCBCZ5Axx | 500 MHz | –40°C to +100°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSC587WCBCZ4Bxx | 450 MHz | –40°C to +105°C | 1 | 2 | 640 kB | N/A | 529-Ball cspBGA | BC-529-1 |
| ADSC587WBBCZ5Bxx | 500 MHz | –40°C to +85°C | 1 | 2 | 640 kB | N/A | 529-Ball cspBGA | BC-529-1 |

1 Z = RoHS Compliant Part.

2 xx denotes the current die revision.

3 Referenced temperature is ambient temperature. The ambient temperature is not a specification. Please see the Operating Conditions section for the junction temperature (TJ) specification which is the only temperature specification.

4 N/A means not applicable.

ADSP-SC582/SC583/SC584/SC587/SC589/ADSP-21583/21584/21587

## ORDERING GUIDE

| Model1 | Processor Instruction Rate (Max) | Temperature Range2 | Arm Cores3 | SHARC+ Cores | SHARC+ SRAM | PCIe Lanes3 | Package Description | Package Option |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADSP-21583KBCZ-4A | 450 MHz | 0°C to +70°C | N/A | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-21583BBCZ-4A | 450 MHz | –40°C to +85°C | N/A | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-21583CBCZ-4A | 450 MHz | –40°C to +95°C | N/A | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-21584KBCZ-4A | 450 MHz | 0°C to +70°C | N/A | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-21584KBCZ-5A | 500 MHz | 0°C to +70°C | N/A | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-21584BBCZ-4A | 450 MHz | –40°C to +85°C | N/A | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-21584BBCZ-5A | 500 MHz | –40°C to +85°C | N/A | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-21584CBCZ-4A | 450 MHz | –40°C to +95°C | N/A | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-21584CBCZ-5A | 500 MHz | –40°C to +90°C | N/A | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-21587KBCZ-4B | 450 MHz | 0°C to +70°C | N/A | 2 | 640 kB | N/A | 529-Ball cspBGA | BC-529-1 |
| ADSP-21587KBCZ-5B | 500 MHz | 0°C to +70°C | N/A | 2 | 640 kB | N/A | 529-Ball cspBGA | BC-529-1 |
| ADSP-21587BBCZ-4B | 450 MHz | –40°C to +85°C | N/A | 2 | 640 kB | N/A | 529-Ball cspBGA | BC-529-1 |
| ADSP-21587BBCZ-5B | 500 MHz | –40°C to +80°C | N/A | 2 | 640 kB | N/A | 529-Ball cspBGA | BC-529-1 |
| ADSP-SC582KBCZ-4A | 450 MHz | 0°C to +70°C | 1 | 1 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC582BBCZ-4A | 450 MHz | –40°C to +85°C | 1 | 1 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC582CBCZ-4A | 450 MHz | –40°C to +95°C | 1 | 1 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC583KBCZ-3A | 300 MHz | 0°C to +70°C | 1 | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC583BBCZ-3A | 300 MHz | –40°C to +85°C | 1 | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC583CBCZ-3A | 300 MHz | –40°C to +95°C | 1 | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC583KBCZ-4A | 450 MHz | 0°C to +70°C | 1 | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC583BBCZ-4A | 450 MHz | –40°C to +85°C | 1 | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC583CBCZ-4A | 450 MHz | –40°C to +95°C | 1 | 2 | 384 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC584KBCZ-3A | 300 MHz | 0°C to +70°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC584BBCZ-3A | 300 MHz | –40°C to +85°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC584CBCZ-3A | 300 MHz | –40°C to +95°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC584KBCZ-4A | 450 MHz | 0°C to +70°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC584KBCZ-5A | 500 MHz | 0°C to +70°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC584BBCZ-4A | 450 MHz | –40°C to +85°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC584BBCZ-5A | 500 MHz | –40°C to +85°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC584CBCZ-4A | 450 MHz | –40°C to +95°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC584CBCZ-5A | 500 MHz | –40°C to +90°C | 1 | 2 | 640 kB | N/A | 349-Ball cspBGA | BC-349-1 |
| ADSP-SC587KBCZ-4B | 450 MHz | 0°C to +70°C | 1 | 2 | 640 kB | N/A | 529-Ball cspBGA | BC-529-1 |
| ADSP-SC587KBCZ-5B | 500 MHz | 0°C to +70°C | 1 | 2 | 640 kB | N/A | 529-Ball cspBGA | BC-529-1 |
| ADSP-SC587BBCZ-4B | 450 MHz | –40°C to +85°C | 1 | 2 | 640 kB | N/A | 529-Ball cspBGA | BC-529-1 |
| ADSP-SC587BBCZ-5B | 500 MHz | –40°C to +80°C | 1 | 2 | 640 kB | N/A | 529-Ball cspBGA | BC-529-1 |
| ADSP-SC589KBCZ-4B | 450 MHz | 0°C to +70°C | 1 | 2 | 640 kB | 1 | 529-Ball cspBGA | BC-529-1 |
| ADSP-SC589KBCZ-5B | 500 MHz | 0°C to +70°C | 1 | 2 | 640 kB | 1 | 529-Ball cspBGA | BC-529-1 |
| ADSP-SC589BBCZ-4B | 450 MHz | –40°C to +85°C | 1 | 2 | 640 kB | 1 | 529-Ball cspBGA | BC-529-1 |
| ADSP-SC589BBCZ-5B | 500 MHz | –40°C to +80°C | 1 | 2 | 640 kB | 1 | 529-Ball cspBGA | BC-529-1 |

1 Z =RoHS Compliant Part.

2 Referenced temperature is ambient temperature. The ambient temperature is not a specification. Please see the Operating Conditions section for the junction temperature (TJ) specification which is the only temperature specification.

3 N/A means not applicable.

I2C refers to a communications protocol originally developed by Philips Semiconductors (now NXP Semiconductors).

©2018 Analog Devices, Inc. All rights reserved. Trademarks and registered trademarks are the property of their respective owners.

D13317-0-12/18(B)

Rev. B   |   Page 173 of 173   |   December 2018